| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B031 Filename: JVT-B031.doc Generated: 2002-01-10 |
| --- | --- |

| Title: | 16-Bit Based Transform and Quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA | Tel: Emal: | +1 214 480-3816 zhou@ti.com |
| Source: | Texas Instruments |  |  |

# Introduction

In the contribution, a 16-bit transform and quantiztion proposal is described. The proposal shares the following features:

- Least departure from the current transform and quantization used in TML, only change is the normative scales introduced to the decoder side and the corresponding scales used on the encoder side.

- Least memory requirement.

- Single quantization and inverse-quantization for the first and second level transform/inverse-transform outputs.

- full 16-bit solution, all the data access including the intermediate one is limited to 16-bit, no shifts over 16-bit

- Verified coding efficiency compared to the 32-bit solution, no coding efficiency loss throughout the entire bit-rate range.

- Easy to incorporate the finer quantization scales and quantization matrices.

- True single transform and quantization for N bits (N = 9, 10, 11, 12, 13, etc.) residuals, no bit-depth dependence.

# Description of Proposal

2.1 4x4  transform & quantization for 24 4x4 luminace and chrominance blocks

- 4x4 forward transform

f00f01f02f03f10f11f12f13f20f21f22f23f30f31f32f33=256169x16913131313177−7−1713−13−13137−1717−7a00a01a02a03a10a11a12a13a20a21a22a23a30a31a32a331317137137−13−1713−7−131713−1713−7

where {fi,j |  i,j = 0,1,2,3} and {ai,j | i,j =0,1,2,3} are the coefficient block and the pixel block, repectively.

- Quantization

Let fi,j  be a coefficient defined above, the quantization is defined as

- LEVELi,j = (fi,j x AA(QP) + fx216)/216      f is in the range (0-0.5) and f has the same sign as fi,j                                <2.1>

where { Leveli,j | i,j=0, 1, 2, 3} is the quantized coefficient block, the quantization scale  AA(QP) is defined as

- AA(QP=0,..,31) = {4326, 3854, 3434, 3059, 2725, 2428, 2163, 1927, 1717, 1530, 1363, 1214, 1082, 964, 858, 765, 681, 607, 541, 482, 429, 382, 341, 304, 270, 241, 215, 191, 170, 152, 135, 120}.

- Inverse Quantization

- The inverse quantization is defined as follows:

- f'i,j = (LEVELi,j x BB(QP) + 32)>>6              	<2.2>

where { f’i,j | i,j=0, 1, 2, 3} is the inverse-quantized coefficient block, the inverse-quantization scale  BB(QP) is defined as

- BB(QP=0,..,31)={969, 1088, 1221, 1371, 1538, 1727, 1938, 2176, 2442, 2742, 3077, 3454, 3877, 4352, 4885, 5484, 6155, 6909, 7755, 8705, 9771, 10968, 12311, 13819, 15511, 17411, 19543, 21936, 24622, 27638, 31022, 34822}

- BB(QP) is in 16-bit, the relation between AA() and BB() is: AA(QP) x BB(QP) = 222.

- 4x4 inverse transform

- b'00b'01b'02b'03b'10b'11b'12b'13b'20b'21b'22b'23b'30b'31b'32b'33=1317137137−13−1713−7−131713−1713−7f'00f'01f'02f'03f'10f'11f'12f'13f'20f'21f'22f'23f'30f'31f'32f'33//16

- a'00a'01a'02a'03a'10a'11a'12a'13a'20a'21a'22a'23a'30a'31a'32a'33=b'00b'01b'02b'03b'10b'11b'12b'13b'20b'21b'22b'23b'30b'31b'32b'3313131313177−7−1713−13−13137−1717−7//256

In the above transform, the output of the vertical transform {b’i,j | i,j=0,1,2,3}  is in 16-bit, the reconstructed pixel block where { a’i,j | i,j=0, 1, 2, 3} is the reconstructed pixel block. is in 9-bit.

2.2 4x4  transform & quantization for  one 4x4 luminace DC block

- 4x4 forward transform

fD00fD01fD02fD03fD10fD11fD12fD13fD20fD21fD22fD23fD30fD31fD32fD33=111111−1−11−1−111−11−1d00d01d02d03d10d11d12d13d20d21d22d23d30d31d32d33111111−1−11−1−111−11−1//4

- Quantization

The same quantization <2.1> is applied to the luminance DC coefficient block

- Inverse quantization

On the decoder side, the same inverse-quantization <2.2> is used.

- 4x4 Inverse transform

d'00d'01d'02d'03d'10d'11d'12d'13d'20d'21d'22d'23d'30d'31d'32d'33=111111−1−11−1−111−11−1f'D00f'D01f'D02f'D03f'D10f'D11f'D12f'D13f'D20f'D21f'D22f'D23f'D30f'D31f'D32f'D33111111−1−11−1−111−11−1//4

2.3 2x2 transform & quantization for two 2x2 chrominance DC blocks

- 2x2 forward transform

The 2 dimensional 2x2 transform procedure is illustrated below.  DC0,1,2,3 are the DC coefficients of 2x2 chroma blocks.

DC0	DC1    Two dimensional 2x2 transform  	DDC(0,0)	DDC(1,0)

DC2	DC3	DDC(0,1)	DDC(1,1)

Definition of transform:

- DCC(0,0) = (DC0+DC1+DC2+DC3)//2

- DCC(1,0) = (DC0-DC1+DC2-DC3)//2

DCC(0,1) = (DC0+DC1-DC2-DC3)//2

DCC(1,1) = (DC0-DC1-DC2+DC3)//2

- Quantization

The same quantization of <2.1> is used.

- Inverse quantization

The same inverse quantization of <2.2> is used.

- 2x2 inverse transform

- DC0 = (DCC(0,0)+ DCC(1,0)+ DCC(0,1)+ DCC(1,1))//2

- DC1 = (DCC(0,0)- DCC(1,0)+ DCC(0,1)- DCC(1,1))//2

DC2 = (DCC(0,0)+ DCC(1,0)- DCC(0,1)- DCC(1,1))//2

DC3 = (DCC(0,0)- DCC(1,0)- DCC(0,1)+ DCC(1,1))//2

The following table summarises the bit precision of each processing step for 9-bit input by using the proposed 16-bit based transform and quantization.

| 4x4 transform & quantization for 24 4x4 luminace and chrominance blocks |  |  |  |
| --- | --- | --- | --- |
| Operation | Input data precison | output data precision | comments |
| 4x4 forward transform | 9-bit | 14-bit |  |
| Quantization | 14-bit and 13-bit | 10-bit | shift of 16 and 15 bit is required |
| Inverse-Quantization | 10-bit and 16-bit | 14-bit | shift of 4 bit is required |
| 4x4 inverse transform | 14-bit | 9-bit | shift of 4 and 8 bit is required |
| 4x4 transform & quantization for one 4x4 luminace DC block |  |  |  |
| 4x4 forward transform | 14-bit | 16-bit | shift of 2 bit needed |
| Quantization | 16-bit and 13-bit | 12-bit | shift of 16 and 15 bit is required |
| Inverse-Quantization | 12-bit and 16-bit | 16-bit |  |
| 4x4 inverse transform | 16-bit | 14-bit | shift of 2 bit required |
| 2x2 transform & quantization for two 2x2 Chrominace DC blocks |  |  |  |
| 4x4 forward transform | 14-bit | 15-bit |  |
| Quantization | 15-bit and 13-bit | 11-bit | shift of 16 and 15 bit is required |
| Inverse-Quantization | 11-bit and 16-bit | 15-bit |  |
| 4x4 inverse transform | 15-bit | 14-bit | shift of 2 bit required |

Table 1.  data storage precision and bit-shift in the 16-bit based transform and quantization

# Extension to Finer Quantization Scales

The finer QP could be –8, which corresponds to the finest quantization scale 1. It is easy to extend the proposal to incorporate the finer quantization scale, namely, QP = -8, -7, …-1 by simply adding eight more entrie to the AA and BB array, i.e.

- AA(QP=-8,..,31) = {

- 10902,9712,8653,7709,6868,6118,5451,4856,4326,3854,3434,3059,2725,2428,2163,1927,1717,1530,1363,1214,1082,964,858,765,681,607,541,482,429,382,341,304,270,241,215,191,170,152,135,120}.

- BB(QP=-8,..,31)={ 384,431,484,544,610,685,769,863,969,1088,1221,1371,1538,1727,1938,2176,2442,2742,3077,3454,3877,4352,4885,5484,6155,6909,7755,8705,9771,10968,12311,13819,15511,17411,19543,21936,24622,27638,31022,34822}

The quantization and inverse-quantization formula remains unchanged. Note that AA and BB are still in 16-bit precision, so a full 16-bit solution is still maintained even with the finer quantization scales.

# Extension to Quantization Matrices

It is also straightforward to incorporate quantization matrices.  With the quantization matrices the  quantization and inverse quantization  become:

- Quantization: LEVELi,j = (fi,j x AA(QP+W(i,j)) + fx216)/216      f is in the range (0-0.5) and f has the same sign as fi,j

- Inverse-Quantization: f'i,j = (LEVELi,j x BB(QP+W(i,j)) + 32)>>6

Where W(i,j) is the quantization matrix.

# Experimental Results

TML8.5 software was used to verify the proposal above for 9-bit residual case. The original TML8.5 software was treated as the reference of 32-bit solution, then the 16-bit transform and quantization was integrated into the software to test the proposed 16-bit solution. The following test conditions were set both for the 32-bit and 16-bit cases.

Hadamard transform                : Used

Image format                      : 176x144

Error robustness                  : Off

Search range                      : 16

MV resolution                     : 1/4-pel

No of ref. frames used in P pred  : 5

Sequence type                     : IPPP

Entropy coding method             : UVLC

Search range restrictions         : none

RD-optimized mode decision        : used

Data Partitioning Mode            : 1 partition

Output File Format                : H.26L File Format

The test sequences used were H.26L test sequences [2], QP = 0, 2, 4, 8, 12, 16, 20, 24, 28 were used to simulate various bit-rates. Experimental results on the seven H.26L test sequences were listed in Table 2, where AV denotes the average result over the nine quantization scales used. As shown in Table 2, the proposed 16-bit method provides almost the identical coding efficiency to the 32-bit transform and quantization currently used in TML..

| Container, QCIF (176 x144), 10 frame/s, 100 frames |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 48.33 49.88 50.26 | 344.75 | 48.34 49.95 50.27 | 346.89 | 0.01 0.07 0.01 | 2.14 |
| 2 | 46.71 48.72 48.96 | 262.42 | 46.80 48.82 49.04 | 267.06 | 0.09 0.10 0.08 | 4.64 |
| 4 | 45.00 47.33 47.55 | 196.82 | 44.92 47.35 47.58 | 193.54 | -0.08 0.02 0.03 | -3.28 |
| 8 | 41.70 45.21 45.28 | 101.96 | 41.80 45.18 45.39 | 103.45 | 0.10 -0.03 0.11 | 1.49 |
| 12 | 38.76 42.83 42.72 | 50.62 | 38.79 42.78 42.78 | 50.66 | 0.03 -0.05 0.06 | 0.04 |
| 16 | 36.08 40.44 40.06 | 25.58 | 36.06 40.51 40.01 | 25.43 | -0.02 0.07 -0.05 | -0.15 |
| 20 | 33.53 38.94 38.45 | 14.26 | 33.46 38.80 38.45 | 14.17 | -0.07 -0.14 0.00 | -0.09 |
| 24 | 30.77 37.34 36.82 | 8.53 | 30.71 37.34 36.68 | 8.51 | -0.06 0.00 -0.14 | -0.02 |
| 28 | 27.94 36.64 35.65 | 4.94 | 27.99 36.64 35.54 | 5.02 | 0.05 0.00 -0.11 | 0.08 |
| AV | 38.76 43.04 42.86 | 112.21 | 38.76 43.04 42.86 | 112.75 | 0.01 0.00 -0.00 | 0.54 |
| News, QCIF (176 x144), 10 frame/s, 100 frames |  |  |  |  |  |  |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 48.70 49.31 49.84 | 297.33 | 48.68 49.31 49.89 | 298.29 | -0.02 0.00 0.05 | 0.96 |
| 2 | 47.24 48.11 48.66 | 238.70 | 47.31 48.21 48.80 | 241.49 | 0.07 0.10 0.14 | 2.79 |
| 4 | 45.72 46.70 47.26 | 192.56 | 45.66 46.64 47.19 | 191.02 | -0.06 -0.06 -0.07 | -1.54 |
| 8 | 42.73 44.52 44.95 | 123.83 | 42.82 44.53 45.03 | 125.31 | 0.09 0.01 0.08 | 1.48 |
| 12 | 39.79 41.94 42.56 | 78.53 | 39.80 41.95 42.59 | 78.76 | 0.01 0.01 0.03 | 0.23 |
| 16 | 36.86 39.15 39.63 | 48.74 | 36.84 39.16 39.55 | 48.12 | -0.02 0.01 -0.08 | -0.62 |
| 20 | 33.93 37.55 38.32 | 30.22 | 33.91 37.57 38.28 | 29.95 | -0.02 0.02 -0.04 | -0.27 |
| 24 | 31.09 36.04 36.74 | 18.49 | 31.07 35.92 36.77 | 18.27 | -0.02 -0.12 0.03 | -0.22 |
| 28 | 28.15 34.61 35.65 | 10.78 | 28.21 34.67 35.52 | 10.88 | 0.06 0.06 -0.13 | 0.10 |
| AV | 39.36 41.99 42.62 | 115.46 | 39.37 42.00 42.62 | 115.79 | 0.01 0.00 0.00 | 0.32 |
| Foreman, QCIF (176 x144), 10 frame/s, 100 frames |  |  |  |  |  |  |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 48.07 48.35 49.10 | 639.92 | 48.08 48.36 49.14 | 643.27 | 0.01 0.01 0.04 | 3.35 |
| 2 | 46.45 46.78 48.05 | 503.02 | 46.57 46.93 48.14 | 511.54 | 0.12 0.15 0.09 | 8.52 |
| 4 | 44.81 45.25 46.88 | 392.11 | 44.70 45.20 46.90 | 385.95 | -0.11 -0.05 0.02 | -6.16 |
| 8 | 41.63 42.71 44.99 | 232.81 | 41.76 42.78 45.14 | 237.46 | 0.13 0.07 0.15 | 4.65 |
| 12 | 38.73 40.72 42.78 | 137.16 | 38.73 40.70 42.77 | 136.73 | 0.00 -0.02 -0.01 | -0.43 |
| 16 | 36.08 39.12 40.59 | 79.55 | 36.02 39.06 40.62 | 78.94 | -0.06 -0.06 0.03 | -0.61 |
| 20 | 33.65 37.85 38.95 | 47.81 | 33.66 37.92 38.95 | 47.73 | 0.01 0.07 0.00 | -0.08 |
| 24 | 31.41 36.67 37.46 | 29.64 | 31.38 36.70 37.40 | 29.75 | -0.03 0.03 -0.06 | 0.11 |
| 28 | 28.83 35.94 36.49 | 18.66 | 28.89 35.98 36.58 | 18.86 | 0.06 0.04 0.09 | 0.20 |
| AV | 38.85 41.49 42.81 | 231.19 | 38.87 41.51 42.85 | 232.25 | 0.01 0.03 0.04 | 1.06 |
| Paris, CIF (352 x288), 15 frame/s, 200 frames |  |  |  |  |  |  |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 47.98 48.43 48.95 | 2766.44 | 47.98 48.47 49.01 | 2777.69 | 0.00 0.04 0.06 | 11.25 |
| 2 | 46.24 47.15 47.59 | 2018.08 | 46.29 47.26 47.69 | 2051.29 | 0.05 0.11 0.10 | 33.21 |
| 4 | 44.56 45.75 46.13 | 1501.65 | 44.50 45.82 46.10 | 1482.11 | -0.06 0.07 -0.03 | -19.54 |
| 8 | 41.54 43.44 43.67 | 885.72 | 41.61 43.49 43.73 | 897.51 | 0.07 0.05 0.06 | 11.79 |
| 12 | 38.56 40.87 41.00 | 552.75 | 38.56 40.84 41.01 | 552.44 | 0.00 -0.03 0.01 | -0.31 |
| 16 | 35.61 38.41 38.40 | 338.60 | 35.54 38.39 38.40 | 336.84 | -0.07 -0.02 0.00 | -1.76 |
| 20 | 32.67 36.44 36.67 | 200.35 | 32.65 36.45 36.72 | 198.59 | -0.02 0.01 0.05 | -1.76 |
| 24 | 29.78 34.63 34.98 | 113.02 | 29.75 34.64 34.89 | 112.09 | -0.03 0.01 -0.09 | -0.93 |
| 28 | 26.85 33.14 33.63 | 60.48 | 26.89 33.13 33.54 | 60.96 | 0.04 -0.01 -0.09 | 0.48 |
| AV | 38.20 40.92 41.22 | 937.45 | 38.20 40.94 41.23 | 941.06 | -0.00 0.03 0.01 | 3.60 |
| Silent, QCIF (176 x144), 15 frame/s, 200 frames |  |  |  |  |  |  |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 48.62 48.85 49.27 | 411.10 | 48.61 48.86 49.26 | 412.79 | -0.01 0.01 -0.01 | 1.69 |
| 2 | 46.90 47.47 47.93 | 322.95 | 46.98 47.60 48.01 | 327.83 | 0.08 0.13 0.08 | 4.88 |
| 4 | 45.24 46.03 46.54 | 255.97 | 45.15 45.91 46.53 | 252.75 | -0.09 -0.12 -0.01 | -3.22 |
| 8 | 41.96 43.30 44.00 | 158.93 | 42.02 43.39 44.14 | 161.26 | 0.06 0.09 0.14 | 2.33 |
| 12 | 38.86 40.72 41.47 | 99.04 | 38.87 40.77 41.54 | 99.20 | 0.01 0.05 0.07 | 0.16 |
| 16 | 35.96 38.32 39.22 | 60.90 | 35.89 38.33 39.15 | 60.51 | -0.07 0.01 -0.07 | -0.39 |
| 20 | 33.31 36.53 37.99 | 37.10 | 33.30 36.54 37.95 | 37.08 | -0.01 0.01 -0.04 | -0.02 |
| 24 | 30.73 35.05 36.83 | 21.98 | 30.74 35.03 36.85 | 21.88 | 0.01 -0.02 0.02 | -0.10 |
| 28 | 28.22 34.39 36.16 | 12.27 | 28.18 34.23 36.12 | 12.29 | -0.04 -0.16 -0.04 | 0.02 |
| AV | 38.87 41.18 42.16 | 153.36 | 38.86 41.18 42.17 | 153.95 | -0.01 0.00 0.02 | 0.59 |
| Mobile, CIF (352 x288), 30 frame/s, 300 frames |  |  |  |  |  |  |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 47.69 48.00 48.05 | 11803.55 | 47.69 48.04 48.08 | 11860.73 | 0.00 0.04 0.03 | 57.18 |
| 2 | 46.00 46.31 46.36 | 9622.09 | 46.11 46.47 46.52 | 9744.33 | 0.11 0.16 0.16 | 122.24 |
| 4 | 44.26 44.57 44.64 | 7768.62 | 44.12 44.49 44.52 | 7681.96 | -0.14 -0.08 -0.12 | -86.66 |
| 8 | 40.76 41.30 41.33 | 4885.20 | 40.92 41.44 41.47 | 4960.56 | 0.16 0.14 0.14 | 75.36 |
| 12 | 37.39 38.16 38.15 | 2937.67 | 37.40 38.15 38.15 | 2936.84 | 0.01 -0.01 0.00 | -0.83 |
| 16 | 34.09 35.24 35.14 | 1636.14 | 34.02 35.18 35.09 | 1619.62 | -0.07 -0.06 -0.05 | -16.52 |
| 20 | 30.92 33.17 32.99 | 836.24 | 30.87 33.17 33.02 | 828.42 | -0.05 0.00 0.03 | -7.82 |
| 24 | 28.03 31.29 31.02 | 422.13 | 28.01 31.31 31.01 | 418.93 | -0.02 0.02 -0.01 | -3.20 |
| 28 | 25.19 30.06 29.66 | 236.62 | 25.25 30.03 29.63 | 238.08 | 0.06 -0.03 -0.03 | 1.46 |
| AV | 37.15 38.68 38.59 | 4460.92 | 37.15 38.70 38.61 | 4476.61 | 0.01 0.02 0.02 | 15.69 |
| Tempete, CIF (352 x288), 30 frame/s, 260 frames |  |  |  |  |  |  |
| QP | 32-bit Transform & Quantization |  | 16-bit Transform & Quantization |  | Difference (16-bit vs. 32-bit) |  |
|  | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s | PSNR(Y,U,V) [dB] | Kbit/s |
| 0 | 47.90 48.31 48.36 | 10144.16 | 47.90 48.34 48.38 | 10193.08 | 0.00 0.03 0.02 | 48.92 |
| 2 | 46.28 46.66 46.78 | 8129.68 | 46.39 46.82 46.92 | 8249.42 | 0.11 0.16 0.14 | 119.74 |
| 4 | 44.64 44.97 45.25 | 6463.40 | 44.51 44.89 45.17 | 6383.77 | -0.13 -0.08 -0.08 | -79.63 |
| 8 | 41.34 41.85 42.66 | 3931.94 | 41.48 41.98 42.76 | 3999.89 | 0.14 0.13 0.10 | 67.95 |
| 12 | 38.16 39.01 40.40 | 2298.97 | 38.16 39.01 40.40 | 2298.38 | 0.00 0.00 0.00 | -0.59 |
| 16 | 35.07 36.70 38.57 | 1244.52 | 35.01 36.67 38.54 | 1231.72 | -0.06 -0.03 -0.03 | -12.80 |
| 20 | 32.17 35.16 37.25 | 621.91 | 32.11 35.16 37.23 | 616.90 | -0.06 0.00 -0.02 | -5.01 |
| 24 | 29.52 33.64 35.87 | 313.12 | 29.50 33.64 35.87 | 311.14 | -0.02 0.00 0.00 | -1.98 |
| 28 | 26.85 32.57 34.93 | 168.29 | 26.87 32.53 34.88 | 169.67 | 0.02 -0.04 -0.05 | 1.38 |
| AV | 37.99 39.87 41.12 | 3701.78 | 37.99 39.89 41.13 | 3717.11 | 0.00 0.02 0.01 | 15.33 |

Table 2.  Experimental results on seven H.26L test sequences

# Conclusions

The proposed 16-bit transform and quantization is the simplest solution of implementing the current H.26L transform and quantization fully in 16-bit arithmetic in terms of computational complexity and memory requirement. Experimental results verified that the proposed 16-bit method has the same coding efficiency as the current 32-bit one used in TML. With the proposed method it is also easy to satisfy the requirements like finer quantization scales and quantization matrices which were widely used in MPEG-1, MPEG-2, and MPEG-4.

# References

- H.26L Test Model Long Term Number 8 (TML-8) draft0, Q.6/SG16(VCEG), 6/28/01

- ISO/IEC JTC 1/SC 29 , 14496-2, Information technology – Generic coding of audio-visual objects – Part 2: Visual, October, 1998

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Teaxs Instruments Inc. |  |  |
| Mailing address | 12500 TI Blvd., TX-75093 |  |  |
| Country | USA |  |  |
| Contact person | Minhua ZHOU |  |  |
| Telephone | +1 214 480 3816 |  |  |
| Fax |  |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | 01/18/2002 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | 16-bit transform and quantization |  |
| Contribution number |  | JVT-B031 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks: