| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B038 Filename: JVT-B038.doc Generated: 2002-01-14 |
| --- | --- |

| Title: | Low Complexity Transform and Quantization – Part I: Basic Implementation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro, Marta Karczewicz Nokia Research Center Tampere, Finland Henrique Malvar Microsoft Research Redmond, WA, USA | Tel: Email: Tel: Email: | +358718035390 antti.hallapuro@nokia.com +1(425)706-2978 malvar@microsoft.com |
| Source: | Nokia Corporation and Microsoft Corporation |  |  |

_____________________________

# Summary

We propose an alternative set of transforms that have significantly lower computational complexity than those in the current TML draft standard [1]. While the transforms in the draft call for 32-bit arithmetic, the transforms presented here can be computed in 16-bit arithmetic, for 9-bit residual input data. Furthermore, the proposed transforms are multiplier-free; they require only additions and a minimum number of shifts. This document also proposes a simplified quantization structure, which reduces the size of the quantization tables. Impact on performance due to these changes is negligible – typically less than 0.02 dB distance between the R/D curves for TML and the transform/quantization proposed here. In fact, for a typical set of video sequences, the modifications proposed here led to a slight PSNR gain (~0.02 dB) when compared to the current TML.

This document is a revised version of [2], and it includes also some material from [3]. Here, we restrict the discussion to 9-bit residual, present rate-distortion distance data, and discuss some implementation issues. A companion document [4] extends the algorithms of this proposal to include additional features: processing of higher bit-depth pixel data; minor transform modifications that allow for exact inverse transform computation without divisions, in the absence of quantization (thus allowing for lossless encoding); extended quantization tables for finer granularity; and support for transform-domain weighting matrices.

# Preliminary Considerations

It has been noted that the DCT-like transform and quantization currently employed in TML require 32-bit multiplications. It would be desirable if only 16-bit multiplications were required, since many processors cannot perform 32-bit multiplications with a single instruction. In the following sections we describe how we significantly reduce the computational complexity of the transforms without a performance penalty, using only 16-bit arithmetic.

## Simplified DCT

A 4x4 forward DCT transform can be calculated as follows [5]:

where A is the transform matrix whose entries are

Considering that A is orthogonal, i.e. ATA=I, the corresponding inverse transform (IDCT) is

A butterfly-based factorization of the transform matrix A leads to the following representation of forward transform:

where d=c/b. Since B is a diagonal matrix, one can write above equation as:

where  denotes element-by-element multiplication instead of normal matrix multiplication. Since multiplying with elements of matrix E consists of simply scaling the output with constants, this scaling can be combined with the quantization tables at the encoder, and with de-quantization tables at the decoder. Note that there are three distinct values in the elements of the matrix E. Thus, as we discuss later, there are three quantization and de-quantization tables; we choose the one appropriate for the transform coefficient being processed.

The inverse transform for the previously presented DCT can be calculated accordingly as

.

The nontrivial coefficient  in the transform matrix C is not a rational number, and therefore cannot be implemented exactly using binary arithmetic. We therefore propose that d be approximated by a value that allows for the matrix multiplication to be computed with fixed-point arithmetic. Possible choices include d = 7/16, 3/8, and 1/2, and we propose the simplest choice d = 1/2, since there is virtually no loss in transform coding gain, as discussed in [3].

To maintain orthogonality of the matrix A, that is for ATA=I, we need to modify b accordingly. Solving for b, we obtain:

where, as before, a = 1/2 and . So, .

## Forward Transform Precision

Using the proposed substitution d = 1/2, the forward transform equation becomes

Multiplication by a coefficient 1/2 can be implemented using a right shift, but that introduces a truncation error. We avoid that by scaling by two every other row of the first coefficient matrix and every other column of the second coefficient matrix. The forward transform then becomes

Note that no actual multiplications are necessary to compute the transform, since the transform matrix elements are all equal to ± 1 or ± 2 (a multiplication by two can be implemented as an addition or a left shift), and the final scaling is absorbed by the quantization step. Therefore, we call our proposed transform multiplier-free.

## Dynamic Range

For the forward transform matrix above, if the input values are bound by x < U, then it’s easy to see that the elements of Y prior to multiplication by E are bounded by , where g is the maximum value for the sum of the absolute values of any row. Since , the output coefficients after the 2-D transform above will span 6 more bits than the input. Therefore, for 9-bit input (U = 255) the transform coefficients have a dynamic range of 15 bits. Thus, the 2-D forward transform can be computed with 16-bit arithmetic, without risk of overflow. For the inverse transform, the dynamic range depends on the de-quantization scaling factors. The design presented in the following Section keeps the inverse transform dynamic range at 16 bits, for minimum rounding error without overflow. In TML, both the forward and inverse transforms require 32-bit arithmetic, including many multiplications with 32-bit inputs.

## Inverse Transform Precision

For the inverse transform we use our original construction for d = 1/2, that is

We do not scale the coefficients to avoid the factors equal to 1/2, as we did with the forward transform. Instead, we implement them as right shifts. The small errors introduced are compensated by a larger dynamic range for the data at the input of the inverse transform.

## Quantization

In TML quantization is performed at the encoder by a simple scale-and-shift formula. For each transform coefficient with value K, a quantized coefficient L is computed by

where |f| typically is in the range [0-0.5] and f has the same sign as K, and QP is the quantization parameter, which varies from 0 to 31. The division can be clearly implemented via a shift operation. Also according to TML, de-quantization is performed at the decoder via a simple scaling, via the formula

where K΄ is the value of the reconstructed transform coefficient. Note that both quantization and de-quantization formulas require 32-bit multiplications.

In TML the QP-dependent scaling factors A(QP) and B(QP) are specified by two tables with 32 entries each [1]. They satisfy A(QP)B(QP)6762  240. Also, they satisfy B(QP+6)  2B(QP), with equality for most cases. In the next section we propose modified quantization tables that absorb the scaling factors from the E matrices and use the periodicity in B(QP+6) = 2B(QP).

# Proposed Transform and Quantization Steps

In this section we present a step-by-step description of the proposed algorithms for transform and quantization. Familiarity with TML [1] is assumed. The symbol // denotes division with rounding to the nearest integer:

## 4x4 luma and chroma transform and quantization

Forward transform

We recall that our proposed 2-D transform is defined by:

Quantization

As we mentioned before, we implement a periodic quantization table, because the scaling factors change by a factor of two for every increment of six in QP. Thus, our indices into quantization coefficient tables depend only on QP%6, and the quantization and de-quantization formulas depend both on QP%6 and QP/6.

Quantization is performed according to the following equation

,

where Y are the transformed coefficients, YQ are the corresponding quantized values, Q are the quantization coefficients, and f is equal to 217+QP/6/3 for intra and 217+QP/6/6 for inter frames, with f having the same sign as the coefficient that is being quantized. The quantization coefficients Q are given in the Appendix. Note that while the intermediate value inside square brackets in the equation above has a 32-bit range, the final value YQ is guaranteed to fit in 16 bits.

De-quantization

De-quantization of coefficients is performed by the equation

,

where YQ are the quantized transform coefficients and R are the de-quantization coefficients. Values of R are given in the Appendix.

Inverse transform

Our proposed 2-D inverse transform is defined by

Finally, the inverse transformed coefficients are normalized with

## 4x4 luma DC transform and quantization

The luma DC coefficients of a 1616 block are grouped into a 44 block and further transformed, for intra frames, to improve compression.

Forward transform

The input matrix XD is formed by picking out DC coefficients from the 16 transformed 44 blocks. DC coefficients are then transformed using a symmetric Hadamard transform (since it leads to essentially the same performance as the DCT-like transform in TML), in the form:

We note that each of the 1-D row or column transforms above can be computed by a butterfly structure that uses 8 additions per row or column.

Quantization

Quantization is performed using the following equation

,

Inverse transform

An inverse Hadamard transform must be used, since the forward transform was a Hadamard. If YQD are the quantized luma DC coefficients then the inverse transform is defined by

De-quantization

De-quantization of coefficients is computed by the equation

,

Note that the order of inverse transform and de-quantization of DC coefficients is changed. It does not influence the results as long as scaling is done after both of them, i.e. Y = (AT(X*k)A)/4 is equal to Y = ((ATXA)*k)/4. However, it is better to do inverse transform first, to achieve the best possible dynamic range during inverse transform computations.

## 2x2 chroma DC transform and quantization

Forward transform

Chroma DC transform is added on top of chroma transform. Input matrix is formed by picking out DC coefficients from the 4 transformed 4x4 blocks. If these coefficients are denoted as XD then transform is computed as

Quantization

Quantization is performed using the following equation

,

Inverse transform

Chroma DC inverse transform is computed in the same way as in TML-9, except that scaling by ½ is moved to de-quantization:

XQD=111−1YQD00YQD01YQD10YQD11111−1

De-quantization

De-quantization of chroma DC coefficients is based on the equation

,

Also for chroma the order of inverse transform and de-quantization of DC coefficients is changed.

## Decoder conformance

Since the de-quantization and inverse transform specified in the previous subsections all employ integer arithmetic, all conformant decoders should produce exactly the same decoded residuals, for all valid input quantized transform data. We define as valid quantized transform data any combination of quantized transform coefficients that results in no value outside the range of a 16-bit signed integer [–32768 to 32767] at any stage of the inverse transform process, for any step size. In particular, quantized transform data produced by the recommended steps in the previous subsections are valid.

# Complexity

The proposed transform can be implemented using only additions and shifts, and thus multiplications are only used during quantization and de-quantization. Tables 1 and 2 show the number of operations required by the encoder and the decoder, respectively. The numbers of operations are computed for the fastest possible implementations of the transform and inverse transform which will be shown in the butterfly structures of Subsection 6.2. We see from Table 1 that all the quantization and de-quantization can be computed with 16-bit multiplications, since input values for these operations never exceed 16 bits. Thus, the proposed method can be implemented fully with 16-bit arithmetic, for 9-bit residual input data. Table 3 compares the complexity of both forward and inverse transforms in the proposed method to that of TML. The quantization and dequantization complexity of the proposed method and TML are comparable.

Table 1 – Encoder operations

| 4x4 transform for 24 4x4 luminace and chrominance blocks |  |  |
| --- | --- | --- |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 64 |
|  | << | 16 |
| 4x4 transform for one 4x4 luminace DC block |  |  |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 64 |
|  | // | 16 |
| 2x2 transform for two 2x2 Chrominace DC blocks |  |  |
| Process | Operation | Number per block |
| 4x4 forward transform | + | 8 |
| quantization for coefficient |  |  |
| Process | Operation | Number per coefficient |
| Quantization | * | 1 |
|  | // | 1 |

Table 2 – Decoder operations

| 4x4 inverse transform and dequantization for 24 4x4 luminace and chrominance blocks |  |  |
| --- | --- | --- |
| Process | Operation | Number per block |
| Dequantization | * | 16 |
| 4x4 inverse transform | + | 64 |
|  | >> | 16 |
| Normalization | + | 16 |
|  | >> | 16 |
| 4x4 inverse transform and dequantization for one 4x4 luminace DC block |  |  |
| Process | Operation | Number per block |
| 4x4 inverse transform | + | 64 |
| Dequantization | * | 16 |
|  | // | 16 |
| 2x2 inverse transform and dequantization for two 2x2 Chrominace DC blocks |  |  |
| Process | Operation | Number per block |
| 2x2 inverse transform | + | 8 |
| Dequantization | * | 4 |
|  | // | 4 |

Table 3 – Number of operations for transform

| Operation | Number of operations |  |  |
| --- | --- | --- | --- |
|  | Proposed | TML | TML, multiplier-less |
| Additions | 64 | 64 | 128 |
| Multiplications | 0 | 48 | 0 |
| Shifts | 16 | 0 | 64 |

# Experimental Results

We have modified the TML 8.5 software to compute the transform and quantization as specified in Section 3, with no changes in the other parts of encoder and decoder. Several sequences were processed, for several values of QP, and the resulting peak SNR (PSNR) and bit rates are listed in [2], where we see that the rate-distortion (R/D) curves are nearly identical. Those R/D curves were then compared, and their distances computed according to the method in [6]. Table 3 shows the results.

Table 3 – PSNR gains for the proposed method (dB)

| Sequence | PSNR, Y | PSNR, U | PSNR, V |
| --- | --- | --- | --- |
| News | 0.018 | -0.026 | 0.053 |
| Container | -0.002 | -0.044 | -0.068 |
| Silent | 0.024 | 0.043 | 0.073 |
| Paris | 0.031 | 0.004 | 0.020 |
| Mobile | 0.039 | 0.046 | 0.046 |
| Tempete | 0.014 | 0.010 | 0.022 |
| Average | 0.021 | 0.006 | 0.024 |

We see that the average PSNR gain for the proposed 16-bit transform/quantization algorithm, with respect to the original 32-bit TML code is slightly positive, indicating that for that data set our proposed transform produces a slightly better average PSNR (over all frames of those sequences, for all QP specified in [6]) . We also computed the average PSNR gain for the first intra frames of each sequence, since intra frames have significantly different statistics, and the average gains for Y, U, and V were 0.046, 0.010, and 0.038 dB, respectively. Again, the PSNR results were slightly better for our proposed transform/quantization.

Given the relatively small data set that we processed, average PSNR differences of around 0.02 dB are not statistically significantly different from zero. Therefore, we can state that our proposed transform/quantization method achieves the same R/D performance as that of the current TML.

# Implementation Issues and Other Considerations

In this Section we discuss some alternative implementations of the transform and quantization equations. The best option depends on specifics of the implementation, such as target processor architecture.

## Computing quantization indices

In Section 3 we saw that we need to compute QP%6 and QP/6.  Those values need to be computed only once per macroblock, so the division operations won’t add significant complexity. Still, those divisions by 6 can be avoided, if so desired, by noting that QP/6 = (43QP)>> 8 for all QP in [0...130].

## Transforms via butterflies

The 2-D forward transform of Section 3 can be computed as 1-D row transforms followed by 1-D column transforms. Each of these 1-D transforms is computed by the equations below:

M0 = X0 + X3;

Y0

Y2

Y1

Y3

M3 = X0 - X3;

M0

M1

M2

M3

M1 = X1 + X2;

-

M2 = X1 – X2;

Y0 = M0 + M1;

-

Y2 = M0 – M1;

-2

2

Y1 = M2 + (M3 << 1);

X0

X1

X2

X3

-

Y3 = M3 - (M2 << 1);

The 2-D inverse transform can also be computed as 1-D row transforms followed by 1-D column transforms. Each of these 1-D transforms is computed by the equations below:

M0 = Y0 + Y2;

X0

-

X1

X2

X3

-

-

1/2

Y0

Y2

Y1

Y3

M0

M1

M2

M3

1/2

M1 = Y0 - Y2;

M2 = (Y1 >> 1) - Y3;

M3 = Y1 + (Y3 >> 1);

-

X0 = M0 + M3;

X3 = M0 - M3;

X1 = M1 + M2;

X2 = M1 - M2;

## Transforms via matrix multiplications

In some processor architectures, special instructions for matrix multiplication for length-4 vectors of 16-bit entries may be available. Thus, let us consider the computation of the transforms when matrix multiplication is available

Forward transform

Let us focus on the 1-D transform of either rows or columns. The original definition of the 1-D forward transform is

It’s clear that we can use one 16-bit matrix multiplication to compute the forward transform.

Inverse transform

We recall that the definition of our proposed 1-D inverse transform is

Here we’re faced with the problem that the matrices above have noninteger elements. Furthermore, an alternative implementation must produce the same rounding errors as the butterfly implementation of Subsection 3.1. Fortunately it is possible to build an exact direct matrix multiply by carefully taking care of rounding of values before bit shifts. Thus, the 1-D inverse transform can alternatively be computed by:

X0 = (2*Y0 + 2*Y1 + 2*Y2 + 1*Y3    ) >> 1

X1 = (2*Y0 + 1*Y1 - 2*Y2 - 2*Y3    ) >> 1

X2 = (2*Y0 - 1*Y1 - 2*Y2 + 2*Y3 + 1) >> 1

X3 = (2*Y0 - 2*Y1 + 2*Y2 - 1*Y3 + 1) >> 1

Note that the correction “+ 1” in the equations for X2 and X3 comes from the fact that –(Y >> 1) = (–Y +1) >> 1.

An alternative approach is to use the following steps:

M  = (Y1 >> 1) - Y3;

Y3 = Y1 + (Y3 >> 1);

Y1 = M;

followed by the matrix multiplication

.

That is, we first implement one butterfly and then perform one 16-bit matrix multiply.

## Quantization tables – memory requirements

Our proposed quantization is performed using several quantization tables, whereas the current TML uses only two tables (one for quantization and the other for de-quantization). Our tables would take up to 32162 = 1024 bytes of memory if they were stored fully for each of the QP values. Since we have implemented a periodic quantization table, however, the number of unique quantization matrices is reduced to 6, thus reducing the memory requirement to 6162 = 192 bytes. For de-quantization matrices we need only one byte per matrix element, and so the required memory is 6161 = 96 bytes. TML requires 64 bytes for the quantization table, and 128 bytes for the de-quantization table. Therefore, our total memory footprint for such tables is just 64 bytes more than that of TML.

There can be a concern regarding additional memory access required for loading values of quantization matrix from memory during quantization. It should be noted that we do not use a full quantization matrix, but rather three different quantization values that can be conveniently presented in a quantization matrix form:

Q(k)=akbkakbkbkckbkckakbkakbkbkckbkck

where k = QP%6. Because the matrix above has only three unique values and a regular structure, the number of memory accesses for the quantization matrix can be easily reduced to a minimum of three accesses. In custom ASIC design, it is possible to individually choose one value out of three for each of the 16 coefficients.

In the most straightforward (and inefficient) software implementation of quantization, there is one loop that runs 16 times, once per coefficient, using the quantization values in Q(k). In a more efficient implementation we can load just the values ak, bk, and ck to three processor registers. Then we unroll the loop, i.e. we convert the loop into 16 consecutive code blocks each quantizing one coefficient using one of the three variables. The resulting small increase in code memory size is negligible in view of the entire code for the encoder or decoder.

Another option for implementing the quantization is to write a loop that runs 8 times, once per two coefficients. The first of the two coefficients is read from the first row and first column, and the second from the third row and first column. Afterwards, the associated pointers are incremented in raster-scan order. Since the quantization factors (ak, bk, or ck) are the same for each coefficient pair, we would need to store only the first two rows of the quantization matrix. This not only reduces memory access for quantization tables by half, but it also cuts the quantization and de-quantization matrix storage requirement by half, to 96 and 48 bytes, respectively. If a further reduction is needed, an 8-byte indexing table could be used to pick which quantization factor (ak, bk, or ck) is to be used. That way, for the cost of one more 1-byte memory access per coefficient, one could reduce the table sizes to 632 = 36 bytes for the quantization tables and 631 = 18 bytes for the quantization tables. Adding to that the 8 bytes for the index, we would have a minimum quantization table footprint of 36+18+8 = 62 bytes. That’s about 1/3rd of TML’s 192-byte quantization table memory size.

# Conclusion

We presented a new structure for computation of the transforms, inverse transforms, quantization, and de-quantization in H.26L. This new structure can replace those in the current TML coder, with no changes in the other parts of the system. The TML transform and quantization structures require 32-bit arithmetic and several multiplications within the 44 transforms. Our proposed structures require only 16-bit arithmetic, and the transforms require no multiplications.

Therefore, we conclude that our proposed method achieves a significant reduction in complexity for the transform and quantization procedures. It can be implemented exactly and efficiently in different kinds of processor architectures. The complexity savings come with no penalty in the rate/distortion performance.

# References

- T. Wiegand, “H.26L Test Model Long-Term Number 9 (TML-9) draft0”, ITU-T Q.6/SG16 Document TML9.doc (VCEG-N83d1), December 2001.

- A. Hallapuro, M. Karczewicz, “Low Complexity (I)DCT”, ITU-T Q.6/SG16 Document VCEG-O52, November, 2001. See accompanying file “VCEG-O25.xls”.

- H. S. Malvar, “Low-Complexity Length-4 Transform and Quantization with 16-Bit Arithmetic,” ITU-T Q.6/SG16 Document VCEG-N44, September, 2001.

- A. Hallapuro, M. Karczewicz, and H. Malvar, “Low Complexity Transform and Quantization – Part II: Extensions,” ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6 Document JVT-B039, January 2002.

- K. R. Rao and P. Yip. Discrete Cosine Transform: Algorithms, Advantages, Applications. Boston: Academic Press, 1990, Chapter 4.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-curves,” ITU-T Q.6/SG16 Document VCEG-M33, March 2001.

# Appendix - Quantization coefficients

Q[QP%6][i][j] = quantMat[QP%6][0] for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

Q[QP%6][i][j] = quantMat[QP%6][1] for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

Q[QP%6][i][j] = quantMat[QP%6][2] otherwise.

R[QP%6][i][j] = dequantMat[QP%6][0] for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

R[QP%6][i][j] = dequantMat[QP%6][1] for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

R[QP%6][i][j] = dequantMat[QP%6][2] otherwise.

quantMat[6][3] = {{13107, 5243, 8224}, {11651, 4660, 7358}, {10486, 4143, 6554}, {9198, 3687, 5825}, {8322, 3290, 5243}, {7384, 2943, 4660}};

dequantMat[6][3] = {{40, 64, 51}, {45, 72, 57}, {50, 81, 64}, {57, 91, 72}, {63, 102, 80}, {71, 114, 90}};

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 936 7329

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia |  |  |
| Mailing address | 6000 Connection Drive Irving, TX 75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 214 763 1314 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | 01/23/02 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Low Complexity Transform and Quantization – Part I |  |
| Contribution number |  | JVT-B038 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 936 7329

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | One Microsoft Way, Redmond, WA 98052 |  |  |
| Country | USA |  |  |
| Contact person | Henrique Malvar |  |  |
| Telephone | +1 425 706 2978 |  |  |
| Fax |  |  |  |
| Email | malvar@microsoft.com |  |  |
| Place and date of submission | 01/23/02 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Low Complexity Transform and Quantization – Part I |  |
| Contribution number |  | JVT-B038 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]