| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B047 Filename: JVT-B047.doc Generated: 2002-01-18 |
| --- | --- |

| Title: | Removal of EOB Coding Redundancy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA | Tel: Emal: | +1 214 480-3816 zhou@ti.com |
| Source: | Texas Instruments |  |  |

# Introduction

In the current TML[1], a EOB is always appended after encoding DCT-coefficients of a block, independent of whether the last coefficient of the coded block (index N-1 in Figure 1) is zero or non-zero. Case b in Figure 1 illustrates a case in which the last coefficient in the block is non-zero, it has the same input signal form as case a according to the current run-length coding scheme, which is:

(run0, level0), (run1, level1), (run2, level2), EOB

However, in case b the EOB is redundant. Since the block size is known, the decoder automatically detects that the whole block is decoded if the currently decoded non-zero is the (N-1)th (last) coefficient of the block (the coefficient index of a block cannot go beyond N-1). Therefore, in case b a more efficient coding should remove EOB from the VLC input coding symbol list, using:

(run0, level0), (run1, level1), (run2, level2)

Removal of EOB in this case would save bit-rate. Although H.26L only uses one bit to represent EOB, the used 4x4 block size and double scan for high bit-rate coding could produce a reasonable amount number of redundant EOB bits in the bitstream.

Figure 1. H.26L Run-length coding examples

Reomoval of  EOB Coding Redundancy

The way of removing EOB coding redundancy is very simple, a EOB is inserted into bitstream after encoding (RUN, LEVEL) sequence of a coded block only if its last coefficient is zero.

For a robust decoder that does error detection by checking number of coefficients decoded, there is no complexity increase on the decoder side. For decoding block of N coefficients, if EOB is always transmitted, a robust decoder would do (*dct_zz is array to store the decoded coefficients and is pre-initialized to zero).

ptr = -1; 
do {

UVLD_Dec(stream, &run, &level);    /* decode next (run, level) */
ptr +=run+1;
dct_zz[ptr] = level;

}  while (level && ptr <N);

if (ptr >N || dct_zz[N]) return ERR_DETECTED; /* error detection */

Note: run and level are set to zero in the decoding table if EOB is decoded.

If EOB is conditionally transmitted, a decoder will do

ptr = -1; 
do {

UVLD_Dec(stream, &run, &level);    /* decode next (run, level) */
ptr +=run+1;
dct_zz[ptr] = level;

}  while (level && ptr <N-1);

if (ptr >=N) return ERR_DETECTED; /* error detection */

So in decoding process the only difference is ptr < N replaced by ptr < N-1, no complexity increase. N could be 4, 8, 16 in TML. While in error detection step, we probably can save some cycles.

On the encoder side, the EOB (run=level=0) is conditionally added to the coding symbol (RUN, LEVEL) list by checking the value of the last coefficient. Therefore, there is a branch for each coded block.

# Experimental Results

Table 1 lists the testing results on the seven sequences with the frame-rate recommended in [2], all frames are coded in INTRA-frame. As shown in Table 1, the lower the QP is, the higher the wasted EOB rate is. Up to 0.95% over the total bit-rate was measured for the redundant EOB bits.

|  | QP = 2 | QP = 4 | QP = 8 | QP = 16 | QP = 24 |
| --- | --- | --- | --- | --- | --- |
| Container (QCIF) | 0.81 | 0.74 | 0.69 | 0.41 | 0.01 |
| News (QCIF) | 0.70 | 0.67 | 0.64 | 0.42 | 0.02 |
| Foreman (QCIF) | 0.95 | 0.88 | 0.79 | 0.44 | 0.03 |
| Paris (CIF) | 0.82 | 0.77 | 0.73 | 0.54 | 0.04 |
| Silent (QCIF) | 0.77 | 0.67 | 0.51 | 0.13 | 0.01 |
| Mobile (CIF) | 0.85 | 0.86 | 0.85 | 0.63 | 0.02 |
| Tempete (CIF) | 0.81 | 0.77 | 0.69 | 0.37 | 0.01 |

Table 1. Percentage of redundant EOB bits w.r.t the total bit-rate [%].

# Conclusions

Considering that H.26L is the most advanced coding standard and there is no penalty in terms of complexity by introducing this minor change, it is recommended to adopt this proposal making H.26L one more bit superior to the previous MPEG and ITU-T video standards.  Also, an efficient I-frame only H.26L codec will be a good substitute to the existing motion JPEG (or even Motion JPEG200) and DV format which are currently widely used in applications like DSCs (Digital Still Camera) and Camcorders.

# References

- H.26L Test Model Long Term Number 8 (TML-8) draft0, Q.6/SG16(VCEG), 6/28/01

- VCEG-M75, Recommended Simulation Conditions for H.26L

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 <fax number here>

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Teaxs Instruments Inc. |  |  |
| Mailing address | 12500 TI Blvd., TX-75093 |  |  |
| Country | USA |  |  |
| Contact person | Minhua ZHOU |  |  |
| Telephone | +1 214 480 3816 |  |  |
| Fax |  |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | 01/18/2002 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | H.26L |  |
| Title |  | Removal of EOB Coding Redundancy |  |
| Contribution number |  | JVT-B047 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks:

[end]