| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B048 Filename: JVT-B048.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Supporting Film Mode in JVT Codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sherman (Xuemin) Chen, José Alvarez, Sandy MacInnis 3151 Zanker Road San José, CA 95134 | Tel: Email: | 1-949-585-6185 schen@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

## Introduction

Film material produced at 24 frames/sec is routinely converted to 60 fields/sec for distribution purposes. A Film Mode mechanism for encoding, decoding and display of converted film material exists in MPEG-2, and this solution has served the industry well, however, it requires encoders to be aware of the display device. Optimal encoding of converted film material requires what is known as 3:2 pull-down, where the original film frames are detected and encoded as frames and the encoder is required to convey proper display timing to the decoder.

A necessary requirement for encoding MPEG-2 video using Film Mode is that the frame rate encoded in the sequence header must indicate the frame rate of the original 60 field/sec input video sequence. Flags top_field_first and repeat_first_field are used for indicating how a picture should be displayed. However, this requirement is not desirable when the display device can vary, e.g. interlaced TV vs. progressive monitors, and the encoder has no way of knowing what kind of display will be used at the decoder end.

In this document we propose an simple optional picture header display extension to facilitate the coding and display of converted film material. One main advantage of this proposal is the separation of coding and display processes.

## Background

The 3:2 pull-down process is commonly used in converting a 24 frame/second film sequence into a 30 (29.97) frame/second or 25 frame/second sequence for display.  Figure 1 shows the 3:2 pull-down process.

Figure 1 Mapping of the 3:2 pull-down process

In MPEG-2 video elementary streams, two flags in the coded picture header, top_field_first and repeat_first_field, are used to indicate the current film state. Let us define the film states as shown in Table 1 and Figure 1. The four possible film mode states (represented as A, B, C, and D) are repeated in the same order every four pictures. Film ode processing will always commence with state A (or C) and exit with state D (or B).

Film Mode encoding refers to the case when an encoder directly compresses 24 frame/sec material, or when an encoder uses the 3:2 pull-down process to convert 30 frame/sec video to 24 frame/sec material prior to encoding. If the input video sequence  prior to compression by an encoder is derived from a 24-frame/second film sequence, top_field_first and repeat_first_field flags indicate the “fields” that need to be repeated for a 30 (29.97) frame/second or a 25 frame /second display device.  If the input sequence is a video sequence that has already been converted from a film sequence, a 3:2 pull-down detector is often required to restore the film sequence prior to encoding.  In this case, two repeated fields are removed from each ten-field sequence by the 3:2 pull-down detector, as shown in Figure 1.

| - Pictures | - Transmitted Fields | - Repeat First Field Flag | - Top Field First Flag | - Displayed Field |
| --- | --- | --- | --- | --- |
| - A | - A1 | - 1 | - 1 | - A1 |
|  | - A2 |  |  | - A2 |
|  |  |  |  | - A1 |
| - B | - B1 | - 0 | - 0 | - B2 |
|  | - B2 |  |  | - B1 |
| - C | - C1 | - 1 | - 0 | - C2 |
|  | - C2 |  |  | - C1 |
|  |  |  |  | - C2 |
| - D | - D1 | - 0 | - 1 | - D1 |
|  | - D2 |  |  | - D2 |

- Table 1 Mapping of the 3:2 pull-down flags in MPEG-2

### MPEG-2 System Timestamps

In MPEG-2 Systems, in addition to considering picture reordering when B-pictures are present, it is needed to check if the current picture has been coded in Film Mode in order to correctly compute the PTS and DTS.  Two of every four pictures will be of a three-field duration (one and one-half times the nominal picture period), while the other two are of a two-field duration (the nominal picture period). Therefore, in Film Mode, the time an I- or P-picture is delayed in the reordering buffer will not be merely the number of subsequent B-picture(s) multiplied by the nominal picture period, but instead will be the total display time of the B-picture(s). For example, if a P-picture is followed by two B-pictures, one of which will be displayed for a two-field duration and the other for a three-field duration, then the P-picture will be delayed for a total of five-field times, or two and one-half picture times. The PTS for the picture then becomes the DTS plus two and one-half picture times.

- The decoder will display film state A and C pictures for three field times since they both contain a “un-encoded” field of data. The decoder will re-display the first field to replace the "un-encoded" field. This is because in the 3:2 pull-down algorithm, the first field is repeated every other picture to convert film material at 24 pictures/sec to video mode at 30 pictures/second or 25 picture/second. Film state B and D pictures are displayed for only two field times. A Film Mode sequence of four pictures will therefore be displayed as a total of 10 field times. In this way, the decoded video is displayed at the correct video picture rate.

| - Displayed Frames | - Film States | - Repeat First Field Flag | - Top Field First Flag | - Displayed Field |
| --- | --- | --- | --- | --- |
| - 0 |  | - 0 | - 0 | - Frm0Bot - Frm0Top |
| - 1 | - C | - 1 | - 0 | - Frm1Bot - Frm1Top - Frm1Bot |
| - 2 | - D | - 0 | - 1 | - Frm2Top - Frm2Bot |
| - 3 | - A | - 1 | - 1 | - Frm3Top - Frm3Bot - Frm3Top |
| - 4 | - B | - 0 | - 0 | - Frm4Bot - Frm4Top |
| - 5 | - C | - 1 | - 0 | - Frm5Bot - Frm5Top - Frm5Bot |
| - 6 | - D | - 0 | - 1 | - Frm6Top - Frm6Bot |
| - 7 | - A | - 1 | - 1 | - Frm7Top - Frm7Bot - Frm7Top |
| - 8 | - B | - 0 | - 0 | - Frm8Bot - Frm8Top |
| - 9 |  | - 0 | - 0 | - Frm9Bot - Frm9Top |
| - 10 |  | - 0 | - 0 | - Frm10Bot - Frm10Top |

- Table 2 A film mode sequence

- Table 2 shows a sequence of eleven coded pictures that are output from the video encoder during which Film Mode is enabled and then disabled.  In this example, Film Mode is enabled on Picture 1 and disabled on Picture 9.

## Proposal

We propose the following optional header and flags to simplify Film Mode coding and display processes.

Let us define frame_rate as the actual frame rate input to the video encoder (possibly after preprocessing such as 3:2 pull-down detector). The frame_rate is sent in the sequence header.

Include the following syntax elements in the (optional) sequence display extension.

Sequence display extension

{

…

display_rate

…

}

Note that if sequence display extension is not carried in the bit stream, then display_rate = frame_rate.

Include the following syntax elements in the picture display extension.

Picture display extension

{

…

film_mode_flag				1 bit

if (film_mode_flag)

film_mode_state				2 bits

…

}

film_mode_flag – Film Mode is enabled if the value of this 1-bit flag equals one.

film_mode_state – This is a 2-bit code that indicates the Film Mode state given in the table below.  In the display order, encoder shall commence with the film state 00 (or 10) and exit with the film state 11 (or 01).

| - Film mode state | - Frame notation | - Field - parity | - Displayed Field |
| --- | --- | --- | --- |
| - 00 | - a | - a1 | - a1 |
|  |  | - a2 | - a2 |
|  |  |  | - a1 |
| - 01 | - b | - b1 | - b2 |
|  |  | - b2 | - b1 |
| - 10 | - c | - c1 | - c2 |
|  |  | - c2 | - c1 |
|  |  |  | - c2 |
| - 11 | - d | - d1 | - d1 |
|  |  | - d2 | - d2 |

The picture display extension is an optional header for transmitting picture level display related information.  This proposal completely separates the coding and display process.  One major difference between this proposal and MPEG-2 is that the frame rate here is the rate for actual coded frame. This implies that in some cases, the time-stamping process for DTS and PTS will be different than that for MPEG-2 video.

Example

The example given is Table 2 can be shown in this proposal as:

| - Displayed Frames | - Film Mode States | - Displayed Field |
| --- | --- | --- |
| - 0 |  | - Frm0Bot - Frm0Top |
| - 1 | - 10 | - Frm1Bot - Frm1Top - Frm1Bot |
| - 2 | - 11 | - Frm2Top - Frm2Bot |
| - 3 | - 00 | - Frm3Top - Frm3Bot - Frm3Top |
| - 4 | - 01 | - Frm4Bot - Frm4Top |
| - 5 | - 10 | - Frm5Bot - Frm5Top - Frm5Bot |
| - 6 | - 11 | - Frm6Top - Frm6Bot |
| - 7 | - 00 | - Frm7Top - Frm7Bot - Frm7Top |
| - 8 | - 01 | - Frm8Bot - Frm8Top |
| - 9 |  | - Frm9Bot - Frm9Top |
| - 10 |  | - Frm10Bot - Frm10Top |

Table 3. An example of film mode states.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Broadcom Corporation |  |  |
| Mailing address | 3151 Zanker Road San José, CA 95134 |  |  |
| Country | United States of America |  |  |
| Contact person | Sherman (Xuemin) Chen |  |  |
| Telephone | 949-585-6185 |  |  |
| Fax |  |  |  |
| Email | schen@broadcom.com |  |  |
| Place and date of submission | Geneva, Switzerland. January 23, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Supporting Film Mode in JVT Codec |  |
| Contribution number |  | JVT-B048 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: