| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B054 Filename: JVT-B054.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Core Experiment Results On Improved Macroblock Prediction Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report and Proposal |  |  |
| Author(s) or Contact(s): | Heiko Schwarz and Thomas Wiegand Heinrich Hertz Institute (HHI) Einsteinufer 37 10587 Berlin, Germany | Tel: Fax Email: | +49-(0)30-31002 206 / 617 +40-(0)30-392 72 00 hschwarz@hhi.de wiegand@hhi.de |
| Source: | Heinrich Hertz Institute (HHI) |  |  |

# Summary

In this document, we present an improvement of our proposal “Tree-structured macroblock partition” (VECG-O17) [1]. Furthermore, we report the results of the Core Experiment on Improved Macroblock Prediction Modes defined in VCEG-O61 [3]. The accompanying Excel sheet contains R-D curves as well as the average PSNR gains and bit-rate reductions calculated by the avsnr4 software [6].

# Introduction

During the Pattaya meeting, VCEG/JVT decided to set up a Core Experiment (CE) to verify the multiple proposals on improved MB prediction modes (VCEG-O17[1] and VCEG-O22[2]).

In VCEG-O17 [1], we proposed a minor syntax alteration that provides tree-structured macroblock partitions in motion-compensated coding modes. The experimental results show a general improvement in terms of coding efficiency while the complexity of the encoding strategy is not increased. The results indicate that the gains are increased for complex scenes and higher bit-rates. In this document we present a slight improvement of VCEG-O17 that further increases the coding gain for B-frames. A further modification concerns the model selection for CABAC.

VCEG-O22 has proposed a low-overhead prediction tool consisting of multi-shape motion segmentation patterns with up to two motion vectors. It has been reported that this proposal could improve the prediction efficiency of TML by integrating it with the current TML motion model. Both proposals permit motion-segment based reference frame selection, which is useful to predict object occlusion area efficiently.

We report the results of the Core Experiment on Improved Macroblock Prediction Modes, which has been defined in VCEG-O61 [3]. Simulations have been carried out for both proposals, the improved version of VCEG-O17 and the extended version of VCEG-O22 (see VCEG-O61 [3]).

# Tree-Structured Macroblock Partition (Improved VCEG-O17)

In the TML [5], seven macroblock partitions for motion compensation are supported. The reference frame parameter is transmitted once per macroblock if a predictive coding mode is chosen. We propose a minor syntax alteration that provides tree-structured macroblock partitions in motion-compensated coding modes. The reference frame parameter is transmitted on block basis for 16x16, 16x8, 8x16, and 8x8 blocks. For smaller block sizes, a single reference frame parameter for all blocks of an 8x8 sub-partition is used.

## 3.1 Proposed Syntax Specification

For P-frames, the predictive modes with block shapes of 8x8, 8x4, 4x8, and 4x4 samples of the TML [5] are replaced by a single macroblock mode called 8x8(split). If this mode is chosen, four additional code words (one for each 8x8 sub-partition) are transmitted. These code words indicate if the corresponding 8x8 sub-partition of the macroblock is coded predictively with block shapes of 8x8, 8x4, 4x8, or 4x4, or if it is coded in INTRA-mode. The Tables 1 and 2 show the corresponding code words for the macroblock modes and the modes of 8x8 sub-partitions.

For the predictive macroblock modes with block shapes of 16x16, 16x8, and 8x16 one reference parameter is coded for each block. However if the 8x8(split) mode is chosen, only one reference parameter is transmitted for each 8x8 sub-partition regardless what mode it is coded in. If UVLC is used for entropy coding, an additional macroblock mode is supported: 8x8(split, all ref=0). It indicates the usage of the 8x8(split)-mode, however no reference frame parameter is transmitted for the whole macroblock; the reference frame parameters are set to zero. Since the indication of this mode costs just as many bits as the indication of the 8x8(split)-mode, 4 bits can always be saved if the previous reference frame was chosen for all 8x8 sub-partitions in the UVLC case. For CABAC, this additional mode is not used.

Table 1: Macroblock modes for P-frames

| Code number | Macroblock mode | CABAC Binarization |
| --- | --- | --- |
| 0 | SKIP | 0 |
| 1 | 16x16 | 1000 |
| 2 | 16x8 | 1011 |
| 3 | 8x16 | 1010 |
| 4 | 8x8 (split) | 1001 |
| 5 (UVLC only) | 8x8 (split, all ref=0) |  |
| 6 | Intra4x4 | 110 |
| 7 … | Intra16x16 | 111 |

Table 2: Modes for 8x8 sub-partitions in P-frames

| Code number | 8x8 partition mode | CABAC Binarization |
| --- | --- | --- |
| 0 | 8x8 | 1 |
| 1 | 8x4 | 000 |
| 2 | 4x8 | 0011 |
| 3 | 4x4 | 0010 |
| 4 | Intra | 01 |

Beside the re-design of macroblock modes and the introduction of additional 8x8 block modes, a minor alteration concerning the motion vector prediction for 8x4 and 4x8 block was necessary. Since neighboring 8x8 partitions are generally not coded in the same mode, the directional prediction of motion vector components should not be used for 8x4 and 4x8 block. It is replaced by median prediction.

For B-frames, a similar tree-structured macroblock partitioning is proposed. As in P-frames, one reference frame parameter is transmitted for each 16x16, 16x8, and 8x16 block as well as for each 8x8 sub-partition. Additionally, for each 16x16, 16x8, 8x16 block, and each 8x8 sub-partition, the prediction direction (forward, backward, bi-directional) can be chosen separately. For avoiding a separate code word to specify the prediction direction, the indication of the prediction direction is incorporated into the codewords for macroblock modes and 8x8 partitioning modes, respectively, as shown in the table 3 and 4. The bi-directional coding mode specified in the TML is removed. Moreover, an 8x8 sub-partition of a B-frame macroblock can also be coded in Direct mode.

Table 3: Macroblock modes for B-frames

| Code number | Macroblock mode | 1. block | 2. block | CABAC Binarization |
| --- | --- | --- | --- | --- |
| 0 | Direct |  |  | 0 |
| 1 | 16x16 | Forw. |  | 100 |
| 2 | 16x16 | Backw. |  | 101 |
| 3 | 16x16 | Bidirect. |  | 110000 |
| 4 | 16x8 | Forw. | Forw. | 110001 |
| 5 | 8x16 | Forw. | Forw. | 110010 |
| 6 | 16x8 | Backw. | Backw. | 110011 |
| 7 | 8x16 | Backw. | Backw. | 110100 |
| 8 | 16x8 | Forw. | Backw. | 110101 |
| 9 | 8x16 | Forw. | Backw. | 110110 |
| 10 | 16x8 | Backw. | Forw. | 110111 |
| 11 | 8x16 | Backw. | Forw. | 111110 |
| 12 | 16x8 | Forw. | Bidirect. | 1110000 |
| 13 | 8x16 | Forw. | Bidirect. | 1110001 |
| 14 | 16x8 | Backw. | Bidirect. | 1110010 |
| 15 | 8x16 | Backw. | Bidirect. | 1110011 |
| 16 | 16x8 | Bidirect. | Forw. | 1110100 |
| 17 | 8x16 | Bidirect. | Forw. | 1110101 |
| 18 | 16x8 | Bidirect. | Backw. | 1110110 |
| 19 | 8x16 | Bidirect. | Backw. | 1110111 |
| 20 | 16x8 | Bidirect. | Bidirect. | 1111000 |
| 21 | 8x16 | Bidirect. | Bidirect. | 1111001 |
| 22 | 8x8(split) |  |  | 111111 |
| 23 | Intra4x4 |  |  | 1111010 |
| 24 … | Intra16x16 |  |  | 1111011 |

Table 4: Modes for 8x8 sub-partitions in B-frames

| Code number | 8x8 partition mode | Prediction | CABAC Binarization |
| --- | --- | --- | --- |
| 0 | Direct |  | 0 |
| 1 | 8x8 | Forw. | 100 |
| 2 | 8x8 | Backw. | 101 |
| 3 | 8x8 | Bidirect. | 11000 |
| 4 | 8x4 | Forw. | 11001 |
| 5 | 4x8 | Forw. | 11010 |
| 6 | 8x4 | Backw. | 11011 |
| 7 | 4x8 | Backw. | 111000 |
| 8 | 8x4 | Bidirect. | 111001 |
| 9 | 4x8 | Bidirect. | 111010 |
| 10 | 4x4 | Forw. | 111011 |
| 11 | 4x4 | Backw. | 111100 |
| 12 | 4x4 | Bidirect. | 111101 |
| 13 | Intra |  | 11111 |

If the entropy coding is performed with CABAC, the binarization depicted in the right column of tables 1-4 is used.

## 3.2 Motion Estimation and Mode Decision

The procedures of motion estimation and mode decision are similar to the high-complexity mode (i.e. R-D optimized mode decision is turned on) of the current TML [5]:

- For each 8x8 sub-partition in coding order:

  - Perform motion estimation for 4x4, 4x8, 8x4, and 8x8 blocks and all reference frames

  - Determine the best reference frame for each of these four partitionings by minimizing

SAD +  Rate(Motion Vectors, Reference frame parameter),

where SAD is calculated between the original and the prediction signal.

  - Determine the coding mode of the 8x8 sub-partition using the rate-constrained mode decision, i.e. minimize

SSD +  Rate(MV, REF, Luma-Coeff, block 8x8 mode)

Here the SSD calculation is based on the reconstructed signal after DCT, quantization, and IDCT.

- Perform motion estimation for 8x16, 16x8, and 16x16 blocks and determine the best reference parameter for each block by minimizing

SAD +  Rate(Motion Vectors, Reference frame parameter)

- Choose macroblock mode from the set (SKIP, 16x16, 16x8, 8x16, 8x8(split), Intra4x4, Intra16x16) by utilizing the rate-constrained mode decision as described in TML-8 [5]. At this point, the distortion and most parts of the rate term calculated during the mode decision for 8x8 partitions are re-used for the cost calculation of the 8x8(split) mode.

For B-frames, the prediction direction is determined by minimzing the following Lagrange cost:

SAD +  Rate (Motion vectors, Reference frames).

The SAD is calculated between the original and the prediction signal. No DCT, quantization, and IDCT is performed for this decision. The rate-constrained macroblock mode decision is only carried out for the best 16x16, 16x8, 8x16, and 8x8(split) partitions as well as for the Direct and Intra modes.

For each block the motion vector is determined by an integer pixel search and a following sub-pixel refinement. As in the current TML software version, the search center for all segments is determined by the prediction vector of the 16x16 block.

It should be noted that the encoding complexity is virtually not increased in comparison with the high-complexity mode of TML-8 [5].

## 3.3 Remaining Coder Parts

All remaining coder parts are operated as described in the H.26L Test Model Long Term Number 8 (TML-8) draft 0

# Core Experiment Results

The simulations are conducted following the conditions specified in VCEG-O61 [3], VCEG-N81 [4]. The VCEG-O17 proposal [1] was integrated in the TML-8.5 software. The VCEG-O22 implementation is based on the TML-9.0 software. The following configurations are used:

- P-frames only, ¼-pel MC, UVLC, 1 and 5 reference frames

- P-frames only, ¼-pel MC, CABAC, 1 and 5 reference frames

- 2 B-frames, ¼-pel MC (QCIF), 1/8-pel MC(CIF), UVLC, 1 and 5 reference frames

- 2 B-frames, ¼-pel MC (QCIF), 1/8-pel MC(CIF), CABAC, 1 and 5 reference frames

For calculating average PSNR gains and bit-rate savings, the method specified in VCEG-M33 [6] is utilized with QP values 28, 24, 20, and 16. Since the VCEG-O17 implementation is based on TML-8.5 software, theses measures are calculated with reference to the original TML-8.5 software for VCEG-O17. For VCEG-O22, the average PSNR gains and bit-rate savings are calculated with reference to the TML-9.0 software, since VCEG-O22 is integrated into this software version.

The tables 5-12 show the average PSNR gains and corresponding bit-rate savings for both methods. Rate-distortion curves for all cases and all software versions (TML-8.5, TML-9.0, VCEG-O17, VCEG-O22) are plotted in the accompanying Excel sheet. As it can be seen from the tables and the rate-distortion curves, the coding efficiency of both proposed methods is generally better than that of the reference software versions (TML-8.5, TML-9.0). Using VCEG-O17, average bit-rate savings of 0.43% - 5.01% are obtained. VCEG-O22 provides average bit-rate savings of 2.01% - 8.66%. The results indicate that the gains are increased for complex scenes.

Generally, the average PSNR gains and corresponding bit-rate savings obtained by VCEG-O22 are larger than those obtained by VCEG-O17. However, VCEG-O22 requires a significantly increased encoding complexity for obtaining these results. An additional motion search has to be performed for each of the new macroblock modes. Whereas the coding gains for VCEG-O17 are obtained nearly without increasing encoder complexity.

By looking at the rate-distortion curves it can be seen further, that the larger PSNR gains of VCEG-O22 (in comparison to VCEG-O17) are mainly attained at low bit-rates (QP =28…24). For higher bit-rates or higher video quality (QP=16), the relative coding gain of VCEG-O22 in comparison to VCEG-O17 is significantly smaller. This applies especially for complex sequences like Mobile&Calendar and Tempete.

# References

- Heiko Schwarz and Thomas Wiegand, “Tree-structured macroblock partition”, VCEG-O17, December 2001

- Satoru Adachi, Shun-ichi Sekiguchi, Sadaatsu Kato, Mitsuru Kobayashi and Minoru Etoh, “Refined Results on the Low-overhead Prediction Modes”, VCEG-O22, December 2001.

- “Core Experiments on Improved Macroblock Prediction Modes”, VCEG-O61, December 2001.

- Gary Sullivan and Gisle Bjontegaard, "Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material", VCEG-N81, September 2001.

- Gisle Bjontegaard and Thomas Wiegand, "H.26L Test Model Long Term Number 8 (TML-8) draft0," document VCEG-N10, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, Doc. VCEG-M33, Mar. 2001.

Table 5: Results for configuration with P-frames only, 1 reference frame, UVLC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,11 | 2,21 | 0,28 | 5,73 |
| Container | QCIF 10Hz | 0,04 | 0,84 | 0,14 | 3,05 |
| News | QCIF 10Hz | 0,08 | 1,30 | 0,17 | 3,03 |
| Silent | QCIF 15Hz | 0,07 | 1,33 | 0,20 | 4,13 |
| Paris | CIF 15Hz | 0,03 | 0,65 | 0,16 | 3,26 |
| Mobile | CIF 30Hz | 0,03 | 0,57 | 0,13 | 2,97 |
| Tempete | CIF 30Hz | 0,04 | 1,01 | 0,15 | 3,59 |
| Average |  | 0,06 | 1,13 | 0,18 | 3,68 |

Table 6: Results for configuration with P-frames only, 1 reference frame, CABAC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,15 | 3,04 | 0,30 | 6,18 |
| Container | QCIF 10Hz | 0,03 | 0,60 | 0,11 | 2,25 |
| News | QCIF 10Hz | 0,13 | 2,27 | 0,18 | 3,21 |
| Silent | QCIF 15Hz | 0,10 | 2,01 | 0,22 | 4,57 |
| Paris | CIF 15Hz | 0,09 | 1,73 | 0,19 | 3,55 |
| Mobile | CIF 30Hz | 0,05 | 1,12 | 0,14 | 3,03 |
| Tempete | CIF 30Hz | 0,07 | 1,67 | 0,14 | 3,50 |
| Average |  | 0,09 | 1,78 | 0,18 | 3,76 |

Table 7: Results for configuration with P-frames only, 5 reference frame, UVLC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,15 | 2,88 | 0,32 | 6,27 |
| Container | QCIF 10Hz | 0,02 | 0,43 | 0,10 | 2,01 |
| News | QCIF 10Hz | 0,12 | 1,98 | 0,22 | 3,75 |
| Silent | QCIF 15Hz | 0,12 | 2,39 | 0,25 | 4,93 |
| Paris | CIF 15Hz | 0,10 | 2,02 | 0,21 | 4,17 |
| Mobile | CIF 30Hz | 0,12 | 2,57 | 0,21 | 6,56 |
| Tempete | CIF 30Hz | 0,11 | 2,61 | 0,29 | 6,67 |
| Average |  | 0,11 | 2,13 | 0,23 | 4,91 |

Table 8: Results for configuration with P-frames only, 5 reference frame, CABAC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,19 | 3,96 | 0,38 | 7,57 |
| Container | QCIF 10Hz | 0,06 | 1,20 | 0,18 | 3,55 |
| News | QCIF 10Hz | 0,15 | 2,55 | 0,25 | 4,38 |
| Silent | QCIF 15Hz | 0,14 | 2,79 | 0,27 | 5,37 |
| Paris | CIF 15Hz | 0,14 | 2,71 | 0,26 | 5,02 |
| Mobile | CIF 30Hz | 0,15 | 3,25 | 0,32 | 6,82 |
| Tempete | CIF 30Hz | 0,14 | 3,34 | 0,28 | 6,67 |
| Average |  | 0,14 | 2,83 | 0,28 | 5,63 |

Table 9: Results for configuration with 2 B-frames, 1 reference frame, UVLC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,08 | 1,69 | 0,19 | 3,81 |
| Container | QCIF 10Hz | 0,09 | 1,84 | 0,20 | 4,01 |
| News | QCIF 10Hz | 0,13 | 2,33 | 0,16 | 2,88 |
| Silent | QCIF 15Hz | 0,12 | 2,56 | 0,17 | 3,58 |
| Paris | CIF 15Hz | 0,17 | 3,28 | 0,23 | 4,40 |
| Mobile | CIF 30Hz | 0,10 | 2,39 | 0,19 | 4,38 |
| Tempete | CIF 30Hz | 0,10 | 2,47 | 0,19 | 4,84 |
| Average |  | 0,11 | 2,37 | 0,19 | 3,99 |

Table 10: Results for configuration with 2 B-frames, 1 reference frame, CABAC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,18 | 3,89 | 0,30 | 6,28 |
| Container | QCIF 10Hz | 0,08 | 1,54 | 0,18 | 3,51 |
| News | QCIF 10Hz | 0,20 | 3,37 | 0,23 | 4,09 |
| Silent | QCIF 15Hz | 0,17 | 3,57 | 0,26 | 5,59 |
| Paris | CIF 15Hz | 0,23 | 4,30 | 0,34 | 6,29 |
| Mobile | CIF 30Hz | 0,15 | 3,50 | 0,23 | 5,34 |
| Tempete | CIF 30Hz | 0,15 | 3,92 | 0,23 | 5,95 |
| Average |  | 0,17 | 3,44 | 0,25 | 5,29 |

Table 11: Results for configuration with 2 B-frames, 5 reference frame, UVLC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,10 | 2,05 | 0,25 | 5,07 |
| Container | QCIF 10Hz | 0,07 | 1,40 | 0,21 | 3,98 |
| News | QCIF 10Hz | 0,19 | 3,27 | 0,22 | 3,94 |
| Silent | QCIF 15Hz | 0,13 | 2,69 | 0,27 | 5,60 |
| Paris | CIF 15Hz | 0,21 | 4,05 | 0,34 | 6,46 |
| Mobile | CIF 30Hz | 0,14 | 3,24 | 0,26 | 5,89 |
| Tempete | CIF 30Hz | 0,12 | 3,03 | 0,23 | 5,93 |
| Average |  | 0,14 | 2,82 | 0,25 | 5,27 |

Table 12: Results for configuration with 2 B-frames, 5 reference frame, CABAC

|  |  | VCEG-O17 |  | VCEG-O22 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Average PSNR gain [dB] | Average rate saving [%] | Average PSNR gain [dB] | Average rate saving [%] |
| Foreman | QCIF 10Hz | 0,18 | 3,89 | 0,42 | 8,66 |
| Container | QCIF 10Hz | 0,06 | 1,27 | 0,16 | 3,17 |
| News | QCIF 10Hz | 0,23 | 3,89 | 0,30 | 5,22 |
| Silent | QCIF 15Hz | 0,19 | 4,00 | 0,35 | 7,23 |
| Paris | CIF 15Hz | 0,26 | 5,01 | 0,40 | 7,56 |
| Mobile | CIF 30Hz | 0,20 | 4,57 | 0,32 | 7,18 |
| Tempete | CIF 30Hz | 0,19 | 4,95 | 0,29 | 7,44 |
| Average |  | 0,19 | 3,94 | 0,32 | 6,64 |

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Heinrich-Hertz-Institute |  |  |
| Mailing address | Einsteinstraße 37 10587 Berlin |  |  |
| Country | Germany |  |  |
| Contact person | Heiko Schwarz |  |  |
| Telephone | +49-30-31 002-206 |  |  |
| Fax | +49-30-392 72 00 |  |  |
| Email | hschwarz@hhi.de |  |  |
| Place and date of submission | Berlin, Jan. 23, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Core Experiment Results On Improved Macroblock Prediction Modes |  |
| Contribution number |  | JVT-B054 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: