| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B061 Filename: JVT-B061.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Adaptive motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author or Contact: | Yoshinori Suzuki 1-280, Higashi-koigakubo Kokubunji-shi, Tokyo 185-8601, Japan | Tel: +81-42-323-1111 Email: yosinori@crl.hitahit.co.jp |  |
| Source: | Hitachi, Ltd. |  |  |

_____________________________

Summary

This document provides the new motion vector coding method for reducing the amount of the motion vector data. The idea of this proposal is to adaptively select the fractional pixel accuracy of differential motion vector components macroblock by macroblock. The proposed scheme provides improvements of 1-4% for similar complexity with JM-1.

1. Introduction

The inter frame coding in JM-1 [1] allows for the motion compensation using the small block size up to 4x4. However, there is a tendency for these small size modes not to be selected at low bitrate even if macroblock has multiple motions, since the increasing of coding bitrate for differential motion vector components causes the bad influence to the coding performance.

As one of the solution to cope with this problem, the accuracy control method for differential motion vectors is proposed in this document.

2. Adaptive motion vector coding

 Motion vector coding/decoding method

## Encoder

MVDs are calculated by the following expression at encoder:

MVDX,MVDY=MVX−PMVXmvshift,MVY−PMVYmvshift

where

MVX,MVY means the vector components of integer version to be coded,

(PMVX,PMVY) means the vector predictor component of integer version,

(DMVX,DMVY) means the differential vector of integer version, and

the meaning of mv_shift is indicated in the table 1.

Table 1

For 1/4 sample interpolation mode

| Fractional accuracy | mv_shift | Code_number |
| --- | --- | --- |
| 1/4 | 0 | 0 |
| 1/2 | 1 | 1 |
| 1 | 2 | 2 |

For 1/8 sample interpolation mode

| Fractional accuracy | mv_shift | Code_number |
| --- | --- | --- |
| 1/8 | 0 | 0 |
| 1/4 | 1 | 1 |
| 1/2 | 2 | 2 |
| 1 | 3 | 3 |

## Decoder

MVs are calculated by the following expression at decoder:

MVX,MVY=.

 Syntax diagram

Syntax diagram for I- and P-frames

Motion Vector Data Accuracy (MVD_accuracy)

MVD_accuracy is present when MB_type is ‘Inter NM (except for 1616)’. This is done according to the table 1.

Syntax diagram for B-frames

Motion Vector Data Accuracy (MVDFW_accuracy and MVDBW_accuracy)

MVDFW_accuracy is present when MB_type is ‘Forward NM (except for 1616) or ‘Bi-directional (except for the case Blk_size for forward motion vector data is 1616)’. This is done according to the table 1.

MVDBW_accuracy is present when MB_type is ‘Backward NM (except for 1616) or ‘Bi-directional (except for the case Blk_size for backward motion vector data is 1616)’. This is done according to the table 1.

 CABAC

## Context Models for Motion Vector Data Accuracy

In case MVD accuracy is present, the context of a current MVD_accuracy C is build by using the MVD_accuracy A and B of neighboring macroblocks which set to ‘0’ if they do not existed. The acutual context number of C is then defined by ctx_mvd_accuracy(C) = ((A==0)?0:1) + 2*(B==0)?0:1), such that ctx_mvd_accuracy(C) indicates one of four models used for coding of the first bin of binary equivalent b(C) of C. Two additional models are given for the second bin and all remaining bins of b(C), which sum up to a total number of six different models for the motion vector data accuracy information.

Neighboring symbols A and B used for conditional coding of a current symbol C.

 Motion Estimation

## Finding the best motion vector accuracy

Assume that the position E is the nearest point to the best motion vector point so that the accuracy of differential motion vector components is integer. For all segmentation blocks in a macroblock, the position E is found. Then, 25 positions indicated in following figure are searched. After this search, the combination of the motion vector accuracy MVA and the associated motion vectors, which minimize the following evaluation function, is decided as the best selection:

Jm,MVA,λMOTION=SADTs,cm+λMOTION⋅Rm−p,MVA+R(MVA).

## Evaluation of motion search for best vector accuracy determination

The following table gives the difference between TML-9.0 and TML-9.0 with the motion vector search for best accuracy determination (mv_shift is always set to ‘0’) in terms of rate and PSNR as calculated using the Bjontegaard measurement method [2].

| Test Sequances | average % bit savings (avsnr) | average dB PSNR gain (avsnr) |
| --- | --- | --- |
| Paris, CIF | -0.38 | 0.01 |

MV resolution: 1/8 pel, UseHadamard: ON, Reference Frames: 1, Search Range: 32,

Symbol Mode: UVLC, Restrict Search Range: 2, RD Optimization: ON, QP: 16, 20, 24, 28

This result indicates that the performance of TML-9.0 with additional motion search is almost same as that of TML-9.0. This means that the additional motion search does not make the visible improvement of the coding performance itself.

 Complexity analysis

## Encoder

 Motion estimation

Additional 25 points searches are needed for each macroblock. However, total computational time is similar to the TML-9.0 motion estimation.

 Motion vector coding

Only a shift operation is additionally needed for the differential motion vector component to be coded.

## Decoder

Only a shift operation is additionally needed for the decoded differential motion vector component.

3. Simulation Results

The proposed method is integrated into the TML-9.0 software. Experiments were conducted according to the guideline of the common conditions document [3]. The proposed coder (AMVC) and TML-9.0 coder (TML) were tested under the condition described below.

| MV resolution | 1/4 pel for QCIF 1/8 pel for CIF |
| --- | --- |
| UseHadamard | ON |
| Reference Frames | 1 |
| B-Picture | USED (2) |
| Search Range | 32 |
| Symbol Mode | UVLC/CABAC |
| Restrict Search Range | 2 |
| RD optimization | ON |
| QP | 16,20,24,28 |

The averaged PSNR values of the decoded images and coding bitrate for TML and AMVC are shown in Table 2.

The rate-distortion curves are shown in Figure 1-5.

The differences between TML and AMVC in terms of rate and PSNR as calculated using the Bjontegaard measurement method [3] are shown in Table 3.

The comparison example of mode selection results between TML and AMVC is shown in Table 4.

Table 2. Comparison of averaged PSNR and bitrate.

(a) UVLC

| Sequence |  | TML |  | AMVC |  | Gain |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) | % | dB (Y, U, V) |
| Container QCIF 10Hz | 28 | 4.33 | 28.06, 36.84, 35.74 | 4.29 | 28.09, 36.85, 35.77 | 0.81 | 0.03, 0.01, 0.03 |
|  | 24 | 6.93 | 30.87, 37.47, 36.84 | 6.93 | 30.84, 37.51, 36.83 | 0.00 | -0.03, 0.04, -0.01 |
|  | 20 | 11.91 | 33.63, 39.00, 38.47 | 11.99 | 33.63, 39.00, 38.45 | -0.67 | 0.00, 0.00, -0.02 |
|  | 16 | 21.38 | 36.18, 40.52, 40.05 | 21.38 | 36.19, 40.50, 40.11 | 0.00 | 0.01, -0.02, 0.06 |
| Foreman QCIF 10Hz | 28 | 17.67 | 28.30, 35.91, 36.56 | 17.41 | 28.36, 35.92, 36.56 | 1.47 | 0.06, 0.01, 0.00 |
|  | 24 | 29.49 | 30.99, 36.60, 37.39 | 28.74 | 30.91, 36.63, 37.49 | 2.54 | -0.08, 0.03, 0.10 |
|  | 20 | 49.07 | 33.30, 37.86, 38.88 | 48.02 | 33.30, 37.86, 38.91 | 2.14 | 0.00, 0.00, 0.03 |
|  | 16 | 83.39 | 35.70, 39.03, 40.56 | 82.31 | 35.72, 39.04, 40.54 | 1.30 | 0.02, 0.01, -0.02 |
| News QCIF 10Hz | 28 | 10.24 | 28.07, 34.78, 35.59 | 9.97 | 28.08, 34.78, 35.63 | 2.64 | 0.01, 0.00, 0.04 |
|  | 24 | 17.61 | 30.98, 36.01, 36.69 | 17.25 | 31.04, 36.04, 36.69 | 2.04 | 0.06, 0.03, 0.00 |
|  | 20 | 29.20 | 33.87, 37.55, 38.37 | 28.89 | 33.85, 37.60, 38.33 | 1.06 | -0.02, 0.05, -0.04 |
|  | 16 | 48.40 | 36.79, 39.19, 39.69 | 48.02 | 36.76, 39.16, 39.65 | 0.79 | -0.03, -0.03, -0.04 |
| Silent QCIF 15Hz | 28 | 11.38 | 28.13, 34.36, 36.09 | 11.20 | 28.12, 34.43, 36.18 | 1.58 | -0.01, 0.07, 0.08 |
|  | 24 | 21.30 | 30.56, 35.03, 36.88 | 20.76 | 30.59, 35.03, 36.86 | 2.54 | 0.03, 0.00, -0.02 |
|  | 20 | 37.35 | 33.20, 36.47, 37.97 | 36.49 | 33.17, 36.49, 37.98 | 2.30 | -0.03, 0.02, 0.01 |
|  | 16 | 62.45 | 35.77, 38.22, 39.15 | 61.69 | 35.75, 38.28, 39.18 | 1.22 | -0.02, 0.06, 0.03 |
| Paris CIF 15Hz | 28 | 65.40 | 26.77, 33.37, 33.66 | 63.08 | 26.79, 33.37, 33.75 | 3.55 | -0.02, 0.00, 0.09 |
|  | 24 | 116.98 | 29.67, 34.81, 34.95 | 112.97 | 29.68, 34.86, 35.01 | 3.43 | 0.01, 0.05, 0.06 |
|  | 20 | 205.59 | 32.52, 36.47, 36.61 | 200.29 | 32.57, 36.55, 36.70 | 2.58 | 0.05, 0.08, 0.09 |
|  | 16 | 350.02 | 35.49, 38.43, 38.46 | 345.22 | 35.48, 38.49, 38.52 | 1.37 | 0.01, 0.06, 0.06 |

(b) CABAC

| Sequence |  | TML |  | AMVC |  | Gain |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) | % | dB (Y, U, V) |
| Container QCIF 10Hz | 28 | 4.04 | 28.05, 36.84, 35.73 | 4.02 | 28.03, 36.85, 35.73 | 0.50 | -0.02, 0.01, 0.00 |
|  | 24 | 6.75 | 30.85, 37.50, 36.75 | 6.65 | 30.88, 37.48, 36.84 | 1.48 | 0.03, -0.02, 0.09 |
|  | 20 | 11.27 | 33.65, 39.03, 38.57 | 11.31 | 33.64, 39.01, 38.56 | -0.35 | -0.01, -0.02, -0.01 |
|  | 16 | 20.14 | 36.21. 40.51, 40.05 | 20.22 | 36.23, 40.55, 40.10 | -0.39 | 0.02, -0.04, 0.05 |
| Foreman QCIF 10Hz | 28 | 15.77 | 28.36, 35.92, 36.56 | 15.76 | 28.43, 35.99, 36.54 | 0.06 | 0.07, 0.07, -0.02 |
|  | 24 | 27.28 | 30.88, 36.64, 37.43 | 26.75 | 30.97, 36.72, 37.50 | 1.94 | 0.09, 0.08, 0.07 |
|  | 20 | 46.31 | 33.28, 37.89, 38.96 | 45.35 | 33.31, 37.93, 38.86 | 2.07 | 0.03, 0.04, -0.10 |
|  | 16 | 79.15 | 35.73, 39.06, 40.53 | 78.19 | 35.75, 39.08, 40.55 | 1.21 | 0.02, 0.02, 0.02 |
| News QCIF 10Hz | 28 | 9.85 | 28.09, 34.88, 35.61 | 9.72 | 28.10, 34.84, 35.62 | 1.32 | 0.01, -0.04, 0.01 |
|  | 24 | 16.92 | 31.01, 36.11, 36.74 | 16.78 | 31.01, 36.05, 36.72 | 0.83 | 0.00, -0.06, -0.02 |
|  | 20 | 28.10 | 33.88, 37.57, 38.36 | 27.80 | 33.87, 37.56, 38.37 | 1.07 | -0.01, -0.01, 0.01 |
|  | 16 | 45.65 | 36.77, 39.27, 39.67 | 45.37 | 36.76, 39.19, 39.69 | 0.61 | -0.01, -0.08, 0.02 |
| Silent QCIF 15Hz | 28 | 10.96 | 28.04, 34.40, 36.17 | 10.84 | 28.10, 34.41, 36.15 | 1.09 | 0.06, 0.01, -0.02 |
|  | 24 | 20.48 | 30.60, 35.03, 36.86 | 19.96 | 30.61, 35.00, 36.78 | 2.54 | 0.01, -0.03, -0.08 |
|  | 20 | 35.61 | 33.21, 36.48, 37.95 | 35.02 | 33.20, 36.54, 38.00 | 1.66 | -0.01, 0.06, 0.05 |
|  | 16 | 59.63 | 35.80, 38.25, 39.18 | 58.79 | 35.77, 38.27, 39.19 | 1.41 | -0.03, 0.02, 0.01 |
| Paris CIF 15Hz | 28 | 60.79 | 26.78, 33.40, 33.73 | 59.10 | 26.79, 33.30, 33.74 | 2.78 | 0.01, -0.10, 0.01 |
|  | 24 | 109.71 | 29.68, 34.84, 34.98 | 107.62 | 29.69, 34.87, 35.00 | 1.91 | 0.01, 0.03, 0.02 |
|  | 20 | 192.55 | 32.55, 36.55, 36.66 | 189.01 | 32.56, 36.56, 36.69 | 1.84 | 0.01, 0.01, 0.03 |
|  | 16 | 325.88 | 35.48, 38.48, 38.49 | 321.28 | 35.50, 38.46, 38.52 | 1.41 | 0.02, -0.02, 0.03 |

(a) UVLC

(b) CABAC

Figure 1. RD curves for Container

(a) UVLC

(b) CABAC

Figure 2. RD curves for Foreman

(a) UVLC

(b) CABAC

Figure 3. RD curves for News

(a) UVLC

(b) CABAC

Figure 4. RD curves for Silent

(a) UVLC

(b) CABAC

Figure 5. RD curves for Paris

Table 3. Results of BD-SNR and BD-Bitrate

(a) UVLC

| Test Sequances | average % bit savings (avsnr) | average dB PSNR gain (avsnr) |
| --- | --- | --- |
| Container, QCIF | 0.25 | -0.01 |
| Foreman, QCIF | -1.64 | 0.08 |
| News, QCIF | -1.83 | 0.10 |
| Silent, CIF | -2.10 | 0.10 |
| Paris, CIF | -3.43 | 0.17 |

(b) CABAC

| Test Sequances | average % bit savings (avsnr) | average dB PSNR gain (avsnr) |
| --- | --- | --- |
| Container, QCIF | -0.65 | 0.03 |
| Foreman, QCIF | -2.88 | 0.13 |
| News, QCIF | -0.89 | 0.05 |
| Silent, CIF | -2.01 | 0.09 |
| Paris, CIF | -2.14 | 0.11 |

Table 4 	The results of mode decision for TML and AMVC (total number of macroblock; intra modes are not counted).

Paris, CIF, QP=24, UVLC

P-picture

|  |  | COPY | 1616 | 168 | 816 | 88 | 84 | 48 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | TML | 12291 | 2030 | 1262 | 872 | 755 | 387 | 667 | 126 |
|  | AMVC | 12287 | 1833 | 1101 | 773 | 815 | 608 | 804 | 312 |
|  | Diff. | -4 | -197 | -161 | -99 | +60 | +221 | +137 | +186 |

B-picture

|  |  | Direct | 1616 | 168 | 816 | 88 | 84 | 48 | 44 | Bidirect |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | TML | 30702 | 4495 | 577 | 970 | 714 | 456 | 375 | 327 | 24 |
|  | AMVC | 30722 | 4194 | 560 | 962 | 688 | 516 | 531 | 429 | 82 |
|  | Diff. | +20 | -301 | -17 | -8 | -26 | +60 | +156 | +102 | +58 |

|  | 1/8 | 1/4 | 1/2 | 1 | COPY | DIRECT |
| --- | --- | --- | --- | --- | --- | --- |
| No, of macroblock | 8173 | 1715 | 3107 | 1438 | 12287 | 30722 |
| % | 14.23 | 2.99 | 5.41 | 2.50 | 21.39 | 53.48 |

4. Conclusions

The adaptive motion vector coding in which the accuracy of different motion vector components is selected macroblock by macroblock is proposed. The following results were obtained by the experiments in section 3:

 	The proposed scheme provides improvements of 1-4% for similar complexity with TML-9.0.

 	The number of macroblock modes with small size blocks for MC selected has been increased by the use of the proposed scheme.

Considering these results, we propose the adoption of the adaptive motion vector coding in JM-2.

# Reference

[1] Th Wiegand, “Joint Model Number 1 (JM-1)”, Doc. JVT-A003, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Jan. 2002.

[2] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves”, Doc. VCEG-M33, ITU-T Q6/16, Austin, TX, USA, 2-4 April 2001.

[3] G. Sullivan, G. Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, Doc. VCEG-N81 draft 1, ITU-T Q6/16, Santa Barbara, CA, USA, 24-27 Sep. 2001.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Hitachi, Ltd. |  |  |
| Mailing address | 1-280, Higashi-koigakubo, Kokubunji-shi, Tokyo 185-8601, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Yoshinori Suzuki |  |  |
| Telephone | +81-423-1111 |  |  |
| Fax | +81-427-7776 |  |  |
| Email | yosinori@crl.hitachi.co.jp |  |  |
| Place and date of submission | Geneva, Switzerland, January 29th, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive Motion Vector Coding |  |
| Contribution number |  | JVT-B061 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: