| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B065 Filename: JVT-B065.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | ABT Coding Results for Interlaced ITU-R 601 Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ulrich Benzler Robert Bosch GmbH FV/SLM PO Box 77 77 77 31132 Hildesheim, Germany | Tel: Fax: Email: | ++49-5121-493087 ++49-5121-49173087 ulrich.benzler@de.bosch.com |
| Source: | Robert Bosch GmbH |  |  |

_____________________________

Summary

In this paper, results of the Adaptive Block Transforms (ABT) for interlaced ITU-R 601 source material are presented, both for Inter-only and Inter&Intra ABT coding. The simulations reveal an improved performance of the proposed scheme compared with the TML (~0.4 dB PSNR gain / ~8% bit rate saving), validating the results presented in [JVT-B053].

# Introduction

In this paper, coding results of the Adaptive Block Transforms (ABT) for interlaced ITU-R 601 source material are presented.

The concept of ABT was presented to VCEG in several documents [VCEG-L15, VCEG-M62, VCEG-N49, VCEG-O31]..At the Pattaya meeting, a simplified ABT concept was presented using only the 4x4 and the 8x8 transform matrix [VCEG-O30]. Here, we present the coding results of this simplified ABT coding of inter and intra blocks when applied to interlaced ITU-R 601 video. A full description of the applied ABT coding scheme can be found in [JVT-B053].

The paper is organized as follows: First we present the simulation conditions for ABT inter and intra coding according to the Interlaced and the Transform AdHoc groups [VCEG-O59]. In the second part, the performance of ABT in the TML-9.0 software is given for these conditions. Due to the size of the sequences and the resulting computation time for the simulations, only a subset of the quantization parameters QP listed in [VCEG-O59] and only ITU-R 601 resolution (no HHR resolution) was used.

# Experimental Results

Simulation results are given according to the common interlaced coding conditions that were agreed on by the Interlaced and the Transform Size AdHoc groups [VCEG-O59]. The ABT coding tools were integrated into the TML9.0 software that was released at the Pattaya meeting.

## Encoding of interlaced Video

The common condition for interlaced video include field based encoding of interlaced sequences with B-frames. It was agreed that the following setting should be used:

IP BB BB PP BB BB PP …

i.e., the top field of frame #0 is coded I, the bottom field P, the fields of frames #1,#2 are coded B, the two fields of frame #3 are coded P, and so forth.

The reference software as well as the ABT implementation were modified to enable IP BB BB PP coding of a field sequence. Encoding of the B-frames is performed after encoding of the top field of the next P-frame.

## Simulation Conditions

The following common conditions were used for the simulations

| Picture Structure (field based) | - II II II - IP PP PP PP - IP BB BB PP BB BB PP |
| --- | --- |
| Motion Resolution | ¼ pel |
| Quantization Parameters | 16,24 [QP(B) = QP(I/P)+2] |
| Search Range | 16 pixel |
| Hadamard (SATD) | ON |
| Reference Frames | 2 |
| Entropy Coding | CABAC |

The following test sequences were used

| Sequence Name | Frame Size | No. of Frames |
| --- | --- | --- |
| F1_car | 720x576 | 220 |
| Canoa | 720x576 | 220 |
| Rugby | 720x576 | 220 |
| Tempete | 720x480 | 260 |
| Football | 720x480 | 260 |
| Bus | 720x480 | 150 |

The sequences (except for Bus) are available from the VQEG ftp site (www.vqeg.org). They were converted from 422 to 420 using the converter tool provided by Peter Borgwardt [VCEG-N85]. In a pre-experiment all sequences showed improved coding performance for field based coding compared to frame based coding. Therefore, only field based coding was used for the simulations.

### ABT Modes

For performance evaluation of ABT two settings were used for the simulations:

- ABT Inter only. For intra coding the TML intra techniques are used

- ABT Inter&Intra. ABT is used for Inter and Intra Coding

## Results

As only a subset of the QP values could be simulated due to the large computation time, the difference in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33] can not be given (four rate-distortion points are needed for this method). The performance of ABT inter and ABT inter&intra in comparison to the performance of the TML encoder validates the results presented in [JVT-B053]. The corresponding RD plots are given in the Appendix.

# Discussion and Conclusion

Simulation results are presented for the concept of Adaptive Block Transforms (ABT) for Inter and Intra coding. The simulations reveal an improved performance of the proposed scheme, validating the results presented in [JVT-B053].

We propose to adopt the ABT into the JM for higher resolutions (larger than CIF), particularly for video formats that use interlaced scanning.

# References

[JVT-B053]	Mathias Wien, ' ABT Coding for Higher Resolution Video ', Document JVT-B053, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Geneva, Switzerland, 29 Jan -1 Feb.. 2002.

[VCEG-O59] 	Peter Borgwardt, 'Core Experiment on Interlaced Video Coding', Document VCEG-O59, ITU-T Video Coding Experts Group (VCEG) Meeting 15, Pattaya, Thailand, 4-6 Dec 2001.

[VCEG-O31]	Mathias Wien and Achim Dahlhoff, 'Intra Coding using Variable Block Sizes',
Document VCEG-O31, ITU-T Video Coding Experts Group (VCEG) Meeting 15, Pattaya, Thailand, 4-6 Dec 2001.

[VCEG-O30]	Mathias Wien and Jens-Rainer Ohm, 'Simplified Adaptive Block Transforms',
Document VCEG-O30, ITU-T Video Coding Experts Group (VCEG) Meeting 15, Pattaya, Thailand, 4-6 Dec 2001.

[VCEG-N85] 	Peter Borgwardt, 'Core Experiment on Interlaced Video Coding', ITU-T Q6/16 VCEG, Document VCEG-N85r1, Santa Barbara, CA, USA, Sep. 2001.

[VCEG-N49]	M. Wien and A. Dahlhoff, 'New ABT Results using CABAC', ITU-T Q6/16 VCEG, Doc. VCEG-N49, Santa Barbara, CA, USA, Sep. 2001.

[VCEG-M62]	M. Wien and A. Dahlhoff, 'Adaptive Block Transforms', ITU-T Q6/16 VCEG, Doc. VCEG-M62, Austin, TX, April 2001.

[VCEG-M33]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' Document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[VCEG-L15]	M.Wien and C.Mayer, ‘ABT Coding Elements’, ITU-T VCEG, 
 	Document VCEG-L15, Eibsee, Germany, January 2000

# Appendix

## IP BB BB PP

f1 car ccir field TML90 IBBP

QP     Rate	PSNR

24   737365	29.01

16  2656229	34.57

f1 car ccir field ABT90 IBBP inter

QP     Rate	PSNR

24   836271	29.61

16  2594077	34.90

f1 car ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

24   854725	29.74

16  2619999	35.08

canoa ccir field TML90 IBBP

QP     Rate	PSNR

24   782908	28.69

16  2561851	33.07

canoa ccir field ABT90 IBBP inter

QP     Rate	PSNR

24   854448	29.19

16  2645725	33.34

canoa ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

24   859093	29.49

16  2764750	33.51

rugby ccir field TML90 IBBP

QP     Rate	PSNR

24   789702	29.35

16  2280146	34.48

rugby ccir field ABT90 IBBP inter

QP     Rate	PSNR

24   855213	29.84

16  2374594	34.96

rugby ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

24   864807	30.10

16  2393694	35.25

tempete ccir field TML90 IBBP

QP     Rate	PSNR

24   502834	28.05

16  1950447	33.67

tempete ccir field ABT90 IBBP inter

QP     Rate	PSNR

24   590152	28.88

16  1991108	34.10

tempete ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

24   591986	28.89

16  1995187	34.11

football ccir field TML90 IBBP

QP     Rate	PSNR

24   443390	30.72

16  1296063	35.31

football ccir field ABT90 IBBP inter

QP     Rate	PSNR

24   484352	31.21

16  1384681	35.79

football ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

24   489597	31.41

16  1395154	36.13

bus ccir field TML90 IBBP

QP     Rate	PSNR

24   548505	28.60

16  1684067	33.91

bus ccir field ABT90 IBBP inter

QP     Rate	PSNR

24   626953	29.17

16  1772842	34.47

bus ccir field ABT90 IBBP inter&intra

QP     Rate	PSNR

24   616777	29.36

16  1756968	34.63

## IP PP PP

f1 car ccir field TML90 IP

QP     Rate	PSNR

24  1064886	30.14

16  3901527	35.79

f1 car ccir field ABT90 IP inter

QP     Rate	PSNR

24  1176804	30.77

16  3607754	36.10

f1 car ccir field ABT90 IP inter&intra

QP     Rate	PSNR

24  1402222	31.11

16  4103269	36.58

canoa ccir field TML90 IP

QP     Rate	PSNR

24  1108726	29.54

16  3756193	33.95

canoa ccir field ABT90 IP inter

QP     Rate	PSNR

24  1173085	30.20

16  3973453	34.31

canoa ccir field ABT90 IP inter&intra

QP     Rate	PSNR

24  1275954	30.53

16  4315838	34.69

rugby ccir field TML90 IP

QP     Rate	PSNR

24  1114671	30.28

16  3250291	35.48

rugby ccir field ABT90 IP inter

QP     Rate	PSNR

24  1195612	30.93

16  3340728	36.13

rugby ccir field ABT90 IP inter&intra

QP     Rate	PSNR

24  1262094	31.22

16  3444645	36.40

tempete ccir field TML90 IP

QP     Rate	PSNR

24   826496	28.76

16  3066808	34.67

tempete ccir field ABT90 IP inter

QP     Rate	PSNR

24   944440	29.66

16  3078028	35.05

tempete ccir field ABT90 IP inter&intra

QP     Rate	PSNR

24   953584	29.69

16  3096790	35.09

football ccir field TML90 IP

QP     Rate	PSNR

24   601620	31.56

16  1764695	36.13

football ccir field ABT90 IP inter

QP     Rate	PSNR

24   647706	32.21

16  1857651	36.88

football ccir field ABT90 IP inter&intra

QP     Rate	PSNR

24   685082	32.42

16  1911550	37.10

bus ccir field TML90 IP

QP     Rate	PSNR

24   648982	29.40

16  2125883	34.96

bus ccir field ABT90 IP inter

QP     Rate	PSNR

24   754399	30.34

16  2272111	35.68

bus ccir field ABT90 IP inter&intra

QP     Rate	PSNR

24   771061	30.43

16  2297254	35.74

## II II II

f1 car ccir field TML90 III intra

QP     Rate	PSNR

24  3294551	31.11

16  7547599	36.86

f1 car ccir field ABT90 III intra

QP     Rate	PSNR

24  3124545	31.29

16  7228862	37.08

canoa ccir field TML90 III intra

QP     Rate	PSNR

24  2854421	30.61

16  7217065	35.22

canoa ccir field ABT90 III intra

QP     Rate	PSNR

24  2665164	30.87

16  7132787	35.37

rugby ccir field TML90 III intra

QP     Rate	PSNR

24  2785558	31.40

16  6737170	36.94

rugby ccir field ABT90 III intra

QP     Rate	PSNR

24  2617677	31.65

16  6480278	37.21

tempete ccir field TML90 III intra

QP     Rate	PSNR

24  4029909	29.98

16  8444831	35.93

tempete ccir field ABT90 III intra

QP     Rate	PSNR

24  3794610	30.15

16  8216266	36.06

football ccir field TML90 III intra

QP     Rate	PSNR

24  1700819	32.46

16  4361774	37.35

football ccir field ABT90 III intra

QP     Rate	PSNR

24  1622178	32.71

16  4213422	37.64

bus ccir field TML90 III intra

QP     Rate	PSNR

24  2890290	30.31

16  6716814	36.16

bus ccir field ABT90 III intra

QP     Rate	PSNR

24  2760994	30.51

16  6528546	36.36

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Robert Bosch GmbH |  |  |
| Mailing address | PO Box 106050 70049 Stuttgart |  |  |
| Country | Germany |  |  |
| Contact person | Ulrich Benzler |  |  |
| Telephone | +49-5121-493087 |  |  |
| Fax | +49-5121-49173087 |  |  |
| Email | ulrich.benzler@de.bosch.com |  |  |
| Place and date of submission | Geneva, Switzerland January 29th 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ABT Coding Results for Interlaced ITU-R 601 Video |  |
| Contribution number |  | JVT-B065 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: