| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B066 Filename: JVT-B066.doc Generated: 2002-01-22 |
| --- | --- |

| Title: | 1/8-pel Motion Vector Resolution for Interlaced Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Thomas Wedi Institute of Communication Theory and Signal Processing University of Hanover, Germany | Tel: Email: | +49 511 762 5304 wedi@tnt.uni-hannover.de |
| Source: | VCEG |  |  |

_____________________________

# Summary

In the past, several VCEG contributions were presented that analyze the coding efficiency of 1/8-pel motion vector resolution for CIF and QCIF sequences (e.g. VCEG-M45, VCEG-M46, D.361). This contribution analyses the coding efficiency of 1/8-pel motion vector (MV) resolution for interlaced sequences. Therefore the TML-9 was used without any modifications. In order to allow interlaced video coding with TML-9, the fields of the interlaced sequences were treated as separate frames. Thus, a field based coding mode was used.

The experimental results show that the sequences can be divided into two different types:

- Sequences with moderate motion and high spatial detail like Bus, Mobile & Calendar, Flower Garden, Tempete, and Waterfall:
For this kind of sequences a significant coding gain is achieved for 1/8-pel compared to 1/4-pel MVs that is higher for higher bit-rates. For higher bit-rates a gain between 0.5 and 1.0 dB is obtained.

- Sequences with active/chaotic motion, like Football, Rugby, F1, Canoa:
Since the motion for this kind of sequences can not be described with block-based motion vectors, no coding gain is obtained with 1/8-pel motion vector resolution. The PSNR difference for this kind of sequences is in a range of 0.0 and -0.2 dB.

# Experimental Results

In the following two tables the interlaced test-sequences, which are used for the experimental results are shown. Table 1 shows the sequences with moderate motion and high spatial detail. Their motion can be well described with block based motion vectors. Thus, a higher motion vector resolution should be beneficial for this kind of sequences.

| Test-sequence | VQEC-Name | Rate |
| --- | --- | --- |
| Mobile & Calendar | src15 | 50 Hz |
| Waterfall | src18 | 60 Hz |
| Tempete | src22 | 60 Hz |
| Bus | - | 60 Hz |
| Flower Garden | - | 50 Hz |

Table 1: Interlaced Sequences with moderate motion an high spatial detail

In Table 2 the interlaced test-sequences with active or chaotic motion are shown. Their motion can not be described correctly with block-based motion vectors. Thus, a higher motion vector resolution should bring no improvements in coding efficiency.

| Test-sequence | VQEC-Name | Rate |
| --- | --- | --- |
| Canoa | src5 | 50 Hz |
| F1 | src6 | 50 Hz |
| Rugby | src9 | 50 Hz |
| Football | src19 | 60 Hz |

Table 2: Interlaced Sequences with active/chaotic motion

For the experimental results RD-motion estimation, CABAC, 4 reference frames (fields), and no B-frames were used. The RD-plots of the test-sequences in Table 1 are given in Appendix A and for the test-sequences of Table 2 are given in Appendix B. Each RD-plot contains the coding results for 1/4-pel and for 1/8-pel motion vector resolution.

# References

M45:	T. Wedi, "1/8-pel Displacement Vector Resolution for TML-6", ITU-T Q.6/SG16, doc. VCEG-M45, Austin, Texas, USA, April 2001

M46:	T. Wedi, "Results on Complexity performance investigations: Displacement Vector Resolution and Interpolation Filter tap Size", ITU-T Q.6/SG16, doc. VCEG-M46, Austin, Texas, USA, April 2001

D.361:	G. Bjontegaard, “Motion Compensation with 1/4 pixel accuracy”, ITU-T SG16/Q15, Geneva, Feb. 00.

Appendix A: RD-plots for sequences with moderate motion and high spatial detail

PSNR Y

PSNR Y

PSNR Y

PSNR Y

PSNR Y

Appendix B: RD-plots for  sequences with active motion

PSNR Y

PSNR Y

PSNR Y

PSNR Y

Canoa