| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B067 Filename: JVT-B067.doc Generated: 2002-01-zz |
| --- | --- |

| Title: | Quantization Tools for High Quality Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Peter Kuhn, and Yoichi Yagasaki Gate City Osaki East Tower 20F 1-11-1 Osaki Shinagawa-ku, Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp pkuhn@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

The extension of quantization to support high quality video was proposed in Pattaya. It proposed weighting matrix and extended quantization table. In this contribution, the results of experiments are shown. It is also studied how to integrate with low complexity transform, which is based on 16-bits arithmetic.

# Proposal

High quality video is very important target for JVT codec. High quality video is not only for professional equipments but also for consumer applications. The application for entertainment, like Digital TV and DVD, requires it. Therefore, quantization should be extended even for 8-bit video (9-bit residual).

## Weighting matrix

4x4 weighting matrix W(i,j)（i,j=0,1,2,3）is introduced to give an weight on each frequency components. Encoder can adaptively use weighting matrix and encode it in bitstream. New quantization scale QQP(i, j), which is 4x4 matrix of q_scale, is defined as follows.

QQP(i,j)=QP+W(i,j)

QQP is clipped in [0:31]. Then quantization is defined as follows.

- LEVEL = (KxA(QQP) + fx220)/220

- Dequantization is defined as follows.

- K' = LEVELxB(QQP)

Since we don’t define new couple of A(QP) and B(QP), integer transform and quantization can be computed with 32-bit arithmetic. Even if bit length of transform and quantization is changed, e.g. 16-bit arithmetic, this method can be applied in the same approach.

## Extension of quantization range

In VCEG-O52, It was shown that the highest quality of TML 8 was limited and lower than that of MPEG-2. We can see artifacts for simple flat image even if QP=0 is used. For high quality applications, codec should be subjectively loss less for such a simple image. Extension of QP range to –8 was proposed. In addition, VCEG-O52 showed that we don’t need to extend the dynamic range for all of the coefficients. Since lower frequency is important for visual quality and it was sufficient to extend the dynamic range only for lower frequencies.

## Low complexity transform

16-bit architecture of transform is studying in the AHG on Transform to reduce the complexity of transform. Two different approaches are proposed. One is based on the similar architecture in TML and the other uses multiplier free design. Since the weighting matrix proposed in VCEG-O52 is not depends on the specific transform design. It can be applied to both transform architecture. The periodic quantization was also studied in the AHG. It formulates A(QP) and B(QP) as follows,

A(QP)=Amantissa(QP)⋅2Aexponent(QP)B(QP)=Bmantissa(QP)⋅2Bexponent(QP)

Quantization can be formulated as follows.

LEVEL(i,j)=sign(f(i,j))((f(i,j)//2fdctshift×Aman[QP%6]+qpconst)2shift(QP)

If we replace QP by QQP, it is possible to integrate with this quantizer design.

## Quantization for DC block

For Intra 16x16 mode, DC values of 16 luminance blocks further form a 4x4 luminance DC block. Additional 4x4 transform is applied for this block. Hadamard transform is used for the second transform. Since DC information important for visual quality, it is better to use smaller QP. This approach is similar to intra_dc_precision in MPEG-2. WM for DC transform is a good way to increase a quantization precision, because it enables encoder only to increase the quantization range for the important coefficients.

However, as the Luminance DC 4x4 transform has an inverted order for the inverse transform and inverse quantization process, there is no direct way of implementing the DC weighting matrix to control individual coefficients. The Luminance DC 4x4 transform and quantization is performed in the following order:

TML software performs the DC DCT in the following order:

DCT->Q->IDCT->IQ

Therefore, obviously the quantization process is in the DCT^2 (?) domain, and the inverse quantization process is in the standard DCT domain. The problem arises for the DC weighting matrix where the coordinates of the weighting coefficients are transformed through the IDCT process. If all weighting coefficients are of the same value, there is no problem. However, if the weighting varies for each frequency, using the same weighting matrix for Q and IQ, will cause misalignment of A(QP) and B(QP) for the coefficients.

It is mentioned in Nokia's proposal (VCEG-N25.doc), that it is better to do the IDCT before IQ for the lowest possible dynamic precision during inverse transform computations.

In this contribution, flat WM is used for 4x4 DC transform. This is only to increase the precision of DC coefficients for Intra 16x16 mode.

## Transmission of matrices

WM may be better to transmit out of band. WM should be changed higher than picture layer. It should be transmitted through Parameter Sets in NAL. The example of the syntax can be described as follows,

Parameter Sets

quant_matrix_extension() {

enum	load_quantiser_matrix

if(load_quantiser_matrix) {

int*16		quantiser_matrix[16]

}

enum	load_chroma_quantiser_matrix

if(load_chroma_quantiser_matrix) {

int*16		chroma_quantiser_matrix[16]

}

}

# Simulations

The extension of quantization in VCEG-O52 is implemented on TML9.4 software.

## Weighing Matrix

In this experiments, efficient of Weighting Matrix was verified. The following Weighting Matrix in Fig 1 is applied to the 4x4 transform for luminance block. This matrix is converted from WM for Intra in MPEG-2 and 4. The same matrix is used for all of pictures. Flat matrix is used for 4x4 transform for luminance DC block and Chrominance block. In this experiment, quantization range is not extended.

0

1

4

5

1

4

5

6

4

5

6

8

5

6

8

1222

Fig 1 Weighting Matrix

In order to check the characteristic of Weighting Matrix, we encoded an intra frame with the Q-scale 4, 8, 12, 16, 20, 24 and 28. Circle zone plate was used for this test. The coding parameters are described in Table 1.

| test sequence | Circle Zone Plate (1Frame) |
| --- | --- |
| Intra period | Only First Frame |
| Q-Scale | 4, 8, 12, 16, 20, 24, 28 |
| Hadamard transform | used |
| slice mode | off |
| r-d optimization mode decision | off |

Table 1 Test Condition of INTRA Frame encoding

Fig 2 R-D curve of INTRA Frame by Circle zone plate

The results are shown in Fig 2, Fig 3 and Fig 4.

Fig 2 shows the R-D curve of TML and TML +WM. Blue line is TML9 + WM and Red is TML. Fig 2 shows that RD curve is improved for the case TML + WM. It improves about 0.5 dB for over all bit rate.

Fig 3 and Fig 4 show BMP images of the decoded pictures for two cases. These two images are encoded at the same bit rate about 400 kbps. PSNR of Fig 3 is 29.25dB and Fig 4 is28.69dB PSNR of Fig 3 is improved about 0.5dB than that of Fig 4. Comparing the visual quality, the subjective quality of Fig 3 is better. The subjective quality is improved at high frequency and the edge of circle at low frequency. Left bottom and top right on Fig 4, some artifacts can be seen. However in Fig 3, the artifact is not visible at the same parts.

Fig 3 Decoded picture of TML+WM (QP=16)

Fig 4 Decoded picture of TML (QP= 20)

## Extension of Quantization Range

### Test Conditions

Table 2 describes the test conditions.

| Sequence | Ramp signal (Fig.3) |
| --- | --- |
| Frame size | Rec 601. |
| Number of Frames | 1 |
| QPP | 8 (equivalent to QP=0) |
| Weighting Matrices (WM) | Described Below |

Table 2: Test Condition

Fig 5 shows the ramp signal used for simulation tests.

Fig 5 Ramp Signal Encoded Picture with QP=0

The weighting matrices are numbered as following.

Weighting Matices:

0000000000000000−8000000000000000−8−800−800000000000−8−8−80−8−800−80000000

(0)		(1)		(2)			(3)

−8−8−8−8−8−8−80−8−800−8000−8−8−8−8−8−8−8−8−8−8−80−8−800−8−8−8−8−8−8−8−8−8−8−8−8−8−8−80−8−8−8−8−8−8−8−8−8−8−8−8−8−8−8−8

(4)			(5)			(6)			(7)

Fig 6 The Example of Weighting Matrix with High Precision for Low Frequency

## Results

Table 3 shows results for PSNRY and bit-stream file size for adaptive mode decision in low complexity mode.

Table 4 shows results for 4x4 prediction only, with 16x16 intra prediction disabled. The 4x4 transform quantization weighting matrix is varied from (0) to (7).

| Sequence : ramp |  |  |  | Sequence : paris |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| WM | PSNRY | bits |  | WM | PSNRY | bits |
| no_weight | 54.18 | 2991 |  | no_weight | 49.17 | 71050 |
| 0 | 54.67 | 5152 |  | 0 | 49.18 | 71240 |
| 1 | 54.67 | 5152 |  | 1 | 49.35 | 73574 |
| 2 | 58.65 | 5588 |  | 2 | 49.73 | 78080 |
| 3 | 59.53 | 7565 |  | 3 | 50.38 | 84414 |
| 4 | 59.82 | 12751 |  | 4 | 51.4 | 92640 |
| 5 | 63.95 | 16559 |  | 5 | 52.49 | 98150 |
| 6 | 63.96 | 16566 |  | 6 | 53.55 | 101922 |
| 7 | 63.55 | 18103 |  | 7 | 54.26 | 103669 |

Table 3: Mode Decision Adaptive (Low Complexity Mode)

| Sequence : ramp |  |  |  | Sequence : paris |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| WM | PSNRY | bits |  | WM | PSNRY | bits |
| no_weight | 52.88 | 18763 |  | no_weight | 49.18 | 70842 |
| 0 | 52.88 | 18763 |  | 0 | 49.18 | 70842 |
| 1 | 52.89 | 25612 |  | 1 | 49.37 | 73319 |
| 2 | 57.16 | 33943 |  | 2 | 49.74 | 77734 |
| 3 | 57.16 | 33943 |  | 3 | 50.39 | 84001 |
| 4 | 57.16 | 33944 |  | 4 | 51.41 | 92151 |
| 5 | 57.16 | 33944 |  | 5 | 52.52 | 97692 |
| 6 | 57.16 | 33944 |  | 6 | 53.57 | 101463 |
| 7 | 57.16 | 33944 |  | 7 | 54.28 | 103195 |

Table 4: Fixed to 4x4 prediction only

# Discussions

In this experiments, it was verified that TML+WM is able to reduce bit rate keeping the same visual quality. Human visual system is not sensitive for diagonal direction. Reducing the bits for the frequency, it is possible to reduce bit rate. In addition, WM gives a flexibility of quantization. TML quantization can only use 31 level of QP, however, the final quantized value is not limited to the combination of A(QP) and B(QP).

This proposal is more useful at high bit rate. It should be tested at high bit rate and test conditions should be defined to evaluate high quality video.

With QP=0, the block artifacts are visually significant, and obtrusive. However, with the extension of the Quantization Table to QP=-8 (QPP=0), the precision can be further extended. The Quantization Weighting Matrix adds flexibility by combining with the extended quantization to allow for increasing precision for individual coefficients. For example, by weighting the Luminance DC 4x4 transform quantization matrix alone, the DC precision can be enhanced to QPP=0. If additional AC coefficients need to be weighted to improve the visual quality, the weighting matrix for the first transform of Luminance block can be modified as shown in the simulation experiment.

Results of subjective testing show that, lower frequency AC coefficients, not only DC coefficient, need to be weighted by -8 in order to remove the block artifacts from the encoded ramp signal. This removes the visually noticeable block artifacts, as well as improving the PSNRY results to a nearly saturated value. Further weighting of other AC coefficients is virtually insignificant towards visual quality improvement. In contrast, for MPEG2 coding it is only necessary to increase the DC precision in order to remove the visually noticeable block artifacts. However, as mentioned above, simulation results show that for TML, lower frequency AC coefficients also need to have increased precision to improve the subjective quality.

Therefore, it can be said that the quantization weighting matrix adds flexibility to the transform and quantization process by providing substantial coding efficiency without sacrificing the visual quality. The extension of the quantization table, combined with the quantization matrix, is a necessary addition to the TML tool set, for high quality video coding.

## TML Software Problem

TML9.4 software crashes when encoding complex sequences such as a circular zone plate (CIF size) or a random signal. The problem seems to be a memory leak. Further investigation is required to solve the problem. This problem must be fixed before we study the tools for high bit rate.

# Conclusions

The extensions of quantization to support high quality video are proposed. It is studied how to integrate with low complexity transform, which is based on 16-bits arithmetic. The quantization weighting matrix adds flexibility to the transform and quantization process by providing substantial coding efficiency without sacrificing the visual quality. The extension of the quantization table, combined with the quantization matrix, is a necessary addition to the TML tool set, for high quality video coding.

The advantage is more obvious at high bit rate and it should be tested at high bit rate.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20 F 1-11-1 Osaki Shinagawa-ku, Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan on Jan. 23rd in 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Extension of quantization for high quality video |  |
| Contribution number |  | 067 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: