| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B069 Filename: JVT-B069.doc Generated: 2002-01-zz |
| --- | --- |

| Title: | Group of Pictures for JVT Codec |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Noriaki Oishi, and Yoichi Yagasaki Gate City Osaki East Tower 20F 1-11-1 Osaki Shinagawa-ku, Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp oishi@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

Group of Pictures (GOP) layer was proposed in VCEG-O53, MPEG m7736 at Pattaya. It proposed video layer syntax to support random access, editing and etc. The AHG on GOP syntax was formed to study how to incorporate GOP syntax into the JVT codec considering VCEG-O53, MPEG-2 and the H.26L

design. During the discussion, it was suggested that there are two conceptual layers, VCL (Video Coding Layer) and NAL (Network Adaptation Layer) in H.26L design. VCL encodes MBs to Slice only and NAL encapsulate them for transmission and storage. In this contribution, group of pictures for JVT codec is proposed considering the H.26L codec design.

# Group of Picture Layer

A GOP consists of one or more consecutive pictures. GOP layer convey information attribute to a GOP. In the bitstream order, the first picture in a GOP is always an Intra picture. The functionalities of GOP are as follows,

Convey information attributed to group of picture

Random Access Functionality

Many applications require random access functionality. It is not only for storage applications. The examples of applications are,

- Random access for storage application, like DVD

- Channel hopping for broadcasting

- Splicing

- Editing

- etc.

Re-synchronization Tool for Error Recovery

The decoder can synchronize at GOP layer after encountering error. GOP layer can be used as a tool for error recovery.

- GOP does not need to be a mandatory for all of the application. It can be an optional layer. If the application doesn’t need to support this functionality, it can be skipped. But it should be supported by the application, which require the above functionalities. Many MPEG1&2 applications, such as DVD, Video CD use the GOP structure for Random Access functionality. The GOP structure is confirmed as a useful tool in real applications. It should be supported at an appropriate layer in the JVT codec.

# Requirements

The requirements to support random access functionality are as follows.

- Random access point must be identified quickly with a reasonable cost

- Timing information: Absolute display time is necessary to identify the target frame.

- Identify whether reconstructed frame is not corrupt due to the loss of reference frame.

The key technologies of GOP layer to support random access are as follows.

- Unique start code: In order to find random access point efficiently, unique start code must be inserted at all random access point. Using the unique code, the decoder can find random access point without parsing a bitstream.

- Time code: Absolute display time is necessary at random access point to identify the target frame.

- Dependency between GOP: The flag to identify whether the GOP is dependent to the other GOP. If one of the frame in the GOP refers to the frame in other GOP, the frame will not be decoded correctly. The flag is necessary for decoder to identify whether the reconstructed frame is correct or not.

# Network Adaptation Layer

During the discussion in the AHG, it was suggested that there are two conceptual layers, VCL (Video Coding Layer) and NAL (Network Adaptation Layer) in H.26L design. VCL encodes MBs to Slice only and NAL encapsulate them for transmission and storage.

NAL also deal with startcode. In case of NAL for RTP, all slices are aligned to RTP packet header, and it can be used as a startcode for VCL. Startcode is necessary for bit serial communication, like MPEG-2 System. It is needed for some applications.

The NAL receives coded elements lower than MB and encapsulate it to slices and form pictures, GOPs, and sequences for transmission and/or storage. In this framework, all information that was traditionally conveyed in high-level headers is transmitted out of band. Parameter sets is define to convey those high-level header information. Encoder and decoder maintain a synchronized those parameter sets.

If we follow this H.26L design, GOP layer proposed in VCEG-O53 should be supported at NAL. In this document, the syntax elements in GOP except startcode are proposed to convey in the Local Parameter Sets. The solution for startcode is proposed in JVT-B-070.

# Proposal

In VCEG-N52, the Parameter Sets, which include the information of Sequence/GOP/Picture layers, is proposed. It is transmitted out of band. The Parameter Sets in VCEG-N52 seems to assume that the parameters are global. However, GOP carries information in the group of pictures and the synchronization and the link between the Parameter Sets and corresponding part of VCL stream. In this contribution, “Local” Parameter Sets is proposed for this purpose.

## Syntax

The syntax elements for GOP in the Local Parameter Sets are as follows,

Local Parameter Sets

### Group of Picture Parameters

GOP() {

enum useAbsoluteDisplayTime;

if(useAbsoluteDisplayTime)

Absolute_Display_Time();  (Optional)

enum closed_gop;

enum broken_link;

}

Absolute_Display_Time() {

enum drop_frame_flag;

enum time_code_hours;

enum time_code_minutes;

enum time_code_second;

enum time_code_pictures;

}

## Semantics

Absolute_Display_Time: This is an optional element and contains drop_frame_flag, time_code_hours, time_code_minutes, marker_bit, time_code_seconds and time_code_pictures as shown in Table1. The parameters correspond to those defined in the IEC standard publication 461 for “time and control codes for video tape recorders”. The time refers to the first frame (in display order) in GOP.

The drop_frame_flag can be set to either ‘0’ or ‘1’. It may be set to ‘1’ only if the frame rate is 29,97Hz. If it is ‘0’ then pictures are counted assuming rounding to the nearest integral number of pictures per second, for example 29,97Hz would be rounded to and counted as 30Hz. If it is ‘1’ then picture numbers 0 and 1 at the start of each minute, except minutes 0, 10, 20, 30, 40, 50 are omitted from the count.

The rest of parameter in tim_code are time_code_hours, time_code_minutes, time_code_seconds and time_code_pictures. The time_code_pictures specifies time based on frame rate. Since the display time of frame is independent from frame rate, we may not use time_code_pictures and fix to ‘0’. So the precision of time_code is second.

NOTE -	The information carried by time_code plays no part in the decoding process.

Table 1 — time_code

| time_code | range of value |
| --- | --- |
| drop_frame_flag |  |
| time_code_hours | 0 - 23 |
| time_code_minutes | 0 - 59 |
| time_code_seconds | 0 - 59 |
| time_code_pictures | 0 - 59 |

closed_gop -- This is a one-bit information which indicates the nature of the predictions used in any frames in the current GOP. The closed_gop is set to ‘1’ to indicate that all of frames following the GOP header have been encoded without predicting the frames previous to GOP header or intra coding. This bit is provided for use during any editing which occurs after encoding. If the previous pictures have been removed by editing, broken_link may be set to ‘1’ so that a decoder may avoid displaying these frames predicting frames in other GOPs. However if the closed_gop bit is set to ‘1’, then the editor may choose not to set the broken_link bit as all frames in the GOP can be correctly decoded.

broken_link -- This is a one-bit information which shall be set to ‘0’ during encoding. It is set to ‘1’ to indicate that all frames following the group of picture header may not be correctly decoded because the reference frame which is used for prediction is not available (because of the action of editing). A decoder may use this flag to avoid displaying frames that cannot be correctly decoded.

## NAL for RTP

Since GOP conveys information in the group of pictures, the Local Parameter Sets must be synchronized with the GOP in VCL bitstream. When the Local Parameter Sets is used, the parameter sets should be allowed to transmit in the payload in the same packet of corresponding VCL stream. In case of more complex system, this synchronization may be guaranteed in other ways.

## NAL for MPEG-2 System

The detail proposal to convey JVT bitstream on MPEG-2 System is described in JVT-B-070. NAL for MPEG-2 adds unique startcode and parameter sets before each AU. Then, all AUs are mapped on PES packets.

# Summaries

In this document, Group of Pictures layer is proposed. Considering the H.26L design concept, GOP is introduced in Network Adaptation Layer. The Local Parameter Sets of GOP contains information in the corresponding group of pictures. Therefore, it must be synchronized with the corresponding part of VCL stream. It is proposed to allow transmitting the parameter sets in the payload in the same packet. The case of NAL for MPEG-2 System is also proposed in JVT-B070.

Since GOP supports important functionalities, it should be included in JVT codec.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20 F 1-11-1 Osaki Shinagawa-ku, Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan on Jan. 23rd in 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Group of Picture for JVT codec |  |
| Contribution number |  | 069 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: