| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B071 Filename: JVT-B071r2.doc Generated: 2002-02-25 |
| --- | --- |

| Title: | Adaptive Frame/Field Coding for JVT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang, Krit Panusopone, Rajeev Gandhi, Yue Yu and Ajay Luthra Motorola Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3893 liwang@gi.com |
| Source: | Motorola Inc. |  |  |

## Abstract

The document presents the computer simulation results for core experiments A (frame coding), B (field coding) and E (picture level adaptive coding) defined in [1] for interlace video coding.  Simulations were carried out for various interlace video sequences, including the six common sequences for interlace testing [1]. The advantage of adaptive coding over frame or field coding is obvious. It is recommended that adaptive coding be adopted in JVT.

1. Introduction

Interlace video sequence consists of two fields, scanned at different time instants. Nature of interlace video sequence and its possible impact on H.26L (JVT) have been discussed in [3-7]. Various coding tools for interlace sequences and their performances were presented in [8-9]. Ad Hoc Group (AHG) on interlace coding was established in the first JVT meeting, Pattaya, in 12/2001. One of major goals for the AHG on interlace coding is to conduct the core experiments in evaluating these interlace coding tools.

This document presents the computer simulation results for core experiments A, B and E defined in [1]. Core experiment A is frame-based coding, B is field-based coding, and E is picture level adaptive frame/field coding.  In frame or field coding, an interlaced video sequence is always coded with either frame or field structure irrespective of the content. In picture level adaptive frame/field coding, however, an input frame can be coded as one frame picture or two field pictures, depending upon the content.

Simulations were carried out for various interlace video sequences, including the six common test sequences [1]. The simulation results demonstrate that either frame or field coding may lead to coding inefficiency because some video sequences, or segments of a sequence, may favor frame coding while other sequences, or other segments of a sequence, may favor field coding. On the other hand, adaptive coding makes a decision regarding frame or field coding based on the content. It is shown that adaptive coding guarantees a performance over frame and field coding.

2. Core Experiments (A, B and E)

Core experiments A, B and E are respectively frame, field and picture level adaptive frame/field coding. The picture structures proposed for the three core experiments are as follows

- A) Frame Coding

  - I P P P P …

  - I B B P B B P …,

- B) Field Coding

  - (IP) (PP) (PP) (PP) (PP)…,

  - (IP) (BB) (BB) (PP) (BB) (BB) (PP)…,

- E) Adaptive Frame/Field Coding

  - I(or IP) P(or PP) P(or PP) P(or PP) …,

  - I(or IP) B(or BB) B(or BB) P(or PP) B(or BB) B(or BB) P(or PP) …,

Note that picture types in bracket are for field coding. If field coded, a I frame is coded as one I field picture and one P field picture, a P frame as two P field pictures and a B frame as two B field pictures. It should be pointed that in general,

- A I frame can be coded as one I frame, or two I fields, or one I field and one P field,

- A P frame can be coded as one P frame, or two P fields, or one P field and one B field, and

- A B frame can be coded as one B frame, or two B fields.

However, in this contribution, the picture structures used in simulations follow the core experiments [1].

3. Coding Methods

The interlace coding tools presented in [8] have been integrated into the current TML9.0 [2]. The new rules, as compared to the reference TML9.0, for frame, field and adaptive coding are described as follows.

3.1 Frame Coding

In frame coding, all the frames of a sequence are encoded as frame picture. Hence, there are no major changes to the current TML9.0 [2]. The only necessary modification is

- Frame coding mode is indicated in the sequence header.

3.2 Field Coding

In field coding, a frame is encoded as two field pictures. The rules for field coding are as follows.

- The sequence header indicates the field-coding mode.

- Two field of a frame are coded sequentially.

- The reference fields can be any coded I or P field.

- The code numbers assigned for field-based references stored in reference frame (field) buffer are slightly different from for frame-based references. Specifically, the code numbers of 0, 1, 2, 3, …, are grouped into pairs of (0,1), (2,3), (4,5), …. These code number pairs are assigned to the pairs of adjacent reference fields according to their distances to the current field (the field to be coded). Note that a pair of the adjacent fields is not necessarily from the same frame. For each pair of reference fields, the field of the same parity as the current field is given the smaller number of the code number pair assigned for the pair of reference fields, as shown in Fig. 1.

- The skipped MB (copy mode) is reconstructed by copying the co-located MB in the most recently coded (past) I or P field of the same field parity.

Fig. 1. Reference frame numbers.

3.3. Picture Level Adaptive Frame/Field Coding

In picture level adaptive frame/field coding, a frame can be encoded as one frame picture or two field pictures. The rules for picture level adaptive coding include

- Picture header indicates whether the current frame is coded as one frame or two fields.

- For field coding, two fields of a frame are coded sequentially.

- For field coding, the reference fields can be any coded I field or P field.

- The code numbers are assigned to the reference fields stored in reference frame (field) buffer following the same rule as field coding (see Fig. 1).

- If in field coding, the skipped MB is reconstructed by copying the co-located MB in the most recently coded (past) I or P field of the same field parity.

The criterion for selecting either frame or field coding per frame is RD-based. The cost function is defined as

cost=Distortion+λBitRate.                                                 (1)

3.4 Direct Mode

Direct mode in field and adaptive coding requires some additional rules. Direct mode in field picture uses bi-directional prediction similar to direct mode in frame picture. Two MVs (forward and backward) of direct mode MBs are derived from forward MVs of its collocated MB in the future reference of I or P. Specifically, calculation of MVs in direct mode follows one of the four following cases:

Case 1: collocated MB uses frame MV and current MB uses frame MV

This mode is the same direct mode in frame picture and uses the same rule as provided in TML9.0  [2].

Case 2: collocated MB uses field MV and current MB uses field MV

Two MVs of direct mode MB in B field are calculated from the forward MV of the collocated MB in the future reference field of the same parity. For the 1st field, the forward MV of the collocated MB will always point to one of the previously coded I or P fields, as shown in Fig. 2. The forward reference field for the direct mode MB will be the same as pointed by the forward MV of the collocated MB. The backward reference field is the 1st field of the future reference frame. The forward and backward MVs (MVF,i,MVB,i) for the direct mode MB are calculated as follows,

MVF,i=TRB⋅MViTRD,iMVB,i=(TRB−TRD,i)⋅MViTRD,i                                                (2)

where the subscript, i, is the field index (1 for the 1st field and 2 for the 2nd field), and MVi is the forward MV of the collocated MB in field i of the future reference frame. TRB is the temporal distance between the current B field and the reference field pointed by the forward MV of the collocated MB in field interval, TRD,iis the temporal distance between the future reference field (i) and the reference field pointed by the forward MV of the collocated MB in field interval.

Fig. 2. Both the MB in current B and its collocated MB in the future reference of I or P are in field mode, and the forward MV of the collocated MB in the 1st field of the future reference frame always points to one of the previously coded I or P fields.

For the 2nd field, the forward MV of the collocated MB may point to one of the previously coded I or P fields. Calculation of the forward and backward MVs therefore follows the same equation (2) above. However, it is also possible that the forward MV of the collocated MB in the 2nd field of the future reference frame points to the 1st field of the same frame, as shown in Fig. 3. In this case, the two MVs (MVF,2,MVB,2) of the direct mode MB are calculated as follows:

MVF,2=−TRB⋅MV2TRD,2MVB,2=−(TRB+TRD,2)⋅MV2TRD,2                                                  (3)

Note that both MVs (MVF,2,MVB,2) are now backward MVs, pointing to the 1st field and the 2nd field of the future reference frame respectively.

Fig. 3. Both the MB in current B and its collocated MB in the future reference of I or P are in field mode, and the forward MV of the collocated MB in the 2nd field of the future reference frame points to the 1st field of the same frame.

Case 3: collocated MB uses frame MV and current MB uses field MV

Two MVs of direct mode MB are calculated from the forward MV of the collocated MB in the future reference frame using equation (2). Since the collocated MB is frame coded, MVi in (2) is derived by dividing the corresponding frame MV by 2 in vertical direction. Forward reference field of direct mode MB is the same parity field in the reference frame used by its collocated MB. Backward reference field is the same parity field of the future anchor frame.

Case 4: collocated MB uses field MV and current MB uses frame MV

The current MB is in frame coding while the collocated MB is in field coding. The two fields of the collocated MB may be coded in different modes. Since the 1st field of the future reference frame is temporally close to the current B frame, the field MV and the reference field of the collocated MB in the 1st future reference field provide certain basis for MVs and reference frame of the current direct mode MB. Two frame-based MVs (MVF,MVB) of direct mode MB are calculated as follows

MVF=TRB⋅MV1TRDMVB=(TRB−TRD)⋅MV1TRD                                                          (4)

where MV1 is determined by doubling the field MV of the collocated MB in the 1st field of the future reference frame in vertical direction, TRB is the temporal distance between the reference frame of one of whose fields pointed by the forward MV of the collocated MB in the 1st field of the future reference frame and the current B frame in field interval, and TRD is the temporal distance between the 1st field of the future reference frame and the reference field pointed by the forward MV of the collocated MB in the 1st future reference field. For example, in Fig. 4, TRD=3 field intervals and TRB=2 field intervals. Forward reference frame of the direct mode MB is the frame of one of whose fields is pointed by the forward field MV of the collocated MB in the 1st field of the future reference frame.

Fig. 4. B picture is in frame structure while its future reference picture is in field.

4. Computer Simulations

Simulations were carried out for nine interlace video sequences, as shown in Table 1. The first six in Table 1 are the common sequences for interlace testing [1], and the last one (Football_Tempete) is a concatenated sequence of Football and Tempete. Football is a high motion sequence while Tempete is of slow motion. Frame, field and adaptive coding were performed for each of those sequences separately. A set of four quantization parameters (QP) was tested per coding method per sequence, where QP for I and P are 16, 20, 24, and 28, and QP for B is equal to QP for I and P plus 2. Other coding parameters are listed in Table 2.

Table 1. Common Sequences for Interlace Testing

|  | Format (4:2:0) | Length | Frames/second |
| --- | --- | --- | --- |
| Canoa | 720x576 | 220 | 25 |
| F1car | 720x576 | 220 | 25 |
| Rugby | 720x576 | 220 | 25 |
| Bus | 720x480 | 150 | 30 |
| Football | 720x480 | 260 | 30 |
| Tempete | 720x480 | 260 | 30 |
| Coastguard | 720x480 | 260 | 30 |
| Mobile | 720x480 | 260 | 30 |
| Football_Tempete | 720x480 | 520 | 30 |

Table 2. Test conditions

| Entropy C. | MC | Hadamard | Ref. Frames | Search Res. | RD Opt. | Search R. |
| --- | --- | --- | --- | --- | --- | --- |
| UVLC | 1/4 pel | Yes | 2 | 2 | yes | 16 |

4.1 Performance Comparison for I and P

Fig. 5-13 show the PSNR with respect to bit rate for the eight test sequences for the picture structure of I and P only. The curves with diamond marks are for frame coding, the curves with square marks for field coding, and the curves with triangle marks for picture level adaptive coding.

Three of nine test sequences (Tempete, Mobile and Coastguard) demonstrate better results with frame-based coding and five sequences (Bus, Rugby, Football, F1car, and Canoa) with field-based coding. The difference in PSNR can be up to more than 2.5 dB (Rugby). This suggests that either frame or field coding is not a good solution for interlace video material because frame coding performs better for some sequences while field coding better for others. Picture level adaptive coding (curve with triangle marks) is seen to be close to, or slightly better than, frame coding for Tempete, Mobile and Coastguard and close to, slightly better than, field coding for Bus, Rugby, Football, F1car, and Canoa. One the other hand, the first half of the concatenated sequence Football_Tempete favors field coding and the second half favors frame coding.  Adaptive coding gives a much better performance than both frame and field coding. Table 3 summarizes the simulation results for the nine test sequences.

Table 3. The result summary for I and P

| Tempete | Adaptive = frame > field (by up to 2.0 dB) |
| --- | --- |
| Mobile | Adaptive = frame > field (by up to 0.7 dB) |
| Coastguard | Adaptive = frame > field (by up to 0.5 dB) |
| Football | Adaptive = field > frame (by up to 2.2 dB) |
| Bus | Adaptive = field > frame (by up to 2.0 dB) |
| Rugby | Adaptive = field > frame (by up to 2.5 dB) |
| F1car | Adaptive = field > frame (by up to 2.1 dB) |
| Canoa | Adaptive = field > frame (by up to 1.5 dB) |
| Football_Tempete | Adaptive > field (by 1.0 dB) & > frame (by up to 1.2 dB) |

Fig. 14 shows % of field-coded frames for picture level adaptive coding.  Picture level adaptive coding selects frame or field coding per frame based upon the content. As seen, the majority of frames of Tempete, Mobile and Coastguard are frame-coded. The performance of adaptive coding for Tempete, Mobile and Coastguard is therefore close to frame coding.  On the other hand, for Bus, Rugby, Football, F1car, and Canoa, adaptive coding codes most of their frames as fields. Hence, adaptive coding performs close to field coding for Bus, Rugby, Football, F1car, and Canoa. For the concatenated sequence Football_Tempete, about 50% of frames are frame coded and another 50% field coding. The performance of adaptive coding is seen to be much better than both frame and field coding. It is again shown that picture level adaptive coding is a better solution for interlace video materials than frame and field coding for I and P pictures.

3.3 Performance Comparison for I, P and B

Fig. 15-23 show the PSNR with respect to bit rate for the eight test sequences with picture structure of I, P and B. The curves with diamond marks are for frame coding, the curves with square marks for field coding, and the curves with triangle marks for picture level adaptive coding.

Similar to the tests with I and P only, frame coding performs better for sequences Tempete, Mobile and Coastguard while field coding for sequences Bus, Rugby, Football, F1car, and Canoa. The difference in PSNR can be up to more than 2.5 dB (Rugby). This again suggests that either frame or field coding is not a good solution for interlace video material. The picture level adaptation of frame/field coding (curve with triangle marks) performs close to, or slightly better than, frame coding for Tempete, Mobile and Coastguard  and closed to, or slightly better than, field coding for Bus, Rugby, Football, F1car, and Canoa.  For the concatenated sequence Football_Tempete, adaptive coding performs much better than both frame and field coding. It is further approved that it is necessary to adapt frame and field coding based upon the content. The results are summarized in table 4.

Table 4. The result summary for I, P and B

| Tempete | Adaptive = frame > field (by up to 2.0 dB) |
| --- | --- |
| Mobile | Adaptive = frame > field (by up to 1.0 dB) |
| Coastguard | Adaptive = frame > field (by up to 0.8 dB) |
| Football | Adaptive = field > frame (by up to 2.2 dB) |
| Bus | Adaptive = field > frame (by up to 2.0 dB) |
| Rugby | Adaptive = field > frame (by up to 2.5 dB) |
| F1car | Adaptive = field > frame (by up to 1.8 dB) |
| Canoa | Adaptive = field > frame (by up to 1.5 dB) |
| Football_Tempete | Adaptive > field (by 1.0 db) > frame (by 1.2 dB) |

Fig. 24 shows % of field-coded frames for picture level adaptive coding. Most frames of Tempete, Mobile and Coastguard are frame-coded. Hence, the performance of picture level adaptive coding for Tempete, Mobile and Coastguard is close to the frame-based coding. For Bus, Rugby, Football, F1car, and Canoa, the majority of frames are field-coded. Hence, picture level adaptive coding for theses sequences therefore performs close to the field-based coding. For sequence Football_Temepte, adaptive coding is able to select field coding for most frames in the first half of the sequence and frame coding in the second half. It performance is therefore better than both frame and field coding. It is shown that picture level adaptive coding is a much better solution for interlace video materials than frame and field coding for picture structures with I, P and B pictures.

4. Conclusions

The interlace coding tools presented in VCEG-O37 are integrated in the current TML9.0. Computer simulations were carried out for various video sequences for interlace testing. Test conditions and picture structures of simulations follow the same as defined in core experiments A, B and E [1]. The simulation results demonstrated that

- Frame coding performs better than field coding for Tempete, Mobile and Coastguard. The difference in PSNR can be up to 2.0 dB.

- Field coding performs better than frame coding for Bus, Rugby, Football, F1car, and Canoa. The difference in PSNR can be up to 2.5 dB.

- Picture level adaptive coding is able to select frame or field coding per frame based upon the content. Hence, it guarantees a performance over frame and field coding. It performs close to, or slightly better than, frame coding for Tempete, Mobile and Coastguard and close to, or slightly better than, field coding for Bus, Rugby, Football, F1car, and Canoa.

- Sequence Football_Tempete is a concatenated sequence of Football and Tempete. The first half of the sequence (Football) of high motions favors field coding, and the second half (Tempete) of slow motions and favors frame coding. Adaptive coding is able to code most frames in the first half of the sequence as fields, and in the second half as frames. Its performance is much better than both frame (1.0 dB) and field (1.2 dB) coding.

The advantage of adaptive coding over frame or field coding is obvious. It is recommended that adaptive coding be adopted in JVT.

References:

- P. Borgwrdt, “Core experiment on interlace video coding”, ITU-T VCEG-O59, 01/02/02.

- ITU-T, Video Coding Expert Group (VCEG), “H.26L test model long term number 9 (TML-9) draft 0”, 12/21/2001.

- A. Luthra, R. Gandhi, K. McKoen, Y. Yu, K. Panusopone, D. Baylon, L. Wang, “Performance of MPEG-4 Profiles Used for Streaming Video and Comparison with H.26L,” ISO/IEC JTC1/SC29/WG11 M7227, Sydney, July 2001.

- P. Borgwardt, “Handling Interlaced Video in H.26L,” ITU-T VCEG-N57, Santa Barbara, Sept 2001.

- C. Fogg, “Adaptive Field/Frame Block Coding Experiment Proposal,” ITU-T VCEG-N76

- M. Gallant, G. Cote, “High Rate, High Resolution Video using H.26L,” ITU-T VCEG-N84, Santa Barbara, Sept 2001.

- P. Borgwardt (Activity Coordinator), “Core Experiments on Interlaced Video Coding,” ITU-T VCEG-N85, Santa Barabara, Sept 2001.

- L. Wang, K. Panusopone, R. Gnadhi, Y. Yue, and A. Luthra, “Interlace coding tools for H.26L video coding”, VCEG-O37, Pattaya, Dec. 2001.

- Michael Gallant, Lowell Winger, Guy Côté, “Interlaced Field Coding Core Experiment”, VCEG-O40, Pattaya, Dec. 2001.

Fig. 5. PSNR vs bit rate curve for sequence Tempete.

Fig. 6. PSNR vs bit rate curve for sequence Mobile.

Fig. 7. PSNR vs bit rate curve for sequence Coastguard.

Fig. 8. PSNR vs bit rate curve for sequence Football.

Fig. 9. PSNR vs bit rate curve for sequence Bus.

Fig. 10. PSNR vs bit rate curve for sequence Rugby.

Fig. 11. PSNR vs bit rate curve for sequence F1car.

Fig. 12. PSNR vs bit rate curve for sequence Canoa.

Fig. 13. PSNR vs bit rate curve for sequence F & T.

Fig. 14. % of field coded picture as a function of QP values.

Fig. 15. PSNR vs bit rate curve for sequence Tempete.

Fig. 16. PSNR vs bit rate curve for sequence Mobile.

Fig. 17. PSNR vs bit rate curve for sequence Coastguard.

Fig. 18. PSNR vs bit rate curve for sequence Football.

Fig. 19. PSNR vs bit rate curve for sequence Bus.

Fig. 20. PSNR vs bit rate curve for sequence Rugby.

Fig. 21. PSNR vs bit rate curve for sequence F1car.

Fig. 22. PSNR vs bit rate curve for sequence Canoa.

Fig. 23. PSNR vs bit rate curve for sequence Football_Tempete.

Fig. 24. % of field coded picture as a function of QP values.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola Inc. |  |  |
| Mailing address | # 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA., USA, 11/27/2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive frame/field coding for JVT video coding |  |
| Contribution number |  | JVT-B071 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| Without having conducted a search or investigation, the submitter/author(s) individually believe(s) they or General Instrument Corporation (Patent Holder) may have granted, pending or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: