| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B077 Filename: JVT-B077.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | # Short tap filter for high resolution sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei-ichi Chono and Yoshihiro Miyamoto NEC Multimedia Laboratories, 4-1-1 Miyazaki Miyamae Kawasaki, 216-8555 JAPAN | Tel: Fax: Email: | +81 44 856 8141 +81 44 856 2232 chono@dsp.cl.nec.co.jp miyamoto@dsp.cl.nec.co.jp |
| Source: | NEC Corporation Kawasaki, 216-8555 JAPAN |  |  |

_____________________________

# 1. Background

In VCEG-O16 of the Pattaya meeting adaptive interpolation scheme, estimating and transmitting 6-tap filter coefficients per picture, was proposed [1] To consider non-stationary statistical properties of video signals, this adaptive interpolation scheme seems to be beneficial [1][2]. However, there are some issues: complexity and so on. AHG was formed to study these issues for the in-corporation of this feature into TML.

We agree to the adaptive interpolation scheme, and have compared coding quality gains of some interpolation filters, for a given set selection of filter coefficients. Experiment results show us two facts. First fact is that TML9 6-tap filter coding gain to Telenor 4-tap filter depends on sequences. Second fact is that 4-tap filter implementation reduces the amount of fractional-pel interpolation calculation to 50% at decoder. We propose that encoder/decoder may use shorter tap filter for high-resolution sequences, in order to reduce the complexity.

# 2. Experiments

# 2.1 Interpolation

In order to produce 2:1 resolution, Telenor 4-tap filter, with impulse response (-4, 20, 20, -4)/32 [3], is used. For 4:1 resolution, Bilinear filter is used as second filter. Hence, the difference between 4-tap filter interpolation and TML9 is first filter coefficients.

# 2.2 Test sequences and coding conditions

Test sequences are shown in Table 1.

Table 1: Test sequence

The coding conditions are: Quantization parameter for I-picture(QI) set to 24, 16 pixel search range(32 pixel is only used for Football CCIR), RestrictSearchRange set to two, UseHadamard set to on, quarter-pel motion, one reference frame, UVLC in entropy coding, RD optimization set to on, no B-picture.

CCIR sequences are interlace-formant, not supported by TML9 yet. The encoder treated 2 fields as 1 frame described Fig 1.

Fig 1	Input YUV format at CCIR encoding

# 2.3 Results

TML9 filter coding gain compared to Telenor 4-tap filters is the following:

QCIF results:

TML9 filter leads to average gain 0.19[dB] with avsnr4, up to 0.48[dB

CIF and SIF results:

TML9 filter leads to average gain 0.17[dB] with avsnr4, up to 0.60[dB]

CCIR results:

TML9 filter leads to average gain 0.24[dB] with avsnr4, up to 0.54[dB]

Avsnr4 is used to calculate objective differences for common measurement method [4]. The avsnr4-results are shown in Table 1-3, and rate-distortion curves are attached.

# 3. Proposal : adaptive filter taps for reducing coding calculation

Experiment results show that TML9’s coding gain to Telenor 4-tap filter depends on sequences. The total amount of fractional-pel calculation increases in proportion to the spatial resolution. The current TML9 uses direct-interpolation for reducing multiplications and memory requirement. Telenor 4-tap direct-interpolation can reduce multiplication to half of 6-tap’s [5]. We have evaluated the decoding time improvement by the multiplication reduction. Evaluation condition is: Pentium4 1.7GHz, 512MB memory, Microsoft Windows2000, and Intel VTune5 with 1msec sampling rate. The multiplication reduction cuts half of interpolation calculation, and 10 percent of total decoding calculation (See Chart 1-3).

From view of complexity reduction, we think that encoder/decoder may use shorter tap filter for high-resolution sequences, because TML9’s coding gain depends on the sequence.

# 4. Conclusion

In this report, two facts are shown:

- TML 6-tap filter coding gain to Telenor 4-tap filter depends on sequences

- 4-tap direct interpolation cuts half of interpolation calculation, and 10 percent of total decoding calculation

We propose that encoder/decoder may use shorter tap filter for high-resolution sequences, in order to reduce the complexity. The additional study on higher resolution sequences, as HDTV, may be valuable for confirming this interpolation filter issue.

# Reference

- Thomas Wedi, “H.26L Interpolation Filtering Ad Hoc Report”, ITU-T SG16, doc. VCEG-O11, Pattaya, Thailand, 4-6 Dec., 2001

- Thomas Wedi, “Adaptive Interpolation Filter for H.26L”, ITU-T SG16, doc. VCEG-N28, Santa Barbara, CA, USA, Sep. 2001

- Thomas Wedi, “Interpolation Filter for motion compensated prediction with 1/4 and 1/8-pel accuracy”, ITU-T SG16, doc.Q15-J-14, Osaka, Japan, May. 2000.

- Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Conditions for H.26L Coding efficiency Experiment on Low-Resolution Progressive-Scan Source Material”, ITU-T SG16, doc. VCEG-N81, Santa Barbara, CA, USA, Sep. 2001

- Thomas Wedi,” Results on complexity and coding performance investigations: Displacement vector resolution and interpolation filter tap size”, ITU-T SG16, doc. VCEG-M46, Austin, Texas, USA, April, 2001.

# Avsnr4-results

Table1: QCIF results

Table2: CIF and SIF results

Table3: CCIR results

Decoding samples

Chart 1	Decoding samples (Sequence: Foreman with QCIF, 10fps, QI=24, QP=20, and 133 frames)

Chart 2	Decoding samples (Sequence: Paris with CIF, 15fps, QI=24, QP=20, and 150 frames)

Chart 3	Decoding samples (Sequence: Flower with CCIR, 30fps, QI=24, QP=20, and 150 frames)

# Rate-distortion curves