| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B084 Filename: JVT-B084.doc Generated: 2002-01-23 |
| --- | --- |

Title:	Downloadable Threshold Tables for Loop-filter

Status:		Input Document to JVT

Purpose:	Proposal

Author(s) or	Teck Wee Foo, Sheng Mei Shen 	Tel: 65-550-5466

Contact(s): 	Block 1022 Tai Seng Avenue		Email: twfoo@psl.com.sg

#04-3530 Singapore 534415

Shinya Kadono (Matsushita Electric	Tel: +81-6-6900-9689

Industrial Co., Ltd.)			Email: kadono@drl.mei.co.jp

1006 Kadoma, Kadomo, Osaka,

Japan

Source:           Panasonic Singapore Laboratories

1.	Introduction

The loop-filter proposed in [1] has threshold tables to determine the amount of filtering to apply to the picture. The values in the threshold tables are selected based on the best result for a group of different sequences. This mean that for certain picture sequences, the filter will not produce the best results (subjectively) because the threshold tables have been set based on compromise made between the group of sequences. The same argument would also apply to the current loop-filter in the JVT.

To ensure the best results for each picture sequences, the threshold tables would need to be tweaked to take into account the type of sequences to be encoded.

The document in [1] proposed a medium complexity loop-filter for use in the JVT codec [2]. The original TML loop-filter required 50% for intra coded frames and 31% for inter coded frames (see Table 1).

| Picture type | TML8.5 | PSL | Reduction |
| --- | --- | --- | --- |
| intra coded | 50.4 | 32.9 | 17.5 |
| inter coded | 31.0 | 19.9 | 11.1 |

Table 1: Complexity comparison for TML loop-filter and PSL loop-filter

High complexity modules mean the modules would require more clock-cycle to perform the operations. The high clock-cycles would demand high power consumption. For low-powered devices like mobile phones and PDA, low power consumption is very much valued to improve battery life and reduce chip size. In designing the loop-filter in [1], we have made a trade-off in the complexity of the loop-filter and the coding efficiency of the JVT codec.

- Proposal

We would like to propose to add a flag, download_lf_table, in the bitstream to indicate whether to use the standard threshold tables or to use threshold tables optimized for the particular sequence. The flag could be put in the GOP level or picture/slice level.

| Syntax | Bit |
| --- | --- |
| if (download_lf_table == 1) | 1 |
| download table from bitstream |  |
| else |  |
| use standard threshold table |  |

For the coding of the threshold table for different sequences, we propose the following:

The new method will code the changes between the new threshold values and the standard threshold values from the standard table (see Table 2). For example,

| QP | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 |
| --- | --- |
|  | 0 0 0 0 0 0 0 3 4 4 5 6 7 8 9 13 13 15 17 17 20 22 25 27 29 30 33 34 36 45 50 50 |
|  | 0 0 0 0 0 0 0 0 1 1 2 3 4 5 6 9 10 11 13 14 16 18 20 22 24 24 29 29 34 36 35 35 |
|  | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 2 2 2 |

Table 2: Threshold tables from [1]

The coding of the changes will only be done for QPs 16 to 31. For QPs 0 to 15, the values in the threshold table do not need to be changed as observed through experiments.

For each QP, 4 bits will be allocated. The MSB will be the signed bit, with 0=positive and 1-negative, while the 3 LSBs will be the magnitude. The maximum deviation from the standard values will be ±7.

For QP 22, a value of 1001 will indicate delta= –1.

For QP 30, a value of 0010 will indicate delta= +2;

The final threshold value for  will be

For QP 22, the new value will be 25+delta = 25-1 = 24.

For QP 30, the new value will be 50+delta = 50+2= 52.

Same applies for  and  tables.

To encode the changes for  and :,

Number of bits required: 4 x 16 x 2 = 128 bits

To encode the changes for :

Number of bits required: 2 x 16 = 32

For encoding the changes for , 2 bits are enough because the maximum value is 2.

Total bits required = 128 + 32 = 160 bits (20 bytes).

For the current TML loop-filter, only 128 bits are required to code the changes in the ALPHA and BETA tables.

3.	Results

The values for the new threshold tables have been produced with the best subjective quality achievable. The idea is to show that downloadable threshold tables could improve the subjective quality of the sequences. Nonetheless, further improvement on the subjective quality could be done.

Threshold tables with the best subjective quality for silent (QCIF):

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 15 | 17 | 17 | 22 | 24 | 27 | 30 | 34 | 35 | 36 | 37 | 40 | 45 | 50 | 50 |
|  | 10 | 11 | 13 | 14 | 15 | 18 | 20 | 22 | 27 | 28 | 29 | 29 | 30 | 35 | 35 | 35 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 2 | 2 | 2 |

Difference:

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 3 | 5 | 5 | 3 | 3 | 4 | 0 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 3 | 4 | 0 | 0 | -4 | -1 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Threshold tables with the best subjective quality for foreman (QCIF):

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 | 15 | 17 | 19 | 23 | 25 | 27 | 29 | 32 | 34 | 35 | 36 | 37 | 45 | 50 | 50 |
|  | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 25 | 27 | 28 | 29 | 29 | 29 | 35 | 35 | 35 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 2 | 2 | 2 |

Difference:

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 0 | 0 | 2 | 3 | 3 | 2 | 2 | 3 | 4 | 2 | 2 | 1 | 0 | 0 | 0 |
|  | 0 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 4 | 0 | 0 | -5 | -1 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Threshold tables with the best subjective quality for container (QCIF):

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 15 | 17 | 19 | 23 | 25 | 27 | 29 | 30 | 33 | 34 | 37 | 40 | 45 | 50 | 50 |
|  | 10 | 12 | 14 | 16 | 18 | 20 | 21 | 22 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |

Difference:

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -1 | 0 | 0 | 0 | -2 | -3 | -1 | -1 | 0 | 0 | -3 | 0 | -1 | -1 | -1 | -1 |
|  | -2 | -1 | -1 | 0 | -1 | -1 | -1 | 0 | 0 | 1 | -2 | 0 | -2 | -1 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

The subjective quality will be shown during the meeting. The sequences have been encoded based on the recommendations in [3].

The subjective viewing will show that sequences encoded with the proposed method look better than the sequences encoded with current loop-filter in TML9.0.

Table 3: Difference of Bitrate and PSNR wrt TML 9.0.

| Sequence Name | Difference of Bitrate (%) | Difference of PSNR(dB) |
| --- | --- | --- |
| Container_QCIF | 8.24 | -0.39 |
| Foreman_QCIF | 5.21 | -0.27 |
| Silent_QCIF | 2.09 | -0.07 |
| Mobile_QCIF | 4.30 | -0.19 |
| Paris_QCIF | 4.50 | -0.22 |
| Average | 4.87 | -0.23 |

4.	Summary

The new method is able to produce pictures that are less distorted when viewed subjectively. The new method is easy to implement and it allows for further picture quality improvement in the future. Therefore, we propose that JVT adopt the above syntax into the JVT codec.

5.	References

[1]	TW Foo, SM Shen, CJ Lee and S. Kadono, “Medium Complexity Loop-filter”, document VCEG-O50r1, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, Thailand, December 2001.

[2]	T.Wiegand, “H.26L Test Model Long-Term Number 9 (TML-9) draft 0”, Dec. 2001.

[3]	G.Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, VCEG-N81, Sep. 2001.