| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B084 Filename: JVT-B084.doc Generated: 2002-01-23 |
| --- | --- |

Title:	Downloadable Threshold Tables for Loop-filter

Status:		Input Document to JVT

Purpose:	Proposal

Author(s) or	Teck Wee Foo, Sheng Mei Shen 	Tel: 65-550-5474

Contact(s): 	Block 1022 Tai Seng Avenue		Email: twfoo@psl.com.sg

#04-3530 Singapore 534415

Shinya Kadono (Matsushita Electric	Tel: +81-6-6900-9689

Industrial Co., Ltd.)			Email: kadono@drl.mei.co.jp

1006 Kadoma, Kadomo, Osaka,

Japan

Source:           Panasonic Singapore Laboratories

1.	Introduction

The loop-filter proposed in [1] has threshold tables to determine the amount of filtering to apply to the picture. The values in the threshold tables are selected based on the best result for a group of different sequences. This mean that for certain picture sequences, the filter will not produce the best results (subjectively) because the threshold tables have been set based on compromise made between the group of sequences. The same argument would also apply to the current loop-filter in the JVT.

To ensure the best results for each picture sequences, the threshold tables would need to be tweaked to take into account the type of sequences to be encoded.

The document in [1] proposed a medium complexity loop-filter for use in the JVT codec [2]. The original TML loop-filter required 50% for intra coded frames and 31% for inter coded frames (see Table 1).

| Picture type | TML8.5 | PSL | Reduction |
| --- | --- | --- | --- |
| intra coded | 50.4 | 32.9 | 17.5 |
| inter coded | 31.0 | 19.9 | 11.1 |

Table 1: Complexity comparison for TML loop-filter and PSL loop-filter

High complexity modules mean the modules would require more clock-cycle to perform the operations. The high clock-cycles would demand high power consumption. For low-powered devices like mobile phones and PDA, low power consumption is very much valued to improve battery life and reduce chip size. In designing the loop-filter in [1], we have made a trade-off in the complexity of the loop-filter and the coding efficiency of the JVT codec.

- Proposal

We would like to propose to add a flag, download_lf_table, in the bitstream to indicate whether to use the standard threshold tables or to use threshold tables optimized for the particular sequence. The flag could be put in the GOP level or picture/slice level.

| Syntax | Bit |
| --- | --- |
| if (download_lf_table == 1) | 1 |
| download table from bitstream |  |
| else |  |
| use standard threshold table |  |

For the coding of the threshold table for different sequences, we propose the following:

The new method will code the changes between the new threshold values and the standard threshold values from the standard table (see Table 2). For example,

| QP | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 |
| --- | --- |
|  | 0 0 0 0 0 0 0 3 4 4 5 6 7 8 9 13 13 15 17 17 20 22 25 27 29 30 33 34 36 45 50 50 |
|  | 0 0 0 0 0 0 0 0 1 1 2 3 4 5 6 9 10 11 13 14 16 18 20 22 24 24 29 29 34 36 35 35 |
|  | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 2 2 2 |

Table 2: Threshold tables from [1]

The coding of the changes will only be done for QPs 16 to 31. For QPs 0 to 15, the values in the threshold table do not need to be changed as observed through experiments.

For each QP, 4 bits will be allocated. The MSB will be the signed bit, with 0=positive and 1-negative, while the 3 LSBs will be the magnitude. The maximum deviation from the standard values will be ±7.

For QP 22, a value of 1001 will indicate delta= –1.

For QP 30, a value of 0010 will indicate delta= +2;

The final threshold value for  will be

For QP 22, the new value will be 25+delta = 25-1 = 24.

For QP 30, the new value will be 50+delta = 50+2= 52.

Same applies for  and  tables.

To encode the changes for  and :,

Number of bits required: 4 x 16 x 2 = 128 bits

To encode the changes for :

Number of bits required: 2 x 16 = 32

For encoding the changes for , 2 bits are enough because the maximum value is 2.

Total bits required = 128 + 32 = 160 bits (20 bytes).

For the current TML loop-filter, only 128 bits are required to code the changes in the ALPHA and BETA tables.

3.	Results

The values for the new threshold tables have been produced with the best subjective quality achievable. The idea is to show that downloadable threshold tables could improve the subjective quality of the sequences. Nonetheless, further improvement on the subjective quality could be done.

Threshold tables with the best subjective quality for silent (QCIF):

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 15 | 17 | 17 | 22 | 24 | 27 | 30 | 34 | 35 | 36 | 37 | 40 | 45 | 50 | 50 |
|  | 10 | 11 | 13 | 14 | 15 | 18 | 20 | 22 | 27 | 28 | 29 | 29 | 30 | 35 | 35 | 35 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 2 | 2 | 2 |

Difference:

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 3 | 5 | 5 | 3 | 3 | 4 | 0 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | -1 | 0 | 0 | 0 | 3 | 4 | 0 | 0 | -4 | -1 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Threshold tables with the best subjective quality for foreman (QCIF):

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 | 15 | 17 | 19 | 23 | 25 | 27 | 29 | 32 | 34 | 35 | 36 | 37 | 45 | 50 | 50 |
|  | 10 | 12 | 14 | 16 | 18 | 20 | 22 | 25 | 27 | 28 | 29 | 29 | 29 | 35 | 35 | 35 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 2 | 2 | 2 |

Difference:

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 0 | 0 | 2 | 3 | 3 | 2 | 2 | 3 | 4 | 2 | 2 | 1 | 0 | 0 | 0 |
|  | 0 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 4 | 0 | 0 | -5 | -1 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Threshold tables with the best subjective quality for container (QCIF):

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 15 | 17 | 19 | 23 | 25 | 27 | 29 | 30 | 33 | 34 | 37 | 40 | 45 | 50 | 50 |
|  | 10 | 12 | 14 | 16 | 18 | 20 | 21 | 22 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |

Difference:

| QP | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -1 | 0 | 0 | 0 | -2 | -3 | -1 | -1 | 0 | 0 | -3 | 0 | -1 | -1 | -1 | -1 |
|  | -2 | -1 | -1 | 0 | -1 | -1 | -1 | 0 | 0 | 1 | -2 | 0 | -2 | -1 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |

The tables above showed that only minor adjustments need to be done to the threshold value in the standard tables.

The subjective quality will be shown during the meeting. The sequences have been encoded based on the recommendations in [3].

The subjective viewing will show that sequences encoded with the proposed method has less block noise and distortion than the sequences encoded with current loop-filter in TML9.0.

Table 3: Difference of Bitrate and PSNR wrt TML 9.0.

| Sequence Name | Difference of Bitrate (%) | Difference of PSNR(dB) |
| --- | --- | --- |
| Container_QCIF | 8.24 | -0.39 |
| News_QCIF | 3.25 | -0.18 |
| Foreman_QCIF | 5.21 | -0.27 |
| Silent_QCIF | 2.09 | -0.07 |
| Mobile_CIF | 4.30 | -0.19 |
| Tempete_CIF | 5.76 | -0.22 |
| Paris_QCIF | 4.50 | -0.22 |

The rate distortion curves are shown in the Appendix.

4.	Summary

The new method is more effective in removing block noise when viewed subjectively. The new method is easy to implement and it allows for further picture quality improvement in the future. Therefore, we propose that JVT adopt the above syntax into the JVT codec.

5.	References

[1]	TW Foo, SM Shen, CJ Lee and S. Kadono, “Medium Complexity Loop-filter”, document VCEG-O50r1, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, Thailand, December 2001.

[2]	T.Wiegand, “H.26L Test Model Long-Term Number 9 (TML-9) draft 0”, Dec. 2001.

[3]	G.Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, VCEG-N81, Sep. 2001.

Appendix

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma-City, Osaka, 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Geneva, CH, Jan. 29 - Feb. 1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ## JVT |  |
| Title |  | ## Downloadable Threshold Tables for Loop-filter |  |
| Contribution number |  | ## JVT-B084 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: