| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B085 Filename: JVT-B085.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | New Syntax for Long-term Buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S M Shen, T W Foo, Indrajit Chakrabarty Panasonic Singapore Laboratories Pte. Ltd., Shinya Kadono (Matsushita Electric Industrial Co. Ltd.) | Tel: Email: | +65-5505466 shen@psl.com.sg |
| Source: | Panasonic Singapore Laboratories Pte Ltd |  |  |

# Introduction

The proposal is made based on the following statements:

- It is noticed that multiple reference frames prediction brings a lot of gain [4]. It is also worth noting that the number of multiple reference frames is needed to pre-fix for hardware implementation.

- In order to support long-distance multiple reference frames prediction for many applications, a long term buffer concept is introduced to assist for the Restricted Multiple Reference Frame Prediction in VCEG-049 [3], and the similar concept is also addressed in the H.263++ Annex U which has been incorporated in the current TML 9.0/JM 1.0.

- In the current TML, to support a long term buffer with multiple reference frames prediction, a sophisticated buffer management scheme is used since the same one buffer will be used for conventional multiple reference frames prediction and long term buffer prediction.

- The buffer management would be clean and simple if two different types of buffer (multiple reference buffer and long term buffer) are employed separately to control and manage different kinds of prediction.

- Since the long term buffer scheme is used as assistance on top of the existing multiple reference frames prediction when there is scene change in a sequence, its operation should be different from the existing multiple reference frames prediction.

By following this direction, a supporting syntax is proposed to add in slice level and MB level to incorporate this long term buffer in the current TML which is presented in the 1st section, the comparison between TML and the proposed approach is elaborated in the 2nd section, and the simulation results based on the proposed approach are shown in the following section.

1. Supporting Syntax added to Slice and MB level

In this proposal, we present a method of using a long-term frame buffer for motion compensated prediction with slices of the long-term frame stored from more than one frames in a video sequence. In addition, with this method, it is possible to predict the motion vectors of a macroblock in a slice from different long-term frames stored in the long-term frame buffer or use INTRA calculations if there is any error present in the macroblock. Thus, we can have a more robust way of motion-compensated prediction of motion vectors from a long-term frame buffer. The method proposed is illustrated in the following diagram.

Time

Long-term frame buffer

Figure 1: This figure shows how a long-term frame is used for prediction. A green arrowhead indicates that macroblocks are being stored in the long-term frame buffer while a red arrowhead indicates that the macroblocks from the long-term frame buffer are being used for the prediction

In the Figure 1, the use of single long-term frame is illustrated. The arrowheads marked green indicate that macroblocks in a slice in the long-term frame buffer is being stored from the corresponding slice of a frame in the video sequence; while the arrowheads marked red indicate that macroblocks in a slice from the long-term frame are being used for the prediction of the corresponding macroblocks present in the slice of a frame in the sequence.

In the followings, we discuss the syntax to be added to both slice level and MB level which is required to implement the above method in the codec

The following syntax has to be introduced in the slice header for long-term frame buffer.

- store_long_term_buffer – (1 bit) – This flag is used to indicate if the slices in the current picture are to be stored (= 1) or not stored (=  0) in the long-term frame buffer.

- predict_long_term_buffer – (1 bit) – This flag is used to indicate if the slices in the current picture are to be predicted from (=1) or not predicted (=0) from the long-term frame buffer.

- long_term_buffer (2/3 bits) – This is an integer indicating the index of the long-term frame buffer. If we use two bits, then up to four long-term frame buffers can be used for motion prediction (00 for 1st buffer, 01 for 2nd buffer, etc.) and if three bits are used, up to 8 long-term frame buffers can be used (000 for 1st buffer, 001 for 2nd buffer, etc.). This index is present in the bitstream only when either of the above two flags is 1.

Subsequently, in the macroblock level, we will have an additional flag to indicate whether the current macroblock in the slice is to be stored into or predicted from the long-term frame buffer.

use_long_term_buffer (1 bit) – This flag is used to indicate if the current macroblock will be stored (=1)/not stored (=0) or predicted (=1)/not predicted (=0) in / from the long-term frame buffer. This flag is present in the bitstream only when either store_long_term_buffer or predict_long_term_buffer is 1.

As shown in Figure 2,  long_term_buffer for the buffer index in slice header and use_long_term_buffer in MB level is depending on either of store_long_term_buffer and predict_long_term_buffer turning to 1. Otherwise they will not be transmitted in the stream.

Slice Header

store_long_

term_buffer

(1 bit)

predict_long_term_buffer

(1 bit)

long_term

_buffer

(3 bits)

MB Header

use_long_

term_buffer

(1 bit)

### Figure 2 Flags added to Slice and MB Header

The  supporting syntax is shown as follows:

Slice Level;

store_long_term_buffer ;							1 bit

predict_long_term_buffer;							1 bit

If (store_long_term_buffer  == 1 || predict_long_term_buffer == 1)

long_term_buffer;    						3 bits

MB Level;

If (store_long_term_buffer  == 1 || predict_long_term_buffer == 1)

use_long_term_buffer;      					1 bit

Note:There is no any change for PType, MB_Type, and Ref_frame that are used in current TML.

2 Comparison with TML

In order to compare the current proposal with the current TML 9.0/JM1.0 and the implementation in TML software, we use the following table where we use a fixed bit length for comparison in both cases.

Table 1 Comparison between the Features and Bit Usage of LT Buffer Scheme in TML and PSL

| Bits Usage/Features | TML 9.0/JM 1.0 | PSL Proposal |
| --- | --- | --- |
| Bits in Slice Layer | *7 bits + No. of bits : 1 bit for RPSL 2 bits for RMPNI No. of bits for ADPN, LPIR 1 bit for RPBT 3 bits for MMCO No. of bits for DPN, LPN, MLIP1 | 2 bits: 1 bit for store 1 bit for predict + 3 bits (for buffer index) |
| Bits in Macroblock Layer | 3 (for MB_Type) | 1 or 0 |
| Number of buffer types used | 1 mixed one used for both multiple reference buffer and Long term buffer | 2 separate two buffers used for multiple reference buffer and Long term buffer |
| Support B picture prediction from B picture in multiple frames buffer and long term buffer | Need to change; See VCEG-B057 | Yes |
| Buffer Management | Sophisticated; See TML 9.0/JM 1.0 | 1st buffer (multiple reference) store and predict by following picture order and FIFO 2nd buffer (long Term) store and predict by indicating Buffer Index |

Note: *7 bits + No. of bits  will become: 7 + 3x5 = 22 bits if 3 bits allocated to each parameter for ADPN, LPIR, DPN, LPN, and MLIP1.

Note that in view of the syntax described in the section 1, the three bits in the slice layer used for long_term_buffer and 1 bit in the macroblock layer used for use_long_term_buffer are utilized only when one of the two flags present in the slice layer is 1. Thus, we have two scenarios, one, where we do not use a long-term frame buffer and another where we use a long-term frame buffer. We compare the bit consumption in both the scenarios below where QCIF size is used as an example.

Table 2 Example of Bits Usage of the Cases of Using/Not Using Long Term Buffer

| Scenario | TML/PSL | Bits used | Total bits used Per Frame |
| --- | --- | --- | --- |
| Long-term frame buffer not used for motion prediction | TML | 9 (per slice) * 9 (slices per picture) | 81 |
|  | PSL | 2 (per slice) * 9 (slices per picture) | 18 |
| Long-term frame buffer used for motion prediction | TML | 22 (per slice) * 9 (slices per picture) + 3 (MB_Type, per MB) * 99 (MBs per picture) | 495 |
|  | PSL | 1 (per MB) * 99 (MBs per picture) + 2 (per slice, for flags) * 9 (slices per picture) + 3 (per slice, for index) * 9 (slices per picture) | 144 |

We observe that when a long-term frame buffer is not used for motion-compensated prediction, there is a saving of 63 bits per picture, and in the other scenario, there is a saving of 351 bits per picture. Thus, the current proposal leads to a significant savings in the bits used by the codec. In addition, we notice that buffer management in the proposed method is simple and clean as an integer can be used for indicating which long-term frame to use from the long-term frame buffer for storage or prediction. Moreover, there is an added flexibility of not using the long-term frame buffer for storage of or prediction from the current slice in the picture and the codec can use the TML’s reference frame buffer.

3 Simulation Results

In order to illustrate the advantages of using a long-term frame buffer, we constructed a composite vide sequence with 15-frame clips from Akiyo, Foreman & Container sequences in the QCIF format. The positioning of these clips in the sequence is illustrated in the Appendix A. Test conditions laid down in VCEG-N81 [2] were used in the simulations experiments. Results were obtained for 4 different values of the quantization parameter, viz., 16, 20, 24 & 28.

The use of a sequence as described above is suggested by the fact the current TML 9.0 and the TML 9.0 software uses up to 5 previously decoded frames in the sequence for motion-compensated prediction of the current frame. When there is a scene-change, such as, for picture number 30 in the sequence, the codec switches itself to the INTRA mode and the consumption of bits for encoding this frame increases accordingly, leading to a decrease in the efficiency and performance of the codec. This is because; all the previously decoded frames in the sequence, viz., frames, 25, 26, 27, 28, & 29 belong to the Foreman sequence. However, if a long-term frame buffer is used for motion-compensated prediction, the codec uses the INTER mode and there is an enormous saving of the bits used by the codec.

Simulations were carried out using the TML 9.0 software under the guidelines laid down in VCEG-N81 with some modifications to implement the syntax discussed in section the above sections. The results for simulations are shown in results.xls. Subjective comparison of a frame where a scene-change occurs is illustrated in the Figure 3. A significant improvement in the quality of the picture is observed when a long-term frame buffer is used in the codec.

Figure 2: Screen shots from the test sequence used to carry out the experiments for illustrating the advantages of the long-term frame buffer. On the left is frame 30 of the sequence encoded with TML 9.0 software and on the right is the same frame encoded with the modified TML software with a long-term frame buffer based on the proposed scheme. The quantization parameter is 24 for both the frames.

We have plotted the PSNR of the decoded sequence with the frames for different quantization parameters. It is observed that there is a significant gain in the PSNR of the clips of Akiyo where the long-term frame buffer was used for motion-compensated prediction of the scene-change frames. The PSNR curve for Y with quantization parameter of 24 is shown in the figure below.

Figure 4: The figure shows the variation of the Luma PSNR with the frames in the sequence used for the simulations. Gains in the PSNR are prominent in the clips where the long-term frame buffer was used

We observe from the rate-distortion curve for Y, U & V of the sequence that there is an improvement in the quality of the pictures in the sequence. We also note that the bits consumed reduce when we use the long-term frame buffer when compared with TML 9.0.

Conclusions

In light of the results obtained and the syntax discussed above, we suggest the incorporation of a long-term frame buffer with the simple syntax for prediction in the JM. The advantage of the use of long-term frame buffer with the syntax has been illustrated with one type of video sequence where gains in quality are observed at frames where a scene change occurs. In addition, we also note that whenever there is a loss of a whole slice or even a macroblock, the codec can still maintain the picture quality as we can skip the motion-prediction from the long-term buffer and use the current error concealment techniques for the macroblock. Thus, we have a method of motion-compensated prediction with built-in error resilience for the long-term frame buffer. There will definitely be various other applications where this method will produce significant improvement in the quality of the video and also increase the efficiency of the codec.

References:

[1]  ITU-T VCEG-NYY, H.26L Test Model Long-Term Number 9 (TML-9) draft 0, VCEG    (SG16/Q6), Fifteenth Meeting, Pattaya, Thailand, December 2001.

[2]  ITU-T VCEG-N81, Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material, VCEG (SG16/Q6), Fourteenth Meeting, Santa Barbara, USA, September 2001.

[3]  ITU-T VCEG-O49, Restricted Multiple Reference Frames with Long Term Frame Buffer, VCEG (SG16/Q6), Fifteenth Meeting, Pattaya, Thailand, December 2001

[4]  ITU-T Q15-J-11, Demonstration of H.263++ Annex U Performance, (SG16/Q6), Osaka, Japan, May 2000

Appendix A

## Frame 0               Frame 14            Frame 15              Frame 29            Frame 30             Frame 44             Frame 45             Frame 59            Frame 60

Figure 5:  The video sequence used for simulations with the TML 9.0 software with the long-term frame buffer. Illustrative screen shots from the first 60 frames of the sequence are shown above.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma-City, Osaka, 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Geneva, CH, Jan. 29 - Feb. 1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ## JVT |  |
| Title |  | ## New Syntax for Long-term Buffer |  |
| Contribution number |  | ## JVT-B085 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: