| ## Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B093 Filename: JVT-B093.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Experiment Results of the Improved 2D-VLC Proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Authors: | Sadaatsu Kato Satoru Adachi Minoru Etoh | Tel: Fax: Email: | +81-468-40-3515 +81-468-40-3788 kato@spg.yrp.nttdocmo.co.jp |
| Source: | NTT DoCoMo, Inc. |  |  |

____________________________________________________________________________________

# 1. Introduction

At the Pattaya meeting, a new 2D-VLC encoding scheme for high-bitrate (VCEG-O27) was proposed and its possibility to improve coding efficiency at high-bitrate was demonstrated. JVT expressed their interest in the proposal and encouraged further work. We also take an interest in the results and evaluated the coding efficiency of the proposed techniques.

VCEG-O27 proposed the following three methods to improve coding efficiency of 2D-VLC.

- Level scaling 2D-VLC table design

- position adaptive 2D-VLC table design

- inverse order zigzag scanning

We adopted the proposed methods to the H.26L TML9.0 software, and compared the coding efficiency with the original H.26L TML9.0. The results show that the proposed methods improve coding efficiency and further consideration will be expected to utilize the methods effectively.

# 2. Simulation Conditions

We defined the following five methods/combinations to evaluate the coding efficiency of each proposed method, and evaluate them separately.

1:  Original TML9.0 (Original)

2:  Original TML9.0 + Level scaling 2D-VLC table design (Original + Level)

3:  Original TML9.0 + position adaptive 2D-VLC table design (Original + Position)

4:  Original TML9.0 + position adaptive 2D-VLC table design + inverse order zigzag scanning

(Original + Position + Inverse)

5:  Original TML9.0 + Level scaling 2D-VLC table design + position adaptive 2D-VLC table design

+ inverse order zigzag scanning (Original + Level + Position + Inverse)

Simulation conditions are as follows. They are the same as VCEG simulation common condition (VCEG-N81) except for small QP values, which are needed to be small to evaluate the proposed methods.

- Software version: H.26L TML9.0.

- Motion estimation and mode selection: High complexity mode.

- QP: 2, 6, 10, 16, and fixed for all over sequence.

- Sequences: Foreman, News, Container, Silent Voice (QCIF), Paris, Tempete, Mobile (CIF).

- Frame rate: 10Hz (Foreman, News, Container), 15Hz (Silent Voice, Paris), 30Hz (Tempete, Mobile)

- Picture Coding Type: IIIIIIII (I-picture simulation) and IPPPPPPP (P-picture simulation)

- Number of reference frames: 5 frames

- The motion vector search range: 32pels

- Other coding options: use Hadamard, 1/4 pel accuracy MC for QCIF, 1/8 pel accuracy for CIF, no CABAC (Context-based Adaptive Binary Arithmetic Coding), no slice structure.

# 3. Simulation Results

From Figure 1 to 14 show R-D curves of 5 methods/combinations for I-picture only (IIII...) and P-picture (IPPP...). In each figure, we attached a magnified graph that can show the detailed results at small QP values.

Each methods/combinations provide improvement of coding efficiency at high-bitrate. In I-picture only coding, all of the methods and combinations show improvement. In P-picture coding, for few cases (depends on QP and/or sequence), the results tend to be worse than that of original TML. The results of the total combination (Original + Level + Position + Inverse) show the similar characteristics shown in VCEG-O27.

# 4. Conclusion

We have evaluated the methods to improve 2D-VLC encoding scheme for high-bitrate, proposed in VCEG-O27. The results support the argument in VCEG-O27, and show the possibility of the proposed methods. At the same time, the results show the necessity to avoid drawback on some encoding parameters and/or sequence characteristics. Further study should be encouraged to improve 2D-VLC encoding.

# IPR Policy

We argue 2.0. See attached policy statement.

# Refenreces

- [1] Shinya Kadono and Martin Schlockermann, " Improved 2D-VLC coding for High-Bitrate” Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG ,  document VCEG-O27, Thailand. Dec., 2001.

[2] Gary Sullivan, "Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material", document VCEG-N81, ITU-T Video Coding Experts Group, Santa Barbara, CA, USA, 24-27 September, 2001.

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 1: R-D curve of I-picture for Foreman

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 2: R-D curve of P-picture for Foreman

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 3: R-D curve of I-picture for News

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 4: R-D curve of P-picture for News

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 5: R-D curve of I-picture for Container

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 6: R-D curve of P-picture for Container

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 7: R-D curve of I-picture for Silent Voice

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 8: R-D curve of P-picture for Silent Voice

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 9: R-D curve of I-picture for Paris

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 10: R-D curve of P-picture for Paris

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 11: R-D curve of I-picture for Tempete

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 12: R-D curve of P-picture for Tempete

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 13: R-D curve of I-picture for Mobile

(a) QP=2, 6, 10, 16                                  (b) QP=2, 6

Figure 14: R-D curve of P-picture for Mobile

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT DoCoMo, Inc. |  |  |
| Mailing address | 3-5, Hikari-no-oka, Yokosuka, Kanagawa |  |  |
| Country | Japan |  |  |
| Contact person | Minoru Etoh |  |  |
| Telephone | +81 468 40 3515 |  |  |
| Fax | +81 468 40 3788 |  |  |
| Email | etoh@mml.yrp.nttdocomo.co.jp |  |  |
| Place and date of submission | Geneva, Jan.29-Feb.1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Proposed Requirements on Network Adaptation Layer Specification |  |
| Contribution number |  | JVT-B090 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: