| ITU-T Video Coding Experts Group (ITU-T SG16 Q.6) 12th Meeting: Redmond, Washington USA July 19-23,2004 | Document: VCEG-W05 Filename: input.doc |
| --- | --- |

| Title: | Dynamic Reference Generation for Advanced Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to VCEG |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis Jill Boyce Thomson Corporate Research 2 Independence Way, Princeton, NJ, 08540, USA | Tel: | 609-987-7329 609-987-7303 |
|  |  | Email: | alexandros.tourapis@thomson.net jill.boyce@thomson.net |
| Source: | Thomson |  |  |

_____________________________

# Introduction

In this document we reinvestigate the concept of first globally processing a reference within the reference buffer prior to using it as a reference. Such an approach has already been investigated in [1] and in related papers such as [2][3], by warping the references using affine motion compensated prediction. Nevertheless in this document we consider not only the affine transformation of a single image to generate such references, but additional possible transformations such as linear or non-linear spatial filters, or by even allowing the combination of such images using weighted averaging (indirect/global bi-prediction). We believe that considering such methods could lead to further considerable performance improvement compared to the existing H.264 (or JVT, or MPEG-4 AVC) [4] video coding standard and should be further investigated. More specifically such an approach could be considered very similar to the way currently weighted prediction is performed in H.264, where a signal within the slice layer suggests whether transformation/filtering semantics exist for each reference for each list. If this indicator is present and set, additional semantic information is transmitted within the bitstream indicating the transformation introduced to each reference (i.e. filtering method or affine transformation) and its associated parameters (filter taps, affine parameters). Preliminary results appear very promising and we believe that this process should be further investigated.

# Global Parametric Motion Models for Motion Compensation

In [1] it was proposed that motion compensation prediction is performed using not only previously decoded references but also if necessary their warped versions. Warping parameters were affine motion parameters and were transmitted as side information in the bitstream. Motion compensation is performed by addressing blocks within these references without having to consider how such were generated. The pixel displacements dx(x,y) and dy(x,y) for generation of the warped references isdescribed using the set of orthonormal polynomial basis functions proposed in LBC-97-029

dx(x,y)=a1f1(x,y)+a2f2(x,y)+a3f3(x,y)dy(x,y)=a4f4(x,y)+a5f5(x,y)+a6f6(x,y)	(1)

with f1(x,y) ... f6(x,y) the orthonormal polynomial basis functions and a1 ... a6 the motion model coefficients as described in LBC-97-029. Such model enables more efficient performance in the presence of non-translational motion such as zooming and shearing. Other models such as the perspective motion model or the parabolic model can also be used in order to better approximate better curved surfaces and perspective effects.

Although H.264 has introduced several very efficient tools for motion compensation including multiple block sizes, a better design of skip and direct macroblocks/blocks, weighted prediction etc, reference warping could still provide significant benefits in cases with non translational motion as the ones mentioned above. Nevertheless, such temporal transforms might not be the best or only way for generating references for motion estimation and compensation. For example, in the presence of noise (such as film grain noise) a spatial low pass filter (i.e. a median, or n-tap separable low pass filter, or a filter similar to the deblocking filter currently employed by H.264) might be a better approach and could lead to even further gain since such could allow the removal of non temporally correlated noise and therefore improve motion prediction.

An additional concept that seems not to have been properly exploited is also the idea of global, apart from local, bi-prediction. That is, a reference is generated by considering not only a single reference but by combining two references (from the same list) after both have been processed globally using spatial or temporal transformations. Note that this concept enables the usage of bi-prediction even in P slices with very little overhead since signaling is only necessary at the slice layer. Furthermore prediction from up to 4 pictures could now be used when performing motion compensation within B slices. Obviously the combination of more pictures could also be considered.

Signaling for this generalized reference generation method could be made very similar to the existing method used for explicit weighted prediction. For each encoded slice an additional table can be transmitted, which for each list and each reference within such list provides information about the transformation process, if any, that is to be used to generate the final reference. For example, the first picture in list0 is signaled to be filtered first using a 3x1 followed by 1x3 median filter, while the second picture in the same list is signaled to be combined (i.e. using weighted averaging) with the first picture in list0 prior to filtering. Note that such process only generates temporary references and does not impact the actual reference store, similar to what is currently done for weighted prediction.  In Figure 1 and Figure 2 a simplified encoder and decoder respectively with consideration of the above described method are shown.

Figure 1. Generalized Encoder using Adaptive References

Figure 2. Generalized Decoder using Adaptive References

# Implementation/Simulation Results

The proposed method was implemented in reference software 8.2. Currently our software only supports adaptive references using median and N-tap separable filtering, and weighted bi-prediction of two warped pictures using affine motion compensation. Nevertheless, and for simplicity in our encoder, for the later method no warping was performed (zero motion is transmitted) while weights were implicitly computed based on poc distances. To simplify our encoding process even further, we have selected to use only the adaptive weighted bi-prediction method for this test, where reference 1 from each list (if available) is replaced by the weighted averaging between the original reference 1 and reference 0 from the same list. RD decision between normal reference generation and our proposed method was also used. Several CIF sequences were encoded at various QP values using the proposed mode. More specifically we have selected CIF sequences Bus, Container, Football, Foreman, Flower, Tempete, and Table Tennis. Although in this first implementation, reference generation is rather simplistic, improvements of up to 9% can be achieved using our proposed method. Obviously the proper consideration of warping parameters and efficient decision of the transformation process (i.e. spatial or temporal filters, number of taps, estimation of warping parameters etc) for each reference is expected to yield even higher performance.

# Conclusion

In this document additional methods were briefly presented that could allow for further improvement in encoding performance for existing video coding standards such as H.264. The concepts rely on globally temporal and spatial transformations of the existing references prior to motion compensation which essentially leads to an improvement in the quality of the reference images, and therefore to the overall reduction of the motion information and the residual signal needed to be encoded. We therefore recommend that such methods should also be considered for future extensions of video coding standards such as H.265, while the most appropriate spatio-temporal transformations be identified within the framework of such approach.

# References

- T. Wiegand, E. Steinbach, B. Girod,and B. D. Andrews, “Video Coding Using Long-Term Memory and Affine Motion Compensation”, ITU-T/SG16/Q15-G-21, February 1999, Monterey, CA, USA.

- T. Wiegand, E. Steinbach, and B. Girod, “Long-Term Memory Prediction Using Affine Motion Compensation”, Proc. of IEEE International Conference on Image Processing, Kobe, Japan, Oct. 1999, vol. 1, pp. 51-54

- T. Wiegand, E. Steinbach, A. Stensrud, and B. Girod, “Multiple Reference Picture Video Coding Using Polynomial Motion Models”, Proc. SPIE Symposium on Visual Communications and Image Processing (VCIP'98), San Jose, USA, Feb. 1998, pp. 134-145.

- Advanced video coding for generic audiovisual services, http://www.itu.int/rec/recommendation.asp?type=folders&lang=e&parent=T-REC-H.264

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson |  |  |
| Mailing address | Thomson Corporate Research 2 Independence Way Princeton, NJ, 08540 |  |  |
| Country | USA |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | 609-987-7329 |  |  |
| Fax | 609-987-7299 |  |  |
| Email | alexandros.tourapis@thomson.net |  |  |
| Place and date of submission | Redmond, WA USA, 7/17/2004 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | VCEG |  |
| Title |  | Dynamic Reference Generation for Advanced Video Coding |  |
| Contribution number |  | VCEG-W04 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: