| ITU-T Video Coding Experts Group (ITU-T SG16 Q.6) 14th Meeting: Hong Kong, Jan 17-21,2004 | Document: VCEG-Y17.doc Filename: VCEG-Y17.doc |
| --- | --- |

| Title: | A New Intra Prediction Method for RRU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to VCEG |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Peng Yin Jill Boyce Purvin Pandit Thomson Corporate Research 2 Independence Way, Princeton, NJ, 08540, USA | Tel: | 609-987-7355 609-987-7303 |
|  |  | Email: | peng.yin@thomson.net jill.boyce@thomson.net |
| Source: | Thomson |  |  |

# Introduction

Reduced Resolution Update (RRU) mode was proposed to VCEG as a potential tool for achieving improved encoding performance within H.264/MPEG-4 AVC [1] and its future extensions [2][3][4]. In this document, we will propose a new intra prediction method for RRU mode based on two previously proposed methods [2][3]. Compared to [2][3], the new method can remove the blocky artifact and jagged edges on macroblock or inner 8x8 block boundaries for intra coded macroblocks which contain a lot of textures or edges. At the same time, it keeps the complexity of intra prediction as low as [4] and does not need to standardize the downsampling process as required by [4]. The new intra prediction can also benefit the complexity scalable video codec [5].

# RRU Intra Prediction Background

In [2], the spatial intra prediction methods in H.264 using either 4x4 or 16x16 block size is extended to 8x8 and 32x32 block sizes for RRU mode. Then the residue is downsampled and is coded using the same transform and quantization process in H.264. During decoding, the residual data needs to be upsampled. The downsampling process is done only in the encoder, and hence does not need to be standardized. The upsampling process must be matched in the encoder and decoder, and so must be standardized. In [2], since the upsampling is performed on the residue, it is done within 4x4 block. But because the intra prediction in general does not have very good prediction for textures, this approach can result in the blocky artifact and jagged edge along inner 8x8 block boundaries and the macroblock boundaries. In [4], instead of working on the full resolution image as [2], the pixels neighboring the 32x32 coding region are downsampled and placed in a temporary buffer. Then the intra prediction is done exactly the same as in the standard implementation. After the prediction and residue coding is complete, the intra macroblock is upsampled to replace the macroblock in the reference image. In this method, since the downsampling process was also performed at the decoder, it requires to be standardized.

# Our Proposal

Our new intra prediction method is based on [4], but with the following modifications.

- downsampling is only performed at the encoder, so it does not need to be standardized

- constrained_intra_prediction_flag needs to always be set to 1

- different upsampling method is performed for intra macroblock and inter macroblock

- loop filter is modified on macroblock boundary when strength equals to 4

In our proposed method, we also perform the intra prediction on a 16x16 macroblock basis as in [4]. But instead of using the first upsampled then downsampled neighboring pixels for intra prediction [4], we store the low resolution version of the neighboring pixels before we upsample it and use these pixels to perform intra prediction on a 16x16 macroblock basis, as specified in H.264. To be specific, given a 32x32 macroblock, we first downsample it to 16x16 macroblock, and then use the neighboring pixels in low resolution to perform intra prediction. After the prediction and residue coding is complete, we store the right column and bottom row in a temporary buffer. Then the intra macroblock is upsampled to replace the macroblock in the reference image. Since the pixels for low resolution is not available for non-intra macroblock, we need to set constrained_intra_prediction_flag to be 1. In order to perform intra prediction with the proposed method, some additional memory is required. For each MB we need to store its right column (16 pixels for luma, 8 pixels for Cr and 8 pixels for Cb) and bottom row (16 pixels for luma, 8 pixels for Cr and 8 pixels for Cb) i.e., 64 additional bytes for the intra coded macroblock.

In [2], the residue is upsampled within 4x4 blocks. In our proposal, for intra macroblock, since upsampling is performed on pixels, we use 6-tap upsampling filter as in H.264 and the upsampling is done on a 16x16 macroblock basis. In this way, the blocky artifact and jagged edges on inner 8x8 block boundaries can be removed. But since the upsampling does not extend the macroblock boundaries, a discontinuity is produced on a 32x32 boundary. We thus increase the strength for loop filter when bS equals to 4.

A side advantage of our approach is that it can benefit the complexity scalable video coding techniques proposed in [5]. By using this intra coding method combined with intra-refresh techniques, it can stop the drift propagation in reduced-resolution video.

# Simulation Result

Our proposed method aims at removing the subjective coding artifact for intra-coded macroblocks. Similar to [4], since this method loses the high-frequency information during the downsampling process, the PSNR is not as good as in [2] for intra-coded macroblocks. But if we only use RRU for inter-pictures, which is typically the case in real applications, the PSNR does not get affected. This is because fewer intra macroblocks are used in inter-pictures. Table 1 shows the average bitrate and PSNR of our proposed method versus [2], where RRU mode is only used for inter-pictures. Figure 1 illustrates a visual comparison of the intra-coded ball from Mobile using the old method in [2] and our new method. Figure 2 illustrates a visual comparison of the intra-coded player from Football using the old method in [2] and our new method. We can see clearly that with the new method, the jagged edges in the ball or the back of the player are smoothed.

Table 1 Comparison of the proposed method versus [2]

|  |  |  |  | old method in [2] |  |  | proposed method |  | Average bitrate | Average PSNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | Resolution | frames | QP | PSNRY | bitrate | QP | PSNRY | bitrate |  |  |
| Bus | CIF | 150 | 28 | 29.59 | 486.88 | 28 | 29.59 | 486.64 | -0.14 | 0.005 |
|  |  |  | 32 | 28.29 | 320.9 | 32 | 28.29 | 320.53 |  |  |
|  |  |  | 36 | 26.68 | 194 | 36 | 26.67 | 193.39 |  |  |
|  |  |  | 40 | 25.05 | 118.82 | 40 | 25.06 | 118.40 |  |  |
| Football | CIF | 260 | 28 | 30.96 | 608.77 | 28 | 30.92 | 616.72 | 2.42 | -0.072 |
|  |  |  | 32 | 29.69 | 404 | 32 | 29.64 | 406.75 |  |  |
|  |  |  | 36 | 28.14 | 242.63 | 36 | 28.08 | 243.39 |  |  |
|  |  |  | 40 | 26.62 | 143.94 | 40 | 26.52 | 142.70 |  |  |
| Foreman | CIF | 300 | 28 | 33.94 | 223.63 | 28 | 33.93 | 223.59 | 0.32 | -0.012 |
|  |  |  | 32 | 32.32 | 145.42 | 32 | 32.30 | 145.37 |  |  |
|  |  |  | 36 | 30.50 | 91.1 | 36 | 30.49 | 91.14 |  |  |
|  |  |  | 40 | 28.67 | 58.46 | 40 | 28.65 | 58.21 |  |  |
| Mobile | CIF | 300 | 28 | 28.99 | 517.59 | 28 | 28.99 | 517.57 | -0.12 | 0.004 |
|  |  |  | 32 | 27.76 | 344.83 | 32 | 27.76 | 344.81 |  |  |
|  |  |  | 36 | 26.11 | 217.38 | 36 | 26.12 | 217.45 |  |  |
|  |  |  | 40 | 24.21 | 137.46 | 40 | 24.21 | 137.37 |  |  |

| method [2] | proposed method |
| --- | --- |

Figure 1 Comparison of visual quality of an intra-coded ball in frame 11

| method [2] | proposed method |
| --- | --- |

Figure 2 Comparison of visual quality of an intra-coded football player in frame 22

# Conclusion

We have presented in this document the new intra prediction method for RRU. The method can improve the subjective quality of RRU intra coding while not increasing the coding complexity. The side advantage of this method is to help reduce the drift propagation for reduced-resolution video using intra-refresh for complexity scalable video codec [5].

# References

- Advanced video coding for generic audiovisual services, http://www.itu.int/rec/recommendation.asp?type=folders&lang=e&parent=T-REC-H.264

- A.M. Tourapis, J. Boyce, “Reduced Resolution Update Mode for Advanced Video Coding”, ITU-T Q6/SG16, document VCEG-V05, Munich, Germany.

- A.M. Tourapis, J. Boyce, “New Results on Reduced Resolution Mode”, ITU-T Q6/SG16, document VCEG-W04, Redmond, USA.

- J. Sievers and D. Lindbergh, “Reduced Resolution Update for Computational Efficiency”, ITU-T Q6/SG16, document D.7, Geneva, Nov. 2004.

- P. Yin, J. Boyce, P. Pandit, A.M. Tourapis, “Compleixty Scalable Video Codec”,ITU-T Q6/SG16, document VCEG-X06, Palma de Mallorca, Spain

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson |  |  |
| Mailing address | Thomson Corporate Research 2 Independence Way Princeton, NJ, 08540 |  |  |
| Country | USA |  |  |
| Contact person | Peng Yin |  |  |
| Telephone | 609-987-7355 |  |  |
| Fax | 609-987-7299 |  |  |
| Email | peng.yin@thomson.net |  |  |
| Place and date of submission | Hongkong, China, 01/17/2005 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | VCEG |  |
| Title |  | A New Intra Prediction Method for RRU |  |
| Contribution number |  | VCEG-X17 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: