| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 15th Meeting: Busan, KR, 16-22 April, 2005 | Document VCEG-Z07 Filename: VCEG-Z07.doc Generated: 5 Apr ‘05 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Arild Fuldseth, Harald Dankworth, Tandberg, Philip Pedersens vei 22 1366 Lysaker, Norway | Tel: Fax: Email: | +47 67125125 +47 67125234 arild.fuldseth@tandberg.net harald.dankworth@tandberg.net |
| Title: | Software: Tandberg computational efficiency tools |  |  |
| Purpose: | Information |  |  |

# Background

Several tools for computational efficiency (CE) tools have been proposed by Tandberg in the recent VCEG meetings:

- Efficient deblocking filter (VCEG-W10, VCEG-X04)

- Efficient CAVLC (VCEG-Y08)

- Efficient interpolation (VCEG-W09, VCEG-Z08)

- Integer vectors preference (VCEG-Z09)

This document describes how these tools are implemented in the reference software.

# Modifications to the Reference Software

The computational efficiency tools have been included in JM8.2.

## Encoder Configuration File

The following flags have been added to the encoder configuration:

CeParameterFlag	= 1	# 0: parameters below ignored, 1: parameters sent

CEinterpolation 	= 1	# Efficient interpolation (0: off, 1: on)

CEloop         	= 1 	# Efficient deblocking filter (0: off, 1: on)

CECAVLC		= 1 	# Efficient CAVLC (0: off, 1: on)

CEskip		= 1 	# Integer vector preference (0: off, 1: on)

## Sequence Parameter Set

To synchronize the encoder and the decoder, we use a new value for profile_idc:

profile_idc = 55.

Furthermore, the following syntax elements (flags) have been added to the sequence parameter set:

| if(profile_idc == 55) { |  |  |
| --- | --- | --- |
| ce_interpolation_flag | 1 | u(1) |
| ce_deblocking_filter_flag | 1 | u(1) |
| ce_CAVLC_flag | 1 | u(1) |
| ce_integer_vector_preference_flag | 1 | u(1) |
| } |  |  |

# Special notes

- The CAVLC has been modified slightly compared to VCEG-Y08. The codewords have been limited to maximum 32 bits, resulting in slightly better compression and lower complexity.

- Use of small block sizes (8x4, 4x8, and 4x4) is incompatible with the new loop filter.

# Further work

- Merge with the latest JM version.

# Software distribution

Send an email to arild.fuldseth@tandberg.net