| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) Busan, KR, 16-22 April, 2005 | Document VCEG-Z11 Filename: VCEG-Z11.doc Generated: 19 March, 05 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Hongbo Zhu, Song Zhang, Institute Of Computing Technology, Chinese Academy of Science, Beijing, China | Tel: Fax: Email: | +86 010 62120683 zhuhb@ict.ac.cn zhangs@ict.ac.cn |
| Title: | Improved VLC coding of transform coefficients |  |  |
| Purpose: | Proposal. |  |  |

_____________________________

# Background

The CAVLC used in H.264 proved to be considerably more efficient for transform coefficient coding than the initially used UVLC.  However, the CAVLC is also considerably more complex to implement both for encoding and decoding, and is much less efficient than the CABAC. In this proposal, we proposed two modifications to the CAVLC to make it faster and more efficient. The first modification is the coding of the coefficients number and the coefficients’ magnitudes, and the second is the coding of zerobefore.

# Coding of Coefficients Number and Magnitude

Firstly, the combination of non-zero coefficients number and the number of coefficients that have absolute value 1 is coded as a syntax element. The combination is mapped to a index firstly, then the index is coded by one of the Huffman code tables tab14-tab22 in appendix. The map is as follows:

- Map the combination to index3 or index4 by table 1 and table 2.

| Index3 | a0 | a1 | a2 | a3 | a4 | a5 | a6 | a7 | a8 | a9 | a10 | a11 | a12 | a13 | a14 | a15 | a16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cfnum | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Abs1num | 0 | 1 | 2 | 3 | - | - | - | - | - | - | - | - | - | - | - | - | - |

Table 1

| Index4 | b0 | b1 | b2 | b3 | b4 | b5 |
| --- | --- | --- | --- | --- | --- | --- |
| Cfnum | 3 | 2 | 1 | 3 | 2 | 3 |
| Abs1num | 2 | 1 | 0 | 1 | 0 | 0 |

Table 2 (Let nA and nB be the numbers of non-zero transform coefficient levels in the block located to the left of the current block and the block located above the current block, respectively. When nA+nB>6,  change the b2 to b3 and  change the b3 to b2)

- Intercross index3 and index4 into index1 by a intercross factor itlv. Let cfnum denotes the non-zero coefficients number in the current block, abs1num denotes the number of coefficients that have absolute value 1 in the current block. The intercross is exampled as follows:

When itlv is 6:

| Index1 | 0 | 1 | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| index | a0 | a1 | a2 |  | a3 |  | a4 |  | a5 |  | b0 |  | a6 | b1 | a7 | b2 | a8 | b3 | a9 | b4 | a10 | b5 |
| Index1 | 17 | 18 |  | 19 |  | 20 |  | 21 |  | 22 |  |  |  |  |  |  |  |  |  |  |  |  |
| index | a11 | a12 |  | a13 |  | a14 |  | a15 |  | a16 |  |  |  |  |  |  |  |  |  |  |  |  |

When itlv is 14:

| Index1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index | a0 | a1 | a2 | a3 | a4 | a5 | a6 | a7 | a8 | a9 | a10 | a11 | a12 | a13 | b0 | a14 | b1 |
| Index1 | 17 | 18 | 19 | 20 | 21 | 22 |  |  |  |  |  |  |  |  |  |  |  |
| index | a15 | b2 | a16 | b3 | b4 | b5 |  |  |  |  |  |  |  |  |  |  |  |

- Map the index1 to index2 by a center. When center is 12:

| Index1 |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index2 |  | 23 |  | 22 |  | 20 |  | 18 |  | 16 |  | 14 |  | 12 |  | 10 | 8 | 6 | 4 | 2 | 1 | 3 | 5 |
| 15 | 16 |  | 17 |  | 18 |  | 19 |  | 20 |  | 21 |  | 22 |  |  |  |  |  |  |  |  |  |  |
| 7 | 9 |  | 11 |  | 13 |  | 15 |  | 17 |  | 19 |  | 21 |  |  |  |  |  |  |  |  |  |  |

When center=5:

| Index1 |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Index2 |  | 10 |  | 8 |  | 6 |  | 4 |  | 2 |  | 1 |  | 3 |  | 5 | 7 | 9 | 11 | 12 | 13 | 14 | 15 |
| 15 | 16 |  | 17 |  | 18 |  | 19 |  | 20 |  | 21 |  | 22 |  |  |  |  |  |  |  |  |  |  |
| 16 | 17 |  | 18 |  | 19 |  | 20 |  | 21 |  | 22 |  | 23 |  |  |  |  |  |  |  |  |  |  |

- Code index2 using one denoted by tabx of the Huffman code tables tab14-tab22 in appendix.

Tabx, center and itlv are selected according to ltsum=nA+nB. They are listed in Table 3.

| Ltsum | tabx | itlv | center |
| --- | --- | --- | --- |
| 0 | 14 | 7 | 0 |
| 1 | 14 | 5 | 0 |
| 2 | 16 | 6 | 0 |
| 3 | 16 | 6 | 0 |
| 4 | 16 | 6 | 0 |
| 5 | 17 | 7 | 0 |
| 6 | 19 | 7 | 0 |
| 7 | 19 | 9 | 0 |
| 8 | 19 | 8 | 3 |
| 9 | 20 | 9 | 4 |
| 10 | 20 | 10 | 5 |
| 11 | 21 | 10 | 5 |
| 12 | 21 | 11 | 6 |
| 13 | 21 | 12 | 6 |
| 14 | 21 | 12 | 6 |
| 15 | 21 | 13 | 7 |
| 16 | 21 | 14 | 7 |
| 17 | 20 | 15 | 8 |
| 18 | 20 | 15 | 8 |
| 19 | 19 | 15 | 8 |
| >=20 | 20 | 16 | 10 |

Table 3 coding parameter selection for combination symbols(If only one of nA and nB do not exists, ltsum=2*nC, in which nC exists. If neither nA nor nB are existed, ltsum=0)

When cfnum>3, the abs1num need to be coded as a syntax element. Firstly, the abs1num is mapped to an index by a center, the map is similar to that above. Then, the index is coded by one denoted by tabx of the code tables tab0-tab9 in the appendix. The tabx and center are selected according to the type and non-zero coefficients number of the current block, as listed in table 4.

| cfNum BlkType |  | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter | Tabx | 0 | 0 | 1 | 2 | 2 | 3 | 3 | 4 | 5 | 5 | 5 | 5 | 8 |
|  | Center | 4 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 2 |
| Other | Tabx | 0 | 1 | 1 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 5 | 8 |
|  | Center | 4 | 4 | 4 | 4 | 4 | 5 | 4 | 5 | 5 | 5 | 5 | 4 | 2 |

Table 4 The selection of tabx and center

For the block with the type chromadc, the cfnum and abs1num are mapped to index2 by the table5.  Index2 is coded by tab0 in appendix. When cfnum>2, the abs1num need to be coded as a single symbol. When cfnum is 3, abs1num will be coded under the parameters center=3 and tab0. When cfnum is 4, the parameters is tab1 and center=0.

| Index2 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cfnum | 0 | 1 | 2 | 3 | 4 | 2 | 1 | 2 |
| Abs1num | 0 | 1 | 2 | - | - | 1 | 0 | 0 |

Table 5

Zig-zag scanning is applied to the current 4x4 block orderly to get the non-zero coefficients array level[] and the run array run[]. The array level[] is continued to run-length scanning reversely in which the consecutive non-zero coefficients with absolute value 1 are looked upon run. For example, level[]={7, 5, -1, 2, -3, 1, 1, 1} is run-length coded, the results is the run array run1[]={3, 0, 1, 0} and the level array levelThan1[]={-3, 2, 5, 7}. The array run1[] is coded firstly. The run is mapped to index by a center firstly, then the index is coded by one denoted by tabx of the Huffman code tables tab0-tab9 in the appendix.

The parameters center and tabx are selected according to the number (denoted by leftT1num) of lefted coefficients that have an absolute value than 1 and the number (denoted by leftabs1num) of lefted coefficients that have an absolute value 1, as listed in table 6.

| LeftT1num Leftabs1num | 1 |  | 2 |  | 3 |  | >=4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | tabx | center | tabx | center | tabx | center | tabx | center |
| 3 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| 4 | 1 | 3 | 1 | 0 | 1 | 0 | 1 | 0 |
| 5 | 2 | 5 | 2 | 3 | 1 | 0 | 1 | 0 |
| 6 | 3 | 6 | 3 | 3 | 2 | 2 | 1 | 0 |
| 7 | 3 | 7 | 3 | 3 | 3 | 2 | 2 | 2 |
| >=8 | 4 | 5 | 5 | 4 | 5 | 4 | 3 | 2 |

Table 6 the parameters for the coding of the array run1[]

When leftT1num=1 and leftabs1num=2, tabx=0, center=2.

When leftT1num>1 and leftabs1num=2 or leftabs1num=1, the number (denoted by trail0) of consecutive 0 in run1[] rather than run is coded. For example: run1[]={3, 0, 0, 1}, when 3 has been coded, leftabs1num=1, trail0=2. Trail0 is mapped to index by a center, then the index is coded by one (denoted by tabx) of the Huffman code tables tab0-tab9 in appendix. When leftabs1num=2, tabx=0, center=0. When leftabs1num=1, set center=0, tabx is selected according to leftT1num, as listed in table 7.

| leftT1num | 2 | 3 | 4 | 5 | 6 | 7 | >=8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| tabx | 0 | 0 | 0 | 1 | 2 | 2 | 3 |

Table 7

The array levelThan1[] is coded subsequently. Firstly, the absolute value of each element in levelThan1[] is decreased by 1. For example, levelThan1[]={-3, 2, 5, 7} become levelThan1[]={-2, 1, 4, 6}. The coding of levelThan1[] is similar to that of coding residual coefficients before trailingones in CAVLC. The table to change the VLC is different from the CAVLC, it is {3, 7, 14, 28, 56}.

# New coding method of the zerobefore

In the CAVLC, the VLC table for coding of zerobefore is selected only according to lefted total zeros, lefttotzero. In this proposal, it will be selected according to lefttotzero and lefted total number (denoted by leftcfnum) of non-zero coefficients.  When lefttotzero>2,  zerobefore is mapped to index firstly, as shown in follows:

When leftcfnum>2, index=zerobefore+1, otherwise, when leftcfnum=2, index is queried from the table 8.

| zerobefore lefttotzero | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | 1 | 2 | 3 | 4 |  |  |  |  |  |  |  |  |  |  |  |
| 4 | 1 | 4 | 5 | 2 | 3 |  |  |  |  |  |  |  |  |  |  |
| 5 | 1 | 3 | 4 | 5 | 6 | 2 |  |  |  |  |  |  |  |  |  |
| 6 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |  |
| 7 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |  |  |  |  |
| 8 | 2 | 3 | 8 | 9 | 4 | 1 | 5 | 7 | 6 |  |  |  |  |  |  |
| >=9 | 7 | 8 | 5 | 1 | 2 | 3 | 4 | 6 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |

Table 8 the map table of mapping the zerobefore to index when leftcfnum=2

The index is coded by one (denoted by tabx) of the Huffman code tables tab0-tab9 in the appendix. The parameter tabx is gotten in table 9.

| leftcfnum lefttotzero | 2 | 3 | 4 | 5 | 6 | >=7 |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 1 | 1 | 0 | 0 | 0 | 0 |
| 4 | 1 | 1 | 1 | 0 | 0 | 0 |
| 5 | 2 | 2 | 2 | 0 | 0 | 0 |
| 6 | 3 | 3 | 3 | 1 | 1 | 0 |
| 7 | 3 | 3 | 2 | 2 | 2 | 0 |
| 8 | 2 | 3 | 3 | 3 | 2 | 1 |
| >=9 | 5 | 5 | 3 | 2 | 2 | 1 |

Table 9 selection of  the tabx for coding of zerobefore

When lefttotzero=2 and leftcfnum=2, index=3-zerobefore, tabx=0.

When lefttotzero=2 and leftcfnum>2, or lefttotzero=1, The number (denoted by run0num) of consecutive run that have value 0 rather than runbefore is coded.  For example, run[]={0,1, 0, 0, 0, 2, 4}, when 4 and 2 are coded subsequently, lefttotzero=1 and run0num=3. Let index=run0num+1, the index will be coded by one (denoted by tabx) of the Huffman code tables  tab0-tab9 in appendix. The tabx is selected according to the parameters lefttotzero and leftcfnum, as shown in Table 10.

| leftcfnum lefttotzero | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | >=13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 5 | 5 | 6 |
| 2 | 0 | 0 | 0 | 1 | 9 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |

Table 10 selection of  the tabx for coding of run0num

# Simulation results

Test conditions similar to the ones in H.264 are used.  Frame rates and Qp values are kept the same. Avsnr performance relative to CAVLC for the different sequences are listed in table 11. The detailed results can be found in the spreadsheet VCEG-Z11.xls.

| Sequence | BDPSNR between the new entropy coding and the CAVLC (%) |
| --- | --- |
| Container | 0.097262 |
| News | 0.062767 |
| Foreman | 0.001808 |
| Silent | -0.0147 |
| Paris | 0.128046 |
| Mobile | 0.211028 |
| Tempete | 0.132941 |
| Average | 0.08845 |

Table 11 BDPSNR difference between the proposal and the CAVLC (A positive value denotes in favor of our algorithms)

This shows that coding performance of the new entropy coding algorithm in the proposal is practically superior to the CAVLC by 0.088dB on average.

# Conclusions

A new method of the entropy coding of 4x4 DCT coefficients has been described in the proposal.  It results in better coding performance than the CAVLC.  As all semantics elements are coded by regular Huffman codes, it is expected that the decoder of the method described in the proposal is faster than that of the CAVLC.

# Appendix (VLC tables)

| Index | Tab14 | Tab15 | Tab16 | Tab17 |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 10 | 01 |  |
| 2 | 01 | 01 | 11 | 100 |  |
| 3 | 001 | 001 | 010 | 101 |  |
| 4 | 00010 | 0001 | 011 | 110 |  |
| 5 | 00011 | 000010 | 0010 | 111 |  |
| 6 | 000010 | 000011 | 0011 | 0010 |  |
| 7 | 000011 | 0000010 | 00010 | 0011 |  |
| 8 | 0000010 | 0000011 | 00011 | 00010 |  |
| 9 | 0000011 | 00000010 | 000010 | 00011 |  |
| 10 | 00000010 | 00000011 | 000011 | 000010 |  |
| 11 | 00000011 | 000000010 | 0000010 | 000011 |  |
| 12 | 000000010 | 000000011 | 0000011 | 0000010 |  |
| 13 | 000000011 | 0000000010 | 00000010 | 0000011 |  |
| 14 | 0000000010 | 0000000011 | 00000011 | 00000010 |  |
| 15 | 0000000011 | 00000000010 | 000000010 | 00000011 |  |
| 16 | 00000000010 | 00000000011 | 000000011 | 000000010 |  |
| 17 | 00000000011 | 000000000010 | 0000000010 | 000000011 |  |
| 18 | 000000000010 | 000000000011 | 0000000011 | 0000000010 |  |
| 19 | 000000000011 | 0000000000010 | 00000000010 | 0000000011 |  |
| 20 | 0000000000010 | 0000000000011 | 00000000011 | 00000000010 |  |
| 21 | 0000000000011 | 00000000000010 | 000000000010 | 00000000011 |  |
| 22 | 00000000000010 | 00000000000011 | 000000000011 | 000000000010 |  |
| 23 | 00000000000011 | 000000000000010 | 0000000000010 | 000000000011 |  |
| Index | Tab18 | Tab19 | Tab20 | Tab21 | Tab22 |
| 1 | 001 | 001 | 001 | 001 | 001 |
| 2 | 010 | 010 | 010 | 010 | 010 |
| 3 | 011 | 011 | 011 | 011 | 0110 |
| 4 | 100 | 100 | 100 | 1000 | 0111 |
| 5 | 101 | 101 | 1010 | 1001 | 1000 |
| 6 | 110 | 1100 | 1011 | 1010 | 1001 |
| 7 | 1110 | 1101 | 1100 | 1011 | 1010 |
| 8 | 1111 | 1110 | 1101 | 1100 | 1011 |
| 9 | 00010 | 1111 | 1110 | 1101 | 1100 |
| 10 | 00011 | 00010 | 1111 | 1110 | 1101 |
| 11 | 000010 | 00011 | 00010 | 1111 | 1110 |
| 12 | 000011 | 000010 | 00011 | 00010 | 1111 |
| 13 | 0000010 | 000011 | 000010 | 00011 | 00010 |
| 14 | 0000011 | 0000010 | 000011 | 000010 | 00011 |
| 15 | 00000010 | 0000011 | 0000010 | 000011 | 000010 |
| 16 | 00000011 | 00000010 | 0000011 | 0000010 | 000011 |
| 17 | 000000010 | 00000011 | 00000010 | 0000011 | 0000010 |
| 18 | 000000011 | 000000010 | 00000011 | 00000010 | 0000011 |
| 19 | 0000000010 | 000000011 | 000000010 | 00000011 | 00000010 |
| 20 | 0000000011 | 0000000010 | 000000011 | 000000010 | 00000011 |
| 21 | 00000000010 | 0000000011 | 0000000010 | 000000011 | 000000010 |
| 22 | 00000000011 | 00000000010 | 0000000011 | 0000000010 | 000000011 |
| 23 | 000000000010 | 00000000011 | 00000000010 | 0000000011 | 0000000010 |

Regular Huffman codes Table 1 (For coding of combination symbol of the number of non-zero coefficients and the number of the coefficients with a absolute value 1)

(tab14 and tab15 are the combination of Golomb code with parameter p=2 and Golomb code with parameter p=1, here, the parameter p denotes that the binary part provides p variations to a unary code. Tab16 is the Golomb code with p=2. tab17-tab22 is the combination of Golomb code with p=2 and a complete binary tree code, for example, the tab19, when the index is less than 10, it is complete binary tree code. When the index is more than 9, it is a Golomb code with p=2)

| index | Tab0 | Tab1 | Tab2 | Tab3 | Tab4 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 01 | 01 | 01 | 001 |
| 2 | 01 | 10 | 10 | 100 | 010 |
| 3 | 001 | 11 | 110 | 101 | 011 |
| 4 | 0001 | 001 | 111 | 110 | 100 |
| 5 | 00001 | 0001 | 001 | 111 | 101 |
| 6 | 000001 | 00001 | 0001 | 001 | 110 |
| 7 | 0000001 | 000001 | 00001 | 0001 | 111 |
| 8 | 00000001 | 0000001 | 000001 | 00001 | 0001 |
| 9 | 000000001 | 00000001 | 0000001 | 000001 | 00001 |
| 10 | 0000000001 | 000000001 | 00000001 | 0000001 | 000001 |
| 11 | 00000000001 | 0000000001 | 000000001 | 00000001 | 0000001 |
| 12 | 000000000001 | 00000000001 | 0000000001 | 000000001 | 00000001 |
| 13 | 0000000000001 | 000000000001 | 00000000001 | 0000000001 | 000000001 |
| 14 | 00000000000001 | 0000000000001 | 000000000001 | 00000000001 | 0000000001 |
| 15 | 000000000000001 | 00000000000001 | 0000000000001 | 000000000001 | 00000000001 |
| 16 | 0000000000000001 | 000000000000001 | 00000000000001 | 0000000000001 | 000000000001 |
| index | Tab5 | Tab6 | Tab7 | Tab8 | Tab9 |
| 1 | 001 | 001 | 001 | 001 | 1 |
| 2 | 010 | 010 | 010 | 010 | 001 |
| 3 | 011 | 011 | 011 | 011 | 010 |
| 4 | 100 | 100 | 100 | 1000 | 011 |
| 5 | 101 | 101 | 1010 | 1001 | 0001 |
| 6 | 110 | 1100 | 1011 | 1010 | 00001 |
| 7 | 1110 | 1101 | 1100 | 1011 | 000001 |
| 8 | 1111 | 1110 | 1101 | 1100 | 0000001 |
| 9 | 0001 | 1111 | 1110 | 1101 | 00000001 |
| 10 | 00001 | 0001 | 1111 | 1110 | 000000001 |
| 11 | 000001 | 00001 | 0001 | 1111 | 0000000001 |
| 12 | 0000001 | 000001 | 00001 | 0001 | 00000000001 |
| 13 | 00000001 | 0000001 | 000001 | 00001 | 000000000001 |
| 14 | 000000001 | 00000001 | 0000001 | 000001 | 0000000000001 |
| 15 | 0000000001 | 000000001 | 00000001 | 0000001 | 00000000000001 |
| 16 | 00000000001 | 0000000001 | 000000001 | 00000001 | 000000000000001 |

Regular Huffman codes table 2   (When the coded symbol has a span, a simple truncation is applied to the huffman codes.  For example, the tab5, when a symbol is in range [1, 10], the code for the index 10 is 0000 rather than 00001)

(tab0 is a Golomb code with p=1. tab1-tab8 is the combination of Golomb codes with p=1 and complete binary tree codes. For example, the tab6, when index<10, it is a complete binary tree codes, when index>=10, it is a Golomb code with p=1. The tab9 is a combination of the first bit and tab1.)