| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 26th Meeting: Busan, KR, 16-22 April, 2005 |  |
| --- | --- |

|  |  |  |  | +86 571 87953107 +86 571 87952116 yul@zju.edu.cn |  |
| --- | --- | --- | --- | --- | --- |
| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 26th Meeting: Busan, KR, 16-22 April, 2005 |  |  | Document VCEG-Z14r1 Filename: VCEG-Z14r1.doc |  |  |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Lu Yu, Feng Yi, Zhejiang University ZheDa Road 38, 310027 Hangzhou, China | Tel: Fax: Email: | +86 571 87953107 +86 571 87952116 yul@zju.edu.cn |
| Title: | Low complexity intra prediction |  |  |
| Purpose: | Proposal. |  |  |

- Introduction

Intra prediction is introduced in H.264[1] to reduce the spatial redundancy and make the coding of intra block more efficiency. H.264 supports two types of intra coding: Intra_4×4 and Intra_16×16.

Intra_16×16 is introduced into H.264 to code very smooth areas. To further improve the coding efficiency, a 4×4 Hadamard transform is applied to the 4x4 DC coefficients when Intra_16×16 is used. The Hadamard transform and reordering of the transform coefficients increases the complexity of both encoder and decoder. Additionally, the Intra_16x16_Plane mode is extremely complicated. If Intra_16x16 is not supported, averagely 0.08dB lose of AVSNR is introduced in all–I sequences. (QP=22, 24, 28, 32)

| Sequence | All-I without 16x16_intra AVSNR gain to H.264 (dB) |
| --- | --- |
| Bus_cif | -0.03408 |
| Bus_qcif | -0.01572 |
| Football_cif | -0.05862 |
| Football_qcif | -0.04257 |
| Foreman_cif | -0.14076 |
| Foreman_qcif | -0.05215 |
| Mobile_cif | -0.00986 |
| Mobile_qcif | -0.0016 |
| News_cif | -0.39686 |
| News_qcif | -0.09538 |
| Paris_cif | -0.06128 |
| Paris_qcif | -0.00447 |
| Tempete_cif | -0.09517 |
| Tempete_qicf | -0.0425 |
| Average | -0.07507 |

The reason is that Intra_4×4 is not efficient enough in coding areas with similar texture. At least 16 bits per macroblock are required to code the mode information of 16 4×4 blocks, when the intra prediction modes of all 4x4 blocks are Most_Probable_Mode.

IPPP result without Intra_16x16 is shown in the following table. (QP= 28, 32, 36, 40)

| Sequence | IPPP without 16x16_intra AVSNR gain to H.264 (dB) |
| --- | --- |
| Bus_cif | -0.03868 |
| Football_cif | -0.12082 |
| Tempete_cif | -0.03089 |
| Average | -0.06346 |

- Proposed Method: Direct Intra Prediction (DIP)

Based on the analysis above, we propose a simple but efficient intra prediction mode, i.e., Direct Intra Prediction (DIP) to replace the conventional Intra_16×16 intra prediction in H.264. Compared to Intra_16×16, DIP has greatly lower implementation complexity of both encoder and decoder with no significant loss in coding efficiency.

In Direct Intra Prediction, all modes of the 4×4 blocks in an intra macroblock are their Most_Probable_Mode. In this situation, no mode information needs to be coded and transmitted except a DIP flag. This intra prediction mode is called Direct Mode and uses to replace all Intra_16x16 modes.

On encoder side, if current macroblock is intra, the encoder decides whether Direct_Mode or Intra_4×4 is used to this macroblock. There is a macroblock type to indicate it. Similar to method in H.264, macroblock type 0 is used to indicate the macroblock is Intra_4×4, and the CBP information is combined in macroblock type when Direct_Mode is used. In which, the code_number of the macroblock type is the value of CBP after mapping plus 1.

On decoder side, decoder first reads macroblock type, then decide the macroblock is Direct_Mode or Intra_4×4, and then decodes the macroblock with Direct_Mode or Intra_4×4.

We propose Intra_16x16 is no longer used in fast profile. In both I-Slice and P-Slice, DIP and normal Intra_4x4 are used for intra prediction.

- Complexity analysis

DIP is much simpler than Intra_16×16, for at least three reasons.

First, all modules of Intra_4×4 prediction can be used for DIP, thus the architectural complexity is reduced more than 50%.

Second, when using DIP, encoder has no need to search for the best mode of Intra_16×16 any more, which generally costs much time in H.264 encoder. The computational complexity can be reduced at least 3/(9+4)=23.1% (Only 9 Intra_4×4 modes and DIP mode in the proposed method, while 9 Intra_4×4 modes and 4 Intra_16×16 modes in H.264).

Third, no DC hadamard transform, coefficients reordering and relative entropy coding is required.

- Experiment result

Less than 0.01dB average AVSNR lose is introduced by replacing Intra_16x16 with DIP in all-I sequences. More details can be seen in the following table. (QP=22, 24, 28, 32)

| Sequence | AVSNR gain of proposed method to H.264 (dB) |
| --- | --- |
| Bus_cif | 0.015129 |
| Bus_qcif | 0.019771 |
| Football_cif | 0.014785 |
| Football_qcif | 0.000586 |
| Foreman_cif | -0.06576 |
| Foreman_qcif | -0.02338 |
| Mobile_cif | -0.0046 |
| Mobile_qcif | 0.000489 |
| News_cif | -0.11106 |
| News_qcif | 0.048747 |
| Paris_cif | 0.00637 |
| Paris_qcif | 0.009375 |
| Tempete_cif | -0.02235 |
| Tempete_qicf | -0.00304 |
| Average | -0.00821 |

Less than 0.01dB average AVSNR lose is introduced by replacing Intra_16x16 with DIP in IPPP sequences. More details can be seen in the following table. (QP=22, 24, 28, 32)

| Sequence | AVSNR gain of proposed method to H.264 (dB) |
| --- | --- |
| Bus_cif | -0.00752 |
| Bus_qcif | -0.00933 |
| Football_cif | -0.01087 |
| Football_qcif | -0.01004 |
| Foreman_cif | -0.02453 |
| Foreman_qcif | -0.00871 |
| Mobile_cif | -0.00048 |
| Mobile_qcif | -0.00549 |
| News_cif | -0.04149 |
| News_qcif | -0.01754 |
| Paris_cif | -0.01036 |
| Paris_qcif | 0.026033 |
| Tempete_cif | -0.00487 |
| Tempete_qicf | 0.004358 |
| Average | -0.00863 |

- Conclusion

We propose DIP to replace Intra_16*16 in H.264 to simplify both encoder and decoder with no significant cost of compression efficiency.

References:

[1] Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264/ISO/IEC 14496-10 AVC), Mar. 2003.

# IPR statement

Zhejiang University may have IPR connected to the proposed method and based on reciprocity Zhejiang University is prepared to grant a license on reasonable and non discriminatory terms.