|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 16 – D 140 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2005-2008 |  |  | (WP 3/16) |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 6/16 | Geneva, 26 July - 5 August 2005 |  |  |
| STUDY GROUP 16 – DELAYED CONTRIBUTION 140 |  |  |  |  |  |
| Source: |  | United States |  |  |  |
| Title: |  | Complexity Analysis: H.264 vs. Computationally Efficient H.264 |  |  |  |

# Abstract

This contribution, which is provided for information, contains a computational complexity analysis performed by an independent party in the United States in which the interpolation and deblocking filter subfunctions of a H.264 software decoder are compared with analogous subfunctions of a computationally efficient variant of H.264 (H.264 CE).  Cycle reduction estimates of H.264 CE over H.264 range from an average of 59.5% for the deblocking filter subfunction to 54.2% for interpolation. In addition, we present a detailed interpolation subfunction analysis in which Single Instruction Multiple Data (SIMD) parallelism is exploited for both H.264 and H.264 CE.  Despite the fact that SIMD provided an average estimated cycle reduction of more than 50% for both H.264 and H.264 CE the relative results remained substantially unchanged.  Our results validate previously reported complexity measurements.

# 1	Introduction

In this contribution, we present results of a computational complexity analysis in which the interpolation and deblocking filter subfunctions of a H.264 [1] software decoder are compared with analogous subfunctions of a computationally efficient variant of H.264 (H.264 CE) [2], [3], [4], [5], and [7]. Together, these subfunctions account for on the order of 58% of a decoder’s overall computational complexity [6].  To develop the complexity estimates, we used a methodology described briefly Section 2 and in detail in [6].  The set of common conditions on which the analysis is based are described in Section 3 and the results are highlighted in Section 4 and presented in detail in this Annex A.  This contribution is intended to provide information and does not contain a proposal.

# 2	Methodology Overview

In our analysis, we use two basic steps to develop an estimate of computational complexity for a given subfunction.  In step one, we estimate the total number of cycles required to execute a subfunction on a chosen hardware platform.  To compute the cycle estimate, we

- Develop a “subfunction operation count” table to account for the number of fundamental operations required to perform a subfunction,

- Account for SIMD parallelism where applicable and,

- Map the operations onto a selected hardware platform (the PNX1500 in this case).

In step two, we multiply the result of step one by the appropriate entry in a “subfunction frequency” table (e.g. Table 2 of [6]).  The subfunction frequency table contains information regarding the frequency of use of various subfunctions over the set of common condition sequences.

We note that the complexity analysis results serve as a lower bound on computational complexity achievable in a practical implementation. In practice, we have found the complexity of well-optimized code to be about two times that of the estimates produced by this analysis (see Table 9 of [6]).  Some reasons for the difference between the theoretical estimates and measured quantities are listed below.

- Only fundamental, platform independent operations are included in the complexity analysis.  For example, the arithmetic required for the interpolation results.  Examples of operations not included: those associated with loop and function call overhead as well as initialization operations that, in well organized code, are heavily amortized and therefore represent a small fraction of the overall compute.

- The interpolation subfunction analysis does not include memory copies associated with processing the integer pixel phase.

- The analysis accounts for neither data nor instruction cache misses.

- The analysis assumes all operations in a given subfunction component may be performed in parallel.  Care was taken to divide each subfunction into components for which this assumption is, for the most part, valid.

- The analysis does not account for platform dependent data alignment details.

- The analysis does not account for special case code, for example, to handle boundary conditions.

- The analysis focuses on baseline-like coding tools (i.e., tests associated with non-baseline tools such as MBAFF are not counted).

# 3	Common Condition Sequences

Originally, we planned to compare our results with the results in [6].  Consequently, we selected our common condition sequences to align with those in that paper.  The sequences are listed in Table 1. We subsequently decided, based primarily on the fact that the analysis in [6] applied to an early, pre-standard version of H.264, to rework the complexity estimates for the H.264 standard.  We did not, however, change the set of sequences.

Table 1 - Common Condition Sequences

| Sequence | QP | Sequence | QP |
| --- | --- | --- | --- |
| Container, QCIF @ 10 Hz | 23 | Foreman, CIF @ 30 Hz | 26 |
|  | 27 |  | 31 |
| Foreman, QCIF @ 10 Hz | 30 |  | 40 |
|  | 36 | Mobile & Calendar, CIF @ 30 Hz | 28 |
| Silent, QCIF @ 15 Hz | 28 |  | 33 |
|  | 34 |  | 38 |
| Paris, CIF @ 15 Hz | 33 |  |  |
|  | 29 |  |  |
|  | 36 |  |  |

# 4	Results

Detailed results are referenced in the annex of this contribution. A summary of those results is presented in this section.  For the analysis, we chose to map our estimates onto the Nexperia PNX1500 media processor.  The PNX1500 and its close relatives, the TriMedia 1000, 1100 and 1300 are at the heart of several hardware platforms capable of real-time H.264 encoding and decoding. That said, it is a relatively straightforward task to map the data provided in the associated contribution onto other hardware platforms and thereby extend our results to those platforms.

## 4.1	Average Estimated Cycle Reduction

Using our complexity analysis methodology highlighted in section 2 and described in detail in [6], we developed estimated complexity and cycle reductions for H.264 CE as compared to standard H.264.  A summary of our results is shown in Table 2.  Detailed results are presented in the accompanying contribution.

## 4.2	Measured Cycle Reduction

Well-optimized software implemented on a PNX1500 based hardware platform has demonstrated significant cycle reductions for the interpolation and loop filter subfunctions when executing H.264 CE as compared to standard H.264 [7].  The average measured cycle reductions for the set of common condition sequences used in this work are shown in Table 2.

## 4.3	Comparing Estimated and Measured Cycle Reductions

As described earlier, practical implementations of H.264 have overhead not included in the computational complexity estimates developed in this contribution.  Consequently, we would, in general, expect

TceTs¹MceMs

where

Tce = theoretical cycle estimate for H.264 CE

Ts = theoretical cycle estimate for standard H.264

Mce = measured cycle count for H.264 CE

Ms = measured cycle count for standard H.264

Rather, we would expect

Tce+VceTs+Vs=MceMs

where

Vce = cycle overhead associated with H.264 CE

Vs = cycle overhead associated with standard H.264

Further, we expect the overhead to differ between subfunctions.  Two specific contributions to the overhead, instruction and data cache misses were measured during the acquisition of the measured cycle reduction data presented in Table 2.  Without counting the combined cache misses, the H.264 CE interpolation subfunction yields a 30% cycle reduction as compared to H.264.  Likewise, the H.264 CE deblocking filter subfunction yields a 50% cycle reduction.

## 4.4	SIMD Analysis

For the interpolation subfunction, we prepared a second complexity analysis that exploited the Single Instruction Multiple Data (SIMD) parallelism inherent in several media and general-purpose processors (including the PNX1500).  As expected, SIMD provided substantial benefit.  The cycle reductions for standard H.264 for interpolation averaged 52.5% across the common condition sequences while those for H.264 CE averaged 59.3%.  Note that SIMD provided a slightly greater advantage for H.264 CE as compared to standard H.264.  Also note that the estimated cycle reductions for the H.264 CE over the standard H.264 are similar in trend and average for both the SIMD and non-SIMD analyses.

## 4.5	Results Summary

A brief summary of results is presented in Table 2. These results validate those presented in contributions [4], [5] as well as [7]. The average estimated cycle reduction represents the results of our analysis over the set of common condition sequences.  The average measured cycle reduction represents the cycle reductions realized by H.264 CE over H.264 as measured using production quality PNX1500 optimized code on a PNX1500 cycle accurate simulator over the set of common condition sequences. It is important to understand that the cycle reduction values in the table relate only to the subfunction being analyzed. The overall decoder cycle reductions are estimated as follows. In [6], it was estimated that the interpolation and deblocking subfunctions account for approximately 25% and 33%, respectively, of the total decoder computational complexity. Consequently, according to this analysis we estimate that the overall reduction in computational complexity for a software decoder implementation due to the H.264 CE interpolation and deblocking filter subfunctions to be on the order of

0.332  =  0.542 × 0.25  +  0.595 ×0.33

or 33.2%.  The cycle reductions estimated here represent a subset of the complexity reduction techniques proposed for H.264 CE.  See [7] for a comprehensive list of techniques, along with descriptions and measured cycle reductions.

Table 2 - Cycle Reductions for Selected Subfunctions 
of H.264 CE as Compared to Standard H.264.

| Subfunction | Average Estimated Cycle Reduction | Average Measured Cycle Reduction | Average Measured Cycle Reduction Excluding Cache Misses |
| --- | --- | --- | --- |
| Interpolation: SIMD | 54.2% | 18.7% | 30% |
| Interpolation: non-SIMD | 47.0% | N/A | N/A |
| Deblocking Filter | 59.5% | 42.4% | 50% |

# Acknowledgements

The author would like to thank Dr. Arild Fuldseth for providing frequency count and measured cycle reduction data over the common condition set of sequences, proof reading early drafts of this contribution and for his valuable suggestions throughout the preparation of this contribution. In addition, the author would like to thank CoVi Technologies for their support

# References

[1]	ITU-T Recommendation H.264, Advanced video coding for generic audiovisual services, 7 May 2004.

[2]	Simplified Fractional Pixel Interpolation, G. Bjontegaard and A. Fuldseth, VCEG-W09. Presented in Redmond WA, USA, July 19-23, 2004.

[3]	Simplified Chroma Deblocking Filter, G. Bjontegaard and A. Fuldseth, VCEG-W10. Presented in Redmond WA, USA, July 19-23, 2004.

[4]	Simplified Luma Deblocking Filter, G. Bjontegaard and A. Fuldseth, VCEG-X04. Presented in Palma, Mallorca, October 17-22, 2004.

[5]	Simplified Chroma Interpolation Filter, G. Bjontegaard and A. Fuldseth, VCEG-Z08. Presented in Busan, Korea, April 16-22, 2005.

[6]	H.264/AVC Baseline Profile Decoder Complexity Analysis, M. Horowitz, A. Joch, F. Kossentini, A. Hallapuro, IEEE Transaction On Circuits and Systems For Video Technology, Vol. 13, No. 7, July 2003.

[7]	Simulation results: Tandberg Computational Efficiency tools, G. Bjontegaard and A. Fuldseth, VCEG-Z10. Presented in Busan, Korea, April 16-22, 2005.

Electronic Attachment:

http://www.itu.int/md/dologin_md.asp?lang=e&id=T05-SG16-050726-D-0140!A1!XLS-E&type=mitems

# Annex A

The embedded spreadsheet below contains detailed results from the complexity analysis described in this contribution (also as an electronic attachment:
http://www.itu.int/md/dologin_md.asp?lang=e&id=T05-SG16-050726-D-0140!A1!XLS-E&type=mitems )