| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) Bangkok, 16 January, 2006 | Document VCEG-AB08 Filename: VCEG- AB08.doc Generated: 10 January, 06 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Hongbo Zhu, Congxiu Wang, Arcsoft Shanghai Co. Ltd. 333 Chengdu North Road, Shanghai, China | Tel: Fax: Email: | +86 021 52981600-1876 hzhu@arcsoft.com.cn cxwang@arcsoft.com.cn |
| Title: | A new arithmetic coding using byte-based renormalization and table-based interval division |  |  |
| Purpose: | Proposal. |  |  |

# Background

In the CABAC[2] of the H.264, all syntax elements are encoded by the M-coder[1]. In the proposal, a new arithmetic coder is proposed. It has the following advantages:

- Higher-precision interval-subdivision and lower storage

- Byte-based renormalization to ease kinds of software and hardware implementations.

- Table-based interval division similar to the M-coder

- Scaled probability estimation similar to the M-coder

- Byte-based output in the encoder and byte-based input in the decoder instead the bit-based input and output in the M-coder

# Probability estimation

Totally 40 LPS probability states are used in the coder and they are realize as a markov chain, which allows a table-driven implementation of probability estimation. We recursively define the following set of representative LPS probability states Pp0,p1,p2,⋯p38,p39:

p0=0.5pi=80.5×pi−1i=1,2,3,⋯38,39

So we have pi+8=pi×0.5. Let I denotes the current interval, MI(I) donotes the position of the most significant bit of I. We have the following relationships:

I×pi=I×0.5i>>3×pi∧7MII×pi=MII×pi∧7−i>>3MII≥MII×pi∧7≥MII−1

The first property allows us to use much less table storage for the interval subdivision, the second and the third relationships allow us to estimate the position of the most significant bit of the current interval without loop.

| State | Probability | Next_State_LPS | Next_State_MPS | Switch_MPS |
| --- | --- | --- | --- | --- |
| 0 | 0.50000 | 0 | 1 | 1 |
| 1 | 0.45850 | 0 | 2 | 0 |
| 2 | 0.42045 | 1 | 3 | 0 |
| 3 | 0.38555 | 2 | 4 | 0 |
| 4 | 0.35355 | 2 | 5 | 0 |
| 5 | 0.32421 | 3 | 6 | 0 |
| 6 | 0.29730 | 4 | 7 | 0 |
| 7 | 0.27263 | 5 | 8 | 0 |
| 8 | 0.25000 | 5 | 9 | 0 |
| 9 | 0.22925 | 6 | 10 | 0 |
| 10 | 0.21022 | 7 | 11 | 0 |
| 11 | 0.19278 | 8 | 12 | 0 |
| 12 | 0.17678 | 8 | 13 | 0 |
| 13 | 0.16210 | 9 | 14 | 0 |
| 14 | 0.14865 | 10 | 15 | 0 |
| 15 | 0.13631 | 10 | 16 | 0 |
| 16 | 0.12500 | 11 | 17 | 0 |
| 17 | 0.11463 | 11 | 18 | 0 |
| 18 | 0.10511 | 12 | 19 | 0 |
| 19 | 0.09639 | 12 | 20 | 0 |
| 20 | 0.08839 | 13 | 21 | 0 |
| 21 | 0.08105 | 13 | 22 | 0 |
| 22 | 0.07433 | 14 | 23 | 0 |
| 23 | 0.06816 | 14 | 24 | 0 |
| 24 | 0.06250 | 15 | 25 | 0 |
| 25 | 0.05731 | 15 | 26 | 0 |
| 26 | 0.05256 | 15 | 27 | 0 |
| 27 | 0.04819 | 16 | 28 | 0 |
| 28 | 0.04419 | 16 | 29 | 0 |
| 29 | 0.04053 | 16 | 30 | 0 |
| 30 | 0.03716 | 17 | 31 | 0 |
| 31 | 0.03408 | 17 | 32 | 0 |
| 32 | 0.03125 | 17 | 33 | 0 |
| 33 | 0.02866 | 18 | 34 | 0 |
| 34 | 0.02628 | 18 | 35 | 0 |
| 35 | 0.02410 | 18 | 36 | 0 |
| 36 | 0.02210 | 18 | 37 | 0 |
| 37 | 0.02026 | 18 | 38 | 0 |
| 38 | 0.01858 | 19 | 39 | 0 |
| 39 | 0.01704 | 19 | 39 | 0 |

Table 1: Probability states and its transition rules for probability estimation

# Table-based interval division

First the interval I is mapped to a quantized value Q, shown as following: Q=I>>MII−4∧15.

then the updated I is gotten as following (s denotes the current LPS state):

R=RTABQ,s∧7<<MII−8>>s>>3ifLPSI=RelseI=I−R

and update the MI(I) lastly:

ifLPSMII−s>>3+1ifI∧1<<MI(I)==0MI(I)−−

| Q State | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 132 | 140 | 148 | 156 | 164 | 172 | 180 | 188 | 196 | 204 | 212 | 220 | 228 | 236 | 244 | 252 |
| 1 | 121 | 128 | 136 | 143 | 150 | 158 | 165 | 172 | 180 | 187 | 194 | 202 | 209 | 216 | 224 | 231 |
| 2 | 111 | 118 | 124 | 131 | 138 | 145 | 151 | 158 | 165 | 172 | 178 | 185 | 192 | 198 | 205 | 212 |
| 3 | 102 | 108 | 114 | 120 | 126 | 133 | 139 | 145 | 151 | 157 | 163 | 170 | 176 | 182 | 188 | 194 |
| 4 | 93 | 99 | 105 | 110 | 116 | 122 | 127 | 133 | 139 | 144 | 150 | 156 | 161 | 167 | 173 | 178 |
| 5 | 86 | 91 | 96 | 101 | 106 | 112 | 117 | 122 | 127 | 132 | 137 | 143 | 148 | 153 | 158 | 163 |
| 6 | 78 | 83 | 88 | 93 | 98 | 102 | 107 | 112 | 117 | 121 | 126 | 131 | 136 | 140 | 145 | 150 |
| 7 | 72 | 76 | 81 | 85 | 89 | 94 | 98 | 103 | 107 | 111 | 116 | 120 | 124 | 129 | 133 | 137 |

Table 2: RTAB(Q, state)

only states from 0…7 is stored because others can be gotten by shift

# Programs

Data Structure of the encoder:

Unsigned short low=0, range=0xffff;

Int pos=7;            //MI(range)-8

Int outstanding=0;    //used in the renormalization

Unsigned char store;  //store the byte

Encoder: val denotes the input, Pseq denotes the current LPS state

Int temp=(Pseq>>3);

Int sel=(Pseq&7);

Int rLPS=(RTAB[sel][(range>>(pos+4))&15]<<(pos-temp));

Int sLow;

If( val== LPS )

{

Range=rLPS;

pos-=(temp+1);

if((range>>pos)<256)

pos--;

}

Else

{

Range-=rLPS;

sLow=low+rLPS;

if(sLow<low)

{

outByte(store+1); //output a byte equal to store+1

outByte(0, outstanding-1); //output outstanding-1 bytes equal to 0

outstanding=0;

}

low=sLow;

If((range>>pos)<256)

pos--;

}

If(range<256)

{

If( ( (low&255)+range ) > 256 )

{

If((low>>8)==255 )

Outstanding++;

Else

{

If(outstanding)

{

outByte(store);

outByte(0xff, outstanding-1);

}
            outstanding=1;

Store=low>>8;

}

}

Else

{

If(outstanding)

{

outByte(store);

outByte(0xff, outstanding-1);

}

outByte(low>>8);

outstanding=0;

}

Low<<=8;

Range<<=8;

}

Update the probability estimation

Data structure of the decoder:

Unsigned short sub=readByte(2);    //read two bytes from the bitstream

Unsigned short range=0xffff;

Int pos=7;            //MI(range)-8

Decoder: val denotes the output, Pseq denotes the current LPS states

Int temp=(Pseq>>3);

Int sel=(Pseq&7);

Int rLPS=(RTAB[sel][(range>>(pos+4))&15]<<(pos-temp));

If( rLPS>sub )

{

val=LPS;

range=rLPS;

pos-=(temp+1);

if((range>>pos)<256)

pos--;

}

Else

{

val=1-LPS;

range-=rLPS;

sub-=rLPS;

If((range>>pos)<256)

pos--;

}

If(range<256)

{

sub<<=8;

range<<=8;

sub+=ReadByte(1); //read a byte as the low byte of sub

}

Update the probability estimation

# Reference

- Detlev Marpe and Thomas Wiegand, “A Highly Efficient Multiplication-free Binary Arithmetic Coder and its Application in Video Coding”, ICIP, 2003.

- Detlev Marpe, Heiko Schwartz and T. Wiegand, “Context-based Adaptive Binary Arithmetic Coding in the H.264/AVC Video Compression Standard”, IEEE Trans. CSVT, July 2003.