| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 29th Meeting: Klagenfurt, Austria, 17-18 July, 2006 | Document VCEG-AC06 Filename:VCEG-AC06 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Joel Jung Guillaume Laroche Orange - France Telecom R&D 38, rue du Général Leclerc 92794 Issy les Moulineaux, France | Tel: Fax: Email: | +(33)145295366 joelb.jung@orange-ft.com |
| Title: | Competition-Based Scheme for Motion Vector Selection and Coding |  |  |
| Purpose: | Information |  |  |

# Introduction

H.264 achieves a significant compression gain compared to its predecessor H.263. However this overall bitrate reduction comes with an increase of the proportion of the bits dedicated to the motion information. Figure 1 shows the percentage of bits used by motion vectors related to the total bitstream, for 4 CIF sequences and for the baseline profile. Indeed, at low bitrate, motion information represent in average 35% of the total bitstream for a QP equal to 35 (and can reach 50%).

In addition, the future standards may even increase this motion information with more accurate motion models and probably with a simultaneous luminance bitrate reduction. Consequently improved prediction and coding of the motion vectors is needed.

We have therefore focused our attention on the reduction of the motion information cost. In this contribution a competitive spatio-temporal scheme for the prediction of the motion vectors is first introduced, including a modification of the RD criterion. Second, the occurrence of the Skip mode is increased using a competition-based Skip mode. The modifications are implemented and tested into the JM10.0 reference software [2]. At this stage of the study, only baseline profile results are provided.

The remaining of this paper is organized as follows. Section 2 is a short state of the art of motion vector coding; the proposed modifications on the standard scheme are discussed in Section 3. Section 4 shortly comments the impact of the proposed method on the complexity. Finally Section 5 presents simulation results for Baseline in which the average 6.1% compression gain (and up to 20% for complex motions) with equivalent quality to H.264 is reported.

# State of the art

The cost reduction of the motion information has already been largely addressed in the literature. We can distinguish two types of methods: methods based on lossy encoding [4] which are not addressed here, and lossless methods which are more widespread. H.264 predicts its motion vector with a spatial median, based on neighboring blocks. The median is adapted depending on the availability of these neighboring blocks [1].

Temporal redundancies have already been addressed. In [5] a temporal predictor is applied to motion vectors and yields good results for sequences with complex motion fields. However temporal only prediction is not more efficient than spatial only prediction when a representative set of sequences is considered.

In [6] temporal and spatial predictors are used.  In [7] a selection is made at the slice level between spatial or combined spatio-temporal correlation. It is not applied to the coding of the motion vectors for sub-partitions 16x16…4x4, but only for Direct and Skip modes, i.e. to try to avoid sending any motion information. Direct mode with spatial and temporal predictors applied to P frames is also proposed in [10].

Baseline

Profile

CIF

Resolution

JM10.2

Figure 1: Percentage of bits dedicated to the motion in a standard H.264 bitstream.

A few competition-based schemes have also been proposed. They usually select the best predictor from a given set, and send the index of this predictor as side information. In [8] the set is composed of three neighboring motion vectors and these three predictors are exhaustively compared. Nevertheless, in this competition-based scheme, temporal predictors are missing.

We have not seen so far in the literature experimental results with a competition-based scheme for the prediction of the motion vectors with several spatial and temporal predictors.

# Proposed method

This section details the two modifications made on the selection of the motion vector predictor, first to improve the prediction of the motion vectors that need to be transmitted, and second on the Skip mode, to increase the amount of macroblocks that do not require to send any motion information.

## Competition for Inter modes

3.1.1 Predictor set

When dealing with lossless coding of motion vectors, the efficiency is closely related to the predictor performance. A competition-based scheme allows selecting the best predictor from a given set, and implies several possible predictors for one motion vector. We have defined a set P including spatial, temporal and spatio-temporal predictors as depicted in Figure 2.

Figure 2: description of spatial and temporal predictors.

The available spatial predictors are the neighboring motion vectors mva, mvb, mvc, mvd, and the H.264 median predictor mvH:264. We have defined an extended spatial predictor mvspaEXT based on successive conditions: it is equal to the median of mva, mvb and mvc if the 3 vectors are available. Otherwise, it is equal to mva if it is available, otherwise equal to mvb if available, and otherwise mvc. If none is available it is equal to 0. The temporal predictors are the collocated motion vector mvcol (motion vector at the same position in the previous frame), or the mvtf, which is intended to follow the motion of a moving object, as depicted in Figure 3.

Figure 3: description of temporal predictors.

Two other temporal median predictors mvtm5 and mvtm9 have been also tested:

mvtm5=medianmvcol,mvt0,...,mvt3

mvtm9=medianmvcol,mvt0,...,mvt8

Spatio-temporal predictors are combinations of spatial and temporal ones. In particular, mvspt is defined by:

mvspt=medianmvcol,mvcol,mva,mvb,mvc

3.1.2 Choice of the predictor

The competition scheme at the macroblock level implies to send a mode i and a residualεmvi:

εmvi=mv−pi, ∀i∈1,n, pi∈P

where n is the number of predictors of defined set P.  The weight of this new information is significant (in average 3.5% of the bitrate, and 12.5% of the motion information for a configuration with 2 predictors).

The efficiency of the competition-based scheme is therefore related to the trade-off between this additional cost and the gain obtained by a more accurate prediction. The classical rate distortion J=D+λR is slightly modified. For the selection of the predictor

R=Rr+λmRm+λmvRmv

where Rr is the rate for block residue, Rm the rate of the coding mode, Rmv the rate of motion vector residue,  λmand λmv are weighting factors, is replaced by

R=Rr+λmRm+λmvRmvmm

where Rmvmm is equal to : Rmvmm=minςεmvi+λiςii=1..n, with ςx is the computed cost of the data x in the bitstream, and λia weighting factor.

## Competition for the Skip mode

Our objective is to increase as much as possible the occurrence of the Skip mode. We introduce a competition scheme: instead of having one single predictor (H.264 median) as in the standard, a set of predictors is used (close to [10]). From each predictor results a distortion. The predictor that minimizes this distortion is selected as the predictor for the Skip mode. If the Skip mode is selected as the best mode (compared to Intra, Inter, …) the index of the predictor is sent.

## Coding of the mode

In our implementation, the mode is encoded only if necessary, i.e. if all the predictions (for each predictor of the set) are not equal.

## Adaptation of the set of predictors

Several experiments have been performed with different kind and number of predictors, either for the motion vector competition, or for the Skip competition. We have presented the configuration that provides the best result on average on the test set. However it is obvious that some other configurations perform better on particular sequences or encoding conditions (frame rate, partition size, etc.)

Consequently, for taking maximum benefit of a competition scheme, it is necessary that:

- The set of predictors (number and type) must remain an encoder choice.

- Some predictors (those who appears to be the most efficient) need to be standardized, so that any decoder can support them.

In addition, additional benefits for such a scheme can be achieved if the set of predictors can be modified on the fly by the encoder, and the latter is able to signal to the decoder this modification of the predictor set.

# Impact on complexity

This section is not intended to provide a complexity study, yet to give an idea of the impact of the proposal on the memory and the complexity.

Memory impact:

The motion information of the reference frames need to be stored. This includes the motion vectors and the corresponding reference frame index. It is to be noticed that these data already need to be stored for the Main/High profiles to support the temporal direct mode.

Complexity impact:

At the encoder side, for each candidate motion vector, the following components need to be computed:

- A residual value εmvi=mv−pi for each predictor of the test set (not only for the spatial H.264 median).

- The cost of this residual value (non normative – an Exp-Golomb lookup table in the JM).

- The cost of the predictor index.

- The equality of the predictions, to avoid sending the mode if possible.

- The distortion of the 16x16 macroblock for each predictor (for the skip mode).

At the decoder side, only the equality of the predictions needs to be checked.

From our point of view, the resulting complexity increase at the decoder is negligible. At the encoder, it remains slight, even if the computation of the distortions (SAD for the Skip mode) can not be neglected.

# Experimental results

## Test conditions

Simulations were performed on the JM10.0 H.264 reference software [2] in which all normative tools and efficient non-normative encoding decisions are implemented [3]. At this stage of the study, only Baseline profile results are provided. Most of the recommendations made in the common conditions [11] are respected, except:

- 1 reference frame is used instead of 4.

- 100 frames are used.

- QP 27, 32, 37 and 40 are selected. The proposed modification is intended for the lowest bitrates.

The test set is composed of CIF, SD and 720p sequences with various contents and motions. All results in this section are given in percentage of bitrate saving computed with the Bjontegaard metric [9], which computes average PSNR difference between RD-curves.

## Results with 2 predictors

The results of the proposed competition scheme depend on the number and type of predictors. We have thus made several experiments to find the best configuration.

Our experiments made with either one, two and four predictors for motion vector competition combined with one, two and four predictors for Skip mode competition, show that the best compromise is obtained with two predictors for motion vectors (H.264 median + Collocated vector) and two predictors for Skip mode (H.264 median + mva). Obviously, the reduction of the motion vector bitrate (Rmv) is increased when using four modes but the compromise with the mode coding (Rmv/mm) leads to slightly worse results.

Concerning the choice of the predictor types, our experiments show that the combination of one spatial predictor and one temporal predictor generally gives better results than two spatial or two temporal predictors for the motion vector competition.

For the Skip mode competition, the combination of two spatial predictors gives better results than one spatial and one temporal in a majority of sequences, especially for sequences with complex and fast motion. Of course for some sequences different association of predictors are more efficient.

| QP | Spatial predictor (H.264 median) | Temporal predictor (Collocated) | Predictions equal (same value) |
| --- | --- | --- | --- |
| 27 | 51% | 39% | 10% |
| 32 | 48% | 42% | 10% |
| 37 | 43% | 46% | 11% |
| 40 | 38% | 49% | 13% |
| Average | 45% | 44% | 11% |

Table 1: selection repartition between the spatial and the temporal predictor.

For motion vector competition the analysis of the selection repartition between the spatial or temporal prediction mode is depicted in Table 1. It shows that on average on the test set, the temporal predictor is selected 44% of the time. Given that the selection results from a RD choice, this result confirms that the temporal predictors are useful. Note that these values exclude the cases where both predictors provide the same value, which represents in average 11%. As an interesting feature, the percentage of selection of the temporal predictor increases when the QP increases. The percentage evolves between 26% and 60% depending on the type of the sequence.

The average increase of the number of macroblocks encoded with the Skip mode is 8.5%. This increase is correlated with the QP and with the sequence type. Sequences with static background as Bbc-news, Modo, Ice and Mobile take reduced benefit from this modification of the Skip which includes a temporal predictor. For these sequences the competition scheme is more effective if a second spatial predictor is in the set.

It is interesting to note the individual gain related to each modification. The average bitrate gain resulting from the competition-based Skip mode only is 2.7% in average. The average bitrate gain resulting from the only modification of motion vector coding have is 3.6% in average. Depending on the sequence content, the proportion varies in favor of one or the other method. It shows that both tools are useful.

Global results are given in Table 2. The average bitrate gain is 6.1%, and can reach 45% for sequence Raven at QP 40 (to be precise, a simultaneous gain of 39% in bitrate in addition to a gain of 0.27dB).

It can be noticed that the proposed method offers a compression gain for all the sequences we have tested so far. We have compression gain on sequences with simple or no motion (e.g., videoconferencing sequences) as Paris and Silent. These gains remain however low.

| CIF@30Hz |  | SD@30Hz |  | 720p@60Hz |  |
| --- | --- | --- | --- | --- | --- |
| Bbc_news | 5.0% | City | 4.6% | Raven | 20.1% |
| City | 5.5% | Crew | 4.3% | City | 9.5% |
| Crew | 3.8% | Ice | 5.5% | Bigships | 10.0% |
| Foreman | 7.1% | Soccer (60Hz) | 6.4% | Stockholm | 10.3% |
| Ice | 5.6% |  |  | Night | 3.1% |
| Mobile | 1.8% |  |  |  |  |
| Modo | 5.3% |  |  |  |  |
| Paris | 2.2% |  |  |  |  |
| Silent | 3.1% |  |  |  |  |
| Soccer | 3.4% |  |  |  |  |

Table 2: bitrate saving with the proposed method.

For sequences with fast or complex motions, the compression gain is higher and sequences exhibiting global and constant motion, combined with a high level of spatial details take full advantage of the temporal prediction. The bitrate reduction is also closely related to the QP value. At low bitrate, the motion information tends to become a significant part of the total bitstream, so its reduction leads to the highest improvements. Finally the bitrate reduction is clearly related to the frame rate. Effectively the bitrate saving for Foreman moves from 4.1% at 15Hz to 7.1% at 30Hz. This is explained by the temporal distance between motion vector fields.

# Conclusion

In this contribution a competition-based scheme for the prediction of the motion vectors is proposed. The predictors are optimally selected via a rate-distortion criterion that considers the cost of the residual of the motion vector and the cost of the prediction mode. Temporal and spatial predictors are added to the motion vector predictor standard to take benefit of both temporal and spatial correlations in motion vector fields. A competition-based Skip mode is also proposed to increase the amount of macroblocks encoded with this mode, and so avoid sending motion information. These two techniques are implemented in the JM10.0 reference software for the Baseline profile. They provide a systematic bitrate reduction (whatever the sequence and the QP). The average bitrate saving is 6.1% and reaches 20%. It also appears that an on the fly adaptation of the set of predictors made at the encoder side, based on sequence characteristics and encoding configuration can bring additional improvements.

As a next step, implementation for the Main/High profile will be done.

# IPR statement

Orange France Telecom R&D may have IPR connected to the proposed method and based on reciprocity Orange – France Telecom R&D is prepared to grant a license on reasonable and non discriminatory terms.

# References

[1] ITU-T. Recommendation H.264 and ISO/IEC 14496-10 AVC, “Advanced video coding for generic audiovisual services,” version 3: 2005.

[2] H.264 software coordination, K. Suehring, http://iphome.hhi.de/suehring/tml/.

[3] K. Lim, G. Sullivan, and T. Wiegand, Text Description of JM Reference Encoding Methods and Decoding Concealment Methods, JVT-N046 contribution, Hong-Kong, Jan. 2005.

[4] L. Da Silva Cruz and J. Woods, “Adaptive motion vector quantization for video coding,” vol. 2, pp. 867–870, Oct. 2000.

[5] J. Yeh, M. Vetterli, and M. Khansari, “Motion compensation of motion vectors,” vol. 1, pp. 574–577, Oct. 1995.

[6] M. Chen and J. Willson, A.N., “A spatial and temporal motion vector coding algorithm for low-bit-rate video coding,” vol. 2, pp. 791–794, Oct. 1997.

[7] A. Tourapis, F.Wu, and S. Li, “Direct mode for bipredictive slices in the h.264 standard,” IEEE Trans. on CVST, vol. 15, no. 1, Jan. 2005.

[8] S. Deuk Kim and J. Beom Ra, “An efficient motion vector coding scheme based on minimum bitrate prediction,” IEEE Trans. on Image. Proc, vol. 8, no. 8, pp. 1117–1120, Aug. 1999.

[9]  G. Bjontegaard, “Calculation of average PSNR differences between RD-curves”, VCEG Contribution VCEG-M33, Austin, April 2001.

[10] A. Tourapis, F. Wu, S. Li, « Direct Prediction for Predictive and Bidirectionally Predictive frames in Video Coding », JVT Contribution JVT-C128, Fairfax, May 2002.

[11] TK Tan, G. Sullivan, T. Wedi, “Recommended Simulation Conditions for Coding Efficiency Experiments”, VCEG Contribution VCEG-AA10, Nice, October 2005.