| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 45th Meeting: Meeting: Stockholm, Sweden, July 11-20, 2012 | Document VCEG-AS12 Filename: VCEG-AS12.doc |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Tomoo Yamakage (Toshiba) 1, Komukai-Toshiba-cho, Saiwai-ku Kawasaki, 212-8582, Japan In Suk Chong (Qualcomm) 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Fax: Email: | +81-44-549-2152 tomoo.yamakage@toshiba.co.jp +1-858-845-5477 ichong@qualcomm.com |
| Title: | Creation of a new profile including ALF in the 1st version of HEVC standard |  |  |
| Purpose: | Proposal |  |  |

_____________________________

1	Introduction

Adaptive loop filtering (ALF) was adopted in TMuC (Test Model under Consideration) and it has been part of WD (Working Draft) and HM (HEVC Test Model) from the beginning. ALF has been studied by JCT-VC and making progress in terms of coding efficiency improvement and complexity reduction. This contribution proposes to create a new profile that includes ALF.

2	Proposal

At the 9th JCT-VC meeting in Geneva and the 10th JCT-VC meeting in Stockholm, ALF has been significantly simplified for both hardware and software implementation[1][2] as well as enhanced by non-normative means[3]. Objective gain of ALF is not negligible (-3.6%/-2.6%/-5.6% (RA/LB/LP) on average of HD or higher resolution sequences in common test condition)[1]. Therefore, ALF is regarded as a coding efficiency enhancement tool for HD or higher resolution sequences. Since HD or higher resolution sequences would require some level of parallel processing, it is suggested to combine parallel processing tool(s) with ALF for a new profile as the first option.

When comparing Main LB with Main LP with ALF, subjective picture quality is similar[4] while Main LP with ALF requires less encoder burden and less memory bandwidth, which results in providing the similar subjective picture quality with low power consumption[5]. Therefore, as the second option, it is suggested to create a new profile that supports LP with ALF.

3	References

[1] JCTVC-J0390

[2] JCTVC-J0544

[3] JCTVC-J0048

[4] JCTVC-J0565

[5] JCTVC-J0049

IPR Notification: Toshiba Corporation and Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).