| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 48th Meeting: 25 July – 2 August 2013, Vienna, AT | Document VCEG-AV13 Filename: VCEG-AV13.doc |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Jill Boyce, Ajay Luthra , Yan Ye, Kei Kawamura, Tomoyuki Yamamoto Vidyo, Arris, InterDigital, KDDI, Sharp | Tel: Email: | +1 201 478 6145 jill@vidyo.com |
| Title: | Proposed SHVC hybrid scalable main profile |  |  |
| Purpose: | Proposal |  |  |

_____________________________

This contribution proposes a Hybrid Scalable Main profile for SHVC, which supports an AVC/H.264 base layer.

The SHM 2.0 software for SHVC currently provides support for an AVC base layer, when used in conjunction with a modified version of the JSVM which is available with the SHM software.

In the proposed Hybrid Scalable Main profile for SHVC, avc_base_layer_flag shall be equal to 1. Inter-layer syntax prediction from an AVC base layer is not allowed in the proposed profile. The AVC base layer is proposed to obey the constraints of the AVC Progressive High profile, although is not required to be marked as Progressive High profile, i.e. AVC Main profile or AVC High profile with frame_mbs_only_flag equal to 1 will satisfy the constraint

JCTVC-N0050/ JCT3V-E0037 provides modified specification text of the SHVC WD to support an AVC base layer, including the specification of the proposed profile.  The provided text fits within one additional page.