| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: 19–26 June 2015, Warsaw, Poland | Document VCEG-AZ06 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Qualcomm Incorporated | Tel: Email: | 1-858-6512680 hongbinl@qti.qualcomm.com cjianle@qti.qualcomm.com |
| Title: | Local Illumination Compensation |  |  |
| Authors: | H. Liu, Y. Chen, J. Chen, L. Zhang, M. Karczewicz |  |  |
| Purpose: | Proposal |  |  |

_____________________________

Abstract

In this contribution, Local Illumination Compensation (LIC) is studied on top of the VCEG HM KTA software. For each inter-mode coded coding unit (CU), one bit flag is explicitly signaled or implicitly derived to indicate the usage of LIC. When the flag is true, LIC parameters are implicitly derived for each prediction unit (PU)/sub-PU belonging to the CU, by using neighboring samples of the current CU and the corresponding pixels in the reference picture. It is reported that noticeable bitrate saving can be achieved on sequences with illumination variance. Even for normal sequences such as HEVC CTC sequences, 0.36% and 0.33% bitrate saving can be achieved for random access and low delay configuration on average, respectively.

# Introduction

Weighted prediction aiming at global illumination compensation has been adopted into both H.264/AVC [1] and HEVC [2]. On the other hand, to compensate local illumination variance, block based illumination compensation has been proposed in [3] for HEVC, where the illumination compensation parameters are explicitly signaled at each partition. Meanwhile, a local illumination compensation with implicit parameter derivation [4] has been adopted into 3D-HEVC for inter-view prediction. In this contribution, the local illumination compensation with implicit parameter derivation is proposed.

# Technical description

In the proposed method, Local Illumination Compensation (LIC) is enabled or disabled adaptively for each inter-mode coded coding unit (CU), and it is based on a linear model for illumination changes, using a scaling factor a and an offset b.

Figure 1: Neighboring samples used for deriving IC parameters

When a CU is coded with 2Nx2N merge mode, the LIC flag is copied from neighboring blocks, in a way  similar to  motion information copy in merge mode; otherwise, an LIC flag is signaled for the CU to indicate whether LIC applies or not. When LIC applies for a CU, for each PU/sub-PU belonging to the CU, LIC parameters are derived in a way similar to how it is done in the inter-view illumination compensation in [4], i.e., by using subsampled (2:1 subsampling) neighboring samples of the CU and the corresponding pixels (identified by motion information of the current PU/sub-PU) in the reference picture. An example is illustrated in Figure 1. The IC parameters are derived and applied for each prediction direction separately. A least square error method is employed to derive the parameters a and b based on the abovementioned neighboring samples.

## LIC flag in 2Nx2N merge mode

For each merge candidate, besides motion information (motion vector, prediction direction, reference index etc.), one LIC flag is also generated, following the rules listed below.

- For a spatial merge candidate, its LIC flag is set equal to that of the corresponding spatial neighboring block.

- For temporal merge candidate, its LIC flag is set equal to that of the corresponding temporal co-located block. For bi-prediction case, motion information may be derived from two different temporal co-located blocks. In this case, LIC flag of temporal merge candidate is set to true if any of the two co-located blocks uses LIC and set to false otherwise.

- For combined bi-predictive candidate, it is generated from two spatial/temporal merge candidates, and its LIC flag is set to true if LIC flag of any of the two source candidates is true and set to false otherwise.

- For the tailing zero merge candidates, LIC flag is always set to false.

Meanwhile, LIC flag is considered in the pruning process, and two merge candidates are considered as different if they have different LIC flags.

## Interaction with other tools

Interaction of LIC with Overlapped Block Motion Compensation (OBMC): in OBMC, when performing motion compensation using a neighboring motion information, LIC flag of the current CU (instead of the neighboring block) applies. When LIC flag of the current CU is true, in the motion compensation process for a neighboring motion information, LIC parameters are derived by using neighboring samples of current CU and its corresponding pixels identified by the neighboring motion information.

## Encoder complexity reduction

To reduce the encoder complexity, LIC can be disabled for the entire picture when there is no significant illumination change between a current picture and its reference pictures. To identify this situation, histograms of current picture and every reference picture are calculated at the encoder. If the histogram difference between current picture and every reference picture is smaller than a given threshold, LIC is disabled for the current picture; otherwise, LIC is enabled. Furthermore, LIC is disabled for CU coded with non-2Nx2N partition modes.

# Simulation results

LIC was implemented on top of VCEG HM KTA-1.0 [5] and simulations were conducted following the JCT-VC Common Test Condition (CTC) used during HEVC development [6]. Besides the test sequences used in HEVC CTC, new sequences including five High Dynamic Range (HDR) sequences [7] and five Wide Color Gamut (WCG) sequences [8] are also used in our test.

As shown in Table 1, LIC provides 0.36% and 0.33 % luma BD rate reduction, for Random Access (RA) and Low Delay (LD) configurations for HEVC CTC sequences. Meanwhile, it provides 3.58% and 3.54% luma BD bit rate reduction, for RA and LD configurations for new sequences, as shown in Table 2.

Table 1: Coding performance improvement of LIC over HM KTA-1.0 (HM CTC sequences)

Table 2: Coding performance improvement of LIC over HM KTA-1.0 (HDR and WCG sequences)

# Conclusion

This contribution investigated the coding performance of local illumination compensation based on VCEG HM KTA-1.0 software. The results show that noticeable performance improvement can be achieved.

# References

- Advanced video coding for generic audiovisual services, Rec. ITU-T H.264 and ISO/IEC 14496-10 (MPEG-4 Part 10), May, 2003.

- High Efficiency Video Coding, Rec. ITU-T H.265 and ISO/IEC 23008-2, Jan. 2013.

- A. Fujibayashi, S. Kanumuri, F. Bossen and TK. Tan “TE12: Performance of Partition Based Illumination Compensation (PBIC),” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C041, 3rd Meeting: Guangzhou, CN. 07-15 Oct., 2010.

- H. Liu, J. Jung, J. Sung, J. Jia and S. Yea, “3D-CE2.h: Results of Illumination Compensation for Inter-View Prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCT3V-B0045, 2nd Meeting: Shanghai, CN. 13-19 Oct., 2012.

- VCEG HM KTA-1.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMKTASoftware/tags/HM-14.0-KTA-1.0.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L1100, 12th Meeting: Geneva, CH. 14–23 Jan. 2013.

- Ajay Luthra, Edouard François, Walt Husak, “Call for Evidence (CfE) for HDR and WCG Video Coding”, ISO/IEC JTC 1/SC 29/WG 11 (MPEG) Doc. N15083.

- P. Andrivon, P. Bordes, “AHG14: Wide Color Gamut Test Material Creation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0163, 14th Meeting: Vienna, AT. 25 July–02 Aug. 2013.