| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: 19–26 June 2015, Warsaw, Poland | Document VCEG-AZ08 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: Author(s) or Contact(s): | Microsoft Research Cuiling Lan, Jizheng Xu, Feng Wu | Tel: Email: | culan@microsoft.com jzxu@microsoft.com fengwu@ustc.edu.cn |
| Title: | Enhancement of HEVC using Signal Dependent Transform (SDT) |  |  |
| Purpose: | [e.g., Information or Proposal] |  |  |

_____________________________

Abstract

For video/image coding, residual blocks usually exhibit flexible structure patterns, e.g., directional patterns, cross edge patterns. Transforms that can tackle flexible patterns are expected, especially for large transform blocks. Exploring the internal video similarity provides a practical way to estimate the statistics of a coding block and model the patterns. In this contribution, we propose various block size Karhunen-Loeve Transform (KLT) to exploit the statistics of the coding block on top of the High Efficiency Video Coding (HEVC) standard. From the reconstructed regions, we collect abundant candidate blocks which are highly correlated to the coding block, and use them as the training samples of the KLT. We have designed the variable block size KLTs, ranging from 4x4 to 32x32. Fast KLT algorithm is adopted to make the large block size KLT feasible. Experimental results show the proposed scheme can achieve the BD-rate savings of 7.1%, 3.3%, 2.4% and 2.0% on average for LP-Main10, LB-Main10, RA-Main10 and AI-Main10 cases, compared to the test model HM12.0 of HEVC. Thanks to the introducing of large block size KLT, the BD-rate savings can be up to 23.3%.

- Introduction

The advance of the latest video compression standard High Efficient Video Coding (HEVC) has brought the video coding efficiency up to a new height. In comparison with H.264/MPEG-4 AVC, HEVC can use 50% less bitrate to reach the similar visual quality over the reconstruction video. It has made full use of the block-based coding structure. Flexible coding block sizes and partitions enable it to possess a high adaptability to the coding contents. Different from H.264/MPEG-4 AVC, it allows the use of large block structures, including the large coding unit and large block size transform. Large block size transforms bring high coding efficiency for the high definition contents and smooth contents. Moreover, it has made elaborate designs for the local predictions such as intra directional prediction, motion vector prediction. Many of the improvements were attempting to make full use of the available correlations. A question arises about the future of video coding: from what aspects the coding efficiency could be further improved? It is of interest to explore the possible directions.

Recently, some work attempted to explore external image similarity to boost the coding performance [1]. The high-quality images are reconstructed from a database relying on the decoded down-sampled image and the differential feature vectors of some local descriptors [1]. It achieved large coding gains in comparison with HEVC. However, such external image/video based methods require a large memory for holding the databases. In [2], from the external training sequences, the statistics of prediction residues were learned for the offline KLT training. The residual blocks were offline grouped into different categories based on the directional characteristics. Then, they were taken as the training samples. However, in considering the richness of the statistics of the residues and the inconsistent of the residual characteristics even within the same directional mode, more KLTs for one direction were trained as that in [3]. In [4], several KLTs were trained with rate distortion optimization taken as the criteria to choose the best one. However, using more KLTs will result in more overheads. More importantly, the adaptability to the flexible residual patterns is limited by the KLT number.

We know that the video contents usually present anisotropy. After the motion compensated prediction or intra directional prediction, the residual contents often present gradient (edge) structures, especially along edges or object boundaries. Neighboring pixels exhibit dynamic correlations. Such dynamic correlations are hard to be explored by the commonly used DCT, which is optimal only for the first-order stationary Markov signals. They are hard to be explored by the above referred KLTs whenever the residues present complex gradient patterns, such as the pattern with more than one directions. Furthermore, there is no model which can describe such dynamic correlations and thus no available approximated transform basis functions for them. For the directional patterns, the anisotropic image correlation model can be used to describe the directional correlations [5]. For such model, two one dimensional Discrete Sine Transform (DST) was used as the approximated transform basis function of the optimal KLTs. DST was proposed and applied to 4x4 transform for intra prediction residues coding in HEVC [5]. It brought little improvement over large block size transform. For the general residual patterns, there is still a lack of the efficient transform tools. It is challenging to design transforms being adaptive to the complex anisotropy residual patterns.

To obtain the transforms being adaptive to the different residual patterns, in our previous work, we had proposed the online trained KLTs by exploiting the non-local region similarity from the internal images/videos [6] on the platform of Key technique area (KTA) software [7]. It overcomes the limitations of using several offline trained KLTs [2][3][4]. First, not limited to those directional related KLTs, it can adapt to the local residual patterns. The proposed KLTs was online trained block-wisely based on the estimated statistics of the coding block. For this reason, we also refer it to as Signal Dependent Transform (SDT). Second, it does not need to transmit the overhead for identifying the category of the KLT. The encoder and decoder can find the same training samples. Due to the coding structure constraint of H.264/MEPG-4 AVC, we only investigated that method over 4x4 transform blocks.

From the evolution of HEVC and some other work, we believe that the full exploration of correlations and the improvement on transforms would shed light on the further performance enhancement. In this paper, we propose the using of various block size adaptive KLTs in HEVC to reduce the correlations even for residual blocks with complex structure patterns. We explore the inter-pixel correlations by means of making use of the internal video similarity. For a coding residual block, we first collect a lot of correlated candidate blocks from the internal reconstructed regions, by matching the neighboring pixels or a prediction block. Then we use these candidate blocks as training samples to derive the signal dependent KLT. Due to the high correlations between the coding block and the motion compensated predicted block, and the self-similarity in video, we can catch the residual patterns from the collected candidate blocks. The proposed transform is not only able to explore the directional characteristics, but also can explore the patterns with other gradient structures.

In this contribution, firstly, we propose large block KLTs which favor the merit of handling complex residual patterns. To the best of our knowledge, this is the first successful attempt on improving the large block size transforms on HEVC. Even though DST has been adopted by HEVC [5] for intra modes, it does not works well on large block sizes, such as 8x8 to 32x32. Secondly, we have used fast KLT for large block size transforms, making the time complexity affordable.

The remainder of this contribution is organized as follows. In Section 2, we review the general idea of KLT. In Section 3, the implementation details are described. Fast KLT and the implementation of KLTs in HEVC are discussed. Section 4 shows the experimental results and analysis. We conclude the paper in Section 5.

- General Idea

In our previous work [6], we have proposed signal dependent transform KLT for intra-frame and inter-frame coding in H.264/AVC, which explores the non-local correlations. Considering that there are many similar patches within a frame and across frames, we explore such correlations to enhance coding performance by means of KLT. This trained KLT plays the role of transform which intends to compact the energy more efficiently.

Figure 1. Flowchart of KLT exploring the non-local correlations.

We show the flowchart in Fig.1. For a coding block, we explore the correlations from non-local similar blocks and use them to train the signal dependent transform KLT. For the current coding block indicated by C, at first, we obtain a reference patch R which consists of the reconstructed left-up template tb and the prediction block p of the coding block. Then, we use this reference patch to search for N most similar patches over the reconstructed regions. Finally, we train one-dimensional KLT based on these blocks and prediction block.

In the real coding, the coding block is unknown at the decoder for the collection of similar candidate blocks. Moreover, it is impossible to transmit the KLT bases to the decoder due to the large overhead cost. Thus, we use the prediction block and the reconstructed template to guide the searching of similar blocks instead of using the original block.

However, there are several constraints in our previous work. In our previous work, we explore the non-local correlation and train KLT on 4x4 coding blocks under the framework of MPEG-4/H.264 AVC. We did not made full exploration of the correlations. We only implemented KLT on 4x4 small blocks in considering (i) the high complexity for deriving KLT for large coding block; (ii) different size blocks take different routines in MPEG-4/H.264 AVC which requires respective designs of the KLTs. There is lack of investigation on the performance of large block size KLTs. The advancement of the latest compression standard High Efficiency Video Coding (HEVC) relights the enthusiasm to investigate the potential of the adaptive KLT for performance improvement. On the one hand, HEVC achieves excellent performance and it is hard to get large bitrate savings now. There is high requirement for exploring possible directions towards the next generation standard. On the other hand, the flexible recursive coding structures of HEVC which supports large block sizes provides good platform to validate the efficiency of various size KLTs.

We have designed KLTs for various block sizes, i.e., 4x4, 8x8, 16x16 and 32x32. Details will be presented in Section 3 and Section 4.

- Algorithm Description

## KLT Training

The support of large block coding and the flexible Coding Unit, Prediction Unit and Transform Unit tree structured design in HEVC provides an excellent framework for investigating better algorithms for performance improvement. Rather than restricting the coding unit size to be 16x16 as in MEPG-4 H.264/AVC, the coding unit size can range from 8x8 to 64x64. Correspondingly, transform block size can range from 4x4 to 32x32. Large block size transform can compact the energy more efficiently especially for smooth and high resolution contents. Small block size transform can better adapt the contents.

It is known that Karhunen-Loeve transform (KLT) is the optimal transform from the energy compaction efficiency. By searching over the reconstructed regions, we obtain N blocks xi, i = 1, 2, …, N, which are most similar to the reference patch. Here, xi = (xi1, xi2, …, xiD)T and D indicates the vector dimension which is the transform block size. For an example, for 4x4 coding block, N is 16. We subtract the prediction p from those blocks and obtain the residual blocks as ui, i = 1, 2, …, N, where ui = (xi – p)/N. Then, we take these residual blocks as the training samples with zero mean for the KLT derivation. These N training samples can be represented by U = (u1, u2,…,uN), which is an D×N matrix. We indicate the covariance matrix Σ as

Σ=UUT                                                                   (1)

where the dimension of this covariance matrix is D×D. KLT bases are then the eigenvectors of this covariance matrix. For natural image/video contents, we find the selection of the candidate number N as 100 is enough for the good performance.

## Fast KLT

The computation complexity for the eigenvalue decomposition is O(D3). For 4x4 block with D being 16, the complexity is O(163), which is acceptable. For a large block, the complexity will be very high. For 32x32 block with D being 1024, the time complexity will be 262144 times slower than that for 4x4 block, being intolerable in the coding framework.

In considering this, we utilize a fast algorithm to make the large block size KLT feasible. The dimension of Σ is D×D. However, UTU has a much lower dimension as N×N. We calculate the eigenvectors ϕ of Σ'=UTU, which satisfy the equation as

UTUϕ=ϕΛ                                                              (2)

ϕ indicates the eigenvector matrix while Λ denotes the diagonal matrix with the eigenvalues   being the diagonal elements. Let us multiply both sides of equation (2) by U, we get

UUTUϕ=UϕΛ                                                         (3)

We add brackets to this equation and obtain

(UUT)(Uϕ)=(Uϕ)Λ                                                   (4)

The column vectors of Uϕ are the eigenvectors of UUT with their corresponding eigenvalues being the diagonal elements of matrixes Λ. Let φ=Uϕ. This indicates the eigenvectors of the high dimensional covariance matrix UTU can be obtained by multiplying U with the eigenvectors ϕ which are obtained from the low dimensional covariance matrix UTU. The dimensions of φ and Λ are both D×N. All the other (D-N) eigenvectors of UUT have zero eigenvectors. We can use Schmidt orthogonalization to fill these (D-N) eigenvectors to get D×D eigenvector matrix.

To reduce the complexity for matrix multiplication, we can use the obtained N eigenvectors to perform KLT transform, leaving the remaining (D-N) transform coefficients as zeros. This will not attenuate the performance since the first N projections can cover the most of the signal energy while the bases are trained from samples being highly correlated with the coding block.

## Implementation in HEVC

We have implemented the proposed KLT at block level on the TU in HEVC. To have high adaptability to the image/video contents, the proposed scheme supports the proposed KLT on 4x4, 8x8, 16x16 and 32x32 transform units. Rate-distortion optimization is utilized to determine the best mode among the proposed transform and the DCT/DST transform in HEVC.

To reduce the time complexity, the original transform of HEVC is checked first. When all the quantization coefficients are zeros, i.e., cbf being 0, we will skip the checking of KLT mode and the original transform is taken as the optimal transform. When the flag cbf for indicating whether all the coefficients are zeros is 1, a flag for indicating whether KLT transform is selected or not will be entropy coded.

For inter coding modes, we will not check the KLT transform at all types of PU partitions for the purpose of complexity reduction. We only check KLT transform when the best PU partitions are determined.

- Experimental Results

To evaluate the performance of the proposed KLT, we have tested the performance in the HEVC test model HM12.0. We show the summary of the performance in Table 1. For lowdelay P Main10, the proposed scheme can achieve 7.1% BD-rate savings on average. Results for some sequences under lowdelay P Main10 configuration is shown in Table 2. We can see that the proposed scheme can achieve up to 23.3% BD-rate savings.

Table 1. Performance Summary for Main10 Profile.

Table 2. Experimantal Results for Lowdelay P Main10 and Lowdelay B Main10

| Sequences | Lowdelay P Main10 |  |  | Lowdelay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | Y | U | V | U | V |
| SteamLocomotive | -11.1% | -0.7% | 0.2% | -4.4% | -9.1% | -12.2% |
| BQTerrace | -23.3% | -6.8% | -7.3% | -10.5% | -20.2% | -23.6% |
| BQSquare | -19.4% | -9.8% | -14.1% | -12.5% | -21.7% | -20.3% |
| Johnny | -10.4% | -4.8% | -3.3% | -6.1% | -3.1% | -2.9% |
| BasketballDrill | -7.5% | -6.0% | 0.0% | -0.3% | -1.9% | -1.2% |
| PartyScene | -7.8% | -4.2% | -5.7% | -5.4% | -9.8% | -9.2% |
| SlideShow | -8.0% | -6.3% | -9.2% | -5.6% | -6.9% | -10.3% |

- Conclusion

In this proposal, we propose various block size Karhunen-Loeve Transform (KLT) to exploit the statistics of the coding block based on the High Efficiency Video Coding (HEVC) standard. We designed the variable size KLTs, ranging from 4x4 to 32x32. Fast KLT algorithm is adopted to make the large block size KLT feasible. Experimental results show the proposed scheme can achieve the BD-rate savings of 7.1%, 3.3%, 2.4% and 2.0% on average for LP-Main10, LB-Main10, RA-Main10 and AI-Main10 cases, compared to the test model HM12.0 of HEVC. The BD-rate savings can be up to 23.3%. Even though some fast algorithms have been utilized, the complexity is still high. More investigation on how to reduce the complexity could be performed in future.

- References

- H. Yue, X. Sun, J. Yang, and F. Wu, “Cloud-based image coding for mobile devices – toward thousands to one compression,” IEEE Trans. Multimedia, vol. 15, no. 4, pp. 845–857, 2013.

- Y. Ye and M. Karczewicz, “Improved h. 264 intra coding based on bi-directional intra prediction, directional transform, and adaptive coefﬁcient scanning,” in Proceedings of the international conference on image processing. IEEE, 2008, pp. 2116–2119.

- K. Zhang, S. Lei, and W. Gao, “Enhanced intra prediction and transform for video coding,” in Proceedings of the international conference on image processing. IEEE, 2010, pp. 3381–3384.

- F. Zou, O. C. Au, C. Pang, J. Dai, X. Zhang, and L. Fang, “Rate distortion optimized transforms based on the lloyd-type algorithm for intra block coding,” IEEE J. Sel. Topics Signal Process., vol. 7, no. 6, pp. 1072–1083, 2013.

- C. Yeo, , Y. H. Tan, Z. Li, and S. Rahardja, “Mode-dependent transforms for coding directional intra prediction residuals,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 4, pp. 545–554, 2011.

- C. Lan, J. Xu, G. Shi, and F. Wu, “Exploiting non-local correlation via signal-dependent transform (sdt),” IEEE J. Sel. Topics Signal Process., vol. 5, no. 7, pp. 1298–1308, 2011.

- Key technical area software of the itu-t, version jm11kta2.4. [Online].

- Available: http://iphome.hhi.de/suehring/tml/

________________________