| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: 19–26 June 2015, Warsaw, Poland | Document VCEG-AZ12_r1 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Kei Kawamura, Sei Naito KDDI Corp. (KDDI R&D Laboratories, Inc.) | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Title: | [AHG3] End-to-end mobile video communication on 5G network |  |  |
| Purpose: | Proposal |  |  |

_____________________________

Abstract

This contribution proposes a future application over mobile network. 5G network may include high bandwidth and low latency compared with a typical video bitrate and a video frame duration (1sec/rate). Total delay of both transmission of an access unit and network latency becomes much less than one frame duration. End-to-end mobile video communication like screen and camera sharing will provide synchronization experience because of sub-one frame delay. Two requirements are then derived. One is error resilience functionality and another is an improvement of an intra-coding performance for an equalization of access unit sizes.

- Background

A mobile phone and a tablet employ 1080p and over 1080p display, while a C-MOS camera sensor provides from 1080p to 2160p resolution. High definition and ultra-high definition video can be consumed easily. A camera frame rate also becomes higher like 120fps/150fps. In the future, 5G network may include high bandwidth and low latency characteristics. In this contribution, 5G network covers from equipment to equipment communication without the internet in Figure 1. It means end-to-end communication is realized on the managed mobile network.

Figure 1 5G network scope in this contribution

In AVC and HEVC era, network bandwidth is equaled or a fewer times wide than the video bitrate. Coded picture buffer (CPB) is useful to suppress bitrate varying due to the bit amount differences between an intra-frame and a non-intra frame. A cyclic intra refresh technique can be utilized to smooth the bit amount differences among picture types. Decoding unit information provides an ultra-low-delay decoding mechanism. These tools/techniques assume hypothetical reference decoder models in the specification document.

In future video coding era, network bandwidth is much larger times wide than an average video bitrate. Each access unit can be transmitted under one frame duration. A camera capturing delay is also shorten by a high frame rate processing. Total delay of transmission and access-unit arrival notification will be less than one frame duration at typical frame rate like 30/60 fps.

Here is the comparison the 4G and 5G network for the video transmission. A bit amount of each coded frame is shown in Figure 2. The bit amount of the first frame is much larger than that in the plot because it is coded as intra frame. When maximum bandwidth is roughly 1Mbps, some frame needs 1/fps duration shown in left of Figure 2. When maximum bandwidth is roughly 20Mbps, all frames can be send within 1/fps duration shown in right of Figure 2. The bandwidth value is a just example.

Figure 2 Generated bit amount of each frame by an encoder

Figure 3 Transmission comparison between 4G (left) and 5G (right).

A mobile phone utilizes a hardware encoder, which supports high levels due to the high resolution camera. When a lower resolution video is inputted, an encoding delay is much smaller than one frame duration.

Error resilience tools are studies so far. Some of them are standardized in the existing video coding standard. Flexible macroblock order (FMO), arbitrary slice order (ASO), and redundant slice (RS) are well-known tools. Although such tools are useful under a varying wireless environment, they are not supported by the most encoders and decoders. One reason is a hardware complexity and another is an improvement of network reliability. For example, 4G long term evolution (4G LTE) employs both hybrid automatic repeat request (HARQ) and ARQ on layer 1 and 2, respectively. UDP/IP datagram is much reliable than the past generation.

Application

Two most commonly used applications is screen and camera sharing like video call and remote desktop. For the mobile terminal application, these applications will be enhanced. The screen sharing is enhanced to the joint web browsing and video watching. The camera sharing is enhanced to the event/scene sharing by a rear-camera.

One of the end-to-end video communication system (one way model) is shown in Figure 4. Backward channel for feedback loop is established in the system. Each frame is capsuled by RTP container. When whole bits of one frame is arrived, an arrival message is sent from receiver to sender.

Figure 4 An example of end-to-end video communication system

To avoid a decoding error propagation due to a transmission error, a feedback loop mechanism is reasonable. Thanks to the 5G network characteristics shown in background, a frame by frame feedback channel can be realized. Let one frame duration is defined as T. As an example, an encoding delay, transmission delay, and round trip latency are 0.5T, 0.3T, and 0.05T. Pipeline stages are illustrated in Figure 4. It is emphasized that one frame transmission in sub-frame duration and an immediate message arrival are realized by wide bandwidth and low-latency on 5G network.

Figure 4 Example of pipeline stages with and without error.

When an encoder receives no loss message on a watch dog timer module, nothing is occured and a normal encoding starts after new frame is inputted. When the encoder does not recieve any message on the watch dog timer module, the encoder decides a current frame may have error and changes the reference picture set (RPS) such that the current frame is not used for the next frame encoding. It is noted that RPS control is one of the example for an error resilience. An error concealment method is implementation issue on a decoder side. The decoding error propagetion can be prevented only to one frame by this feedback loop.

Although these system may be realized by existing coding standards like H.264 and H.265, a study on a dynamic reference-picture-set control is encouraged. New test condition such as low-delay P/B with frame loss is required.

Conclusion

In this contribution, we show the characteristics of 5G mobile network and their influence on a video transmission. Two requirements are derived from the above application. One is that frame-level error-resilience functionality is required to provide an efficient error-propagation prevention on the encoder side. Another is that reduction of intra-frame bit amount is required to realize the frame by frame feedback. Higher resolution requires higher improvement.

________________________