| ITUPublications |  | International Telecommunication Union |
| --- | --- | --- |
| Recommendations |  | Standardization Sector |
| Recommendation |  |
| ITU-T T.266.1 (V1) (XX/20XX) |  |
|  | SERIES T: Terminals for telematic services XXX – XXX |  |
|  | Conformance specification for ITU-T T.261 biomedical and general waveform signal coding |  |

ITU-T T-SERIES RECOMMENDATIONS

Terminals for telematic services

| CHARACTERISTICS OF VISUAL TELEPHONE SYSTEMS | H.100-H.199 |
| --- | --- |
| INFRASTRUCTURE OF AUDIOVISUAL SERVICES | H.200-H.499 |
| General | H.200-H.219 |
| Transmission multiplexing and synchronization | H.220-H.229 |
| Systems aspects | H.230-H.239 |
| Communication procedures | H.240-H.259 |
| Coding of moving video | H.260-H.279 |
| Related systems aspects | H.280-H.299 |
| Systems and terminal equipment for audiovisual services | H.300-H.349 |
| Directory services architecture for audiovisual and multimedia services | H.350-H.359 |
| Quality of service architecture for audiovisual and multimedia services | H.360-H.369 |
| Telepresence, immersive environments, virtual and extended reality | H.420-H.439 |
| Supplementary services for multimedia | H.450-H.499 |
| MOBILITY AND COLLABORATION PROCEDURES | H.500-H.549 |
| VEHICULAR GATEWAYS AND INTELLIGENT TRANSPORTATION SYSTEMS (ITS) | H.550-H.599 |
| BROADBAND, TRIPLE-PLAY AND ADVANCED MULTIMEDIA SERVICES | H.600-H.699 |
| IPTV MULTIMEDIA SERVICES AND APPLICATIONS FOR IPTV | H.700-H.799 |
| E-HEALTH MULTIMEDIA SYSTEMS, SERVICES AND APPLICATIONS | H.800-H.899 |

For further details, please refer to the list of ITU-T Recommendations.

Recommendation ITU-T T.266.1 (V1) Conformance specification for ITU-T T.261 biomedical and general waveform signal coding

Summary Recommendation ITU-T T.261.1 "Conformance specification for ITU-T T.261 biomedical and general waveform signal coding" specifies tests for (non-exhaustive) testing to verify whether bitstreams and decoders meet the normative requirements specified in Rec. ITU-T T.261 ¦ ISO/IEC 23003-8 Biomedical and general waveform signal coding. The bitstreams provided with this document correspond to the XX/20XX (V1) edition of Rec. ITU-T T.261. This Recommendation was developed collaboratively with ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information, and corresponds with ISO/IEC 23003-XX as technically aligned twin text. The conformance bitstreams needed for this Recommendation are available at: https://www.itu.int/myworkspace/t-signals/vectors?val=37.

History Edition; Recommendation; Approval; Study Group; Unique ID. 1.0; ITU-T T.261.1; 2027-XX-XX; 21; 11.1002/1000/14969.

Keywords Conformance, T.261, testing, biomedical and general waveform signal coding.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications, and information and communication technologies (ICTs). The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure, e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met. The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents/software copyrights, which may be required to implement this Recommendation. However, implementers are cautioned that this may not represent the latest information and are therefore strongly urged to consult the appropriate ITU-T databases available via the ITU-T website at https://www.itu.int/ITU-T/ipr/.

ITU 20XX

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1 Scope 1 2 Normative references 1 2.1 General 1 2.2 Identical Recommendations ¦ International Standards 1 2.3 Paired Recommendations ¦ International Standards equivalent in technical content 1 3 Definitions 1 4 Abbreviations 1 5 Conventions 2 6 Conformance testing for ITU-T T.261 ¦ ISO/IEC 23003-8 2 6.1 Introduction 2 6.2 Bitstream conformance 2 6.3 Decoder conformance 2 6.4 Procedure to test bitstreams 2 6.5 Procedure to test decoder conformance 2 6.5.1 Conformance bitstreams 2 6.5.2 Contents of the bitstream file 2 6.5.3 Requirements on output of the decoding process 3 6.5.4 Static tests for output order conformance 3 6.5.6 Decoder conformance test for a particular profile and level 3 6.6 Specification of the test bitstreams 3 6.6.1 General 3 6.6.2 Test bitstreams – Coding tools for Main profile 3 6.7 Conformance test suites for Rec. ITU-T T.261 ¦ ISO/IEC 23003-8 4 6.7.1 Bitstreams for Main profile 4

Recommendation ITU-T T.261.1 (V1)

Conformance specification for ITU-T T.261 biomedical and general waveform signal coding

# Scope

This Recommendation | International Standard1 specifies a set of tests and procedures designed to indicate whether encoders or decoders meet the normative requirements specified in Rec. ITU-T T.261 | ISO/IEC 23003-8.

# Normative references

## General

The following Recommendations and International Standards contain provisions which, through reference in this text, constitute provisions of this Recommendation | International Standard. At the time of publication, the editions indicated were valid. All Recommendations and Standards are subject to revision, and parties to agreements based on this Recommendation | International Standard are encouraged to investigate the possibility of applying the most recent edition of the Recommendations and Standards listed below. Members of IEC and ISO maintain registers of currently valid International Standards. The Telecommunication Standardization Bureau of the ITU maintains a list of currently valid ITU-T Recommendations.

## Identical Recommendations | International Standards

–	None.

## Paired Recommendations | International Standards equivalent in technical content

–	Recommendation ITU-T T.261 (in force), Biomedical and general waveform signal coding.

ISO/IEC 23003-8:in force, Information technology – MPEG audio technologies– Part 8: Biomedical and general waveform signal coding.

–	Recommendation ITU-T T.261.2 (in force), Reference software for ITU-T T.266 Biomedical and general waveform signal coding.

ISO/IEC 23003-XX:in force, Information technology – MPEG audio technologies – Part XX: Reference software for biomedical and general waveform signal coding.

# Definitions

For the purposes of this Recommendation | International Standard, the terms, definitions, abbreviations and symbols specified in Rec. ITU-T T.261 | ISO/IEC 23003-8 (particularly in clause 3) apply. The following modified terms and additional terms are provided for purposes herein:

3.1	bitstream: A Rec. ITU-T T.261 | ISO/IEC 23003-8 waveform bitstream.

3.2	decoder: A Rec. ITU-T T.261 | ISO/IEC 23003-8 waveform decoder, i.e., an embodiment of the decoding process specified by Rec. ITU-T T.261 | ISO/IEC 23003-8. The decoder does not include the display process, which is outside the scope of this Recommendation | International Standard.

3.3	encoder: An embodiment of a process, not specified in this Recommendation | International Standard (except in regard to identification of the reference software encoder), that produces a bitstream.

3.4	reference software decoder: The software decoder provided in Rec. ITU-T T.261.2 | ISO/IEC 23003-XX.

3.5	reference software encoder: The software encoder provided in Rec. ITU-T T.261.2 | ISO/IEC 23003-XX.

# Abbreviations

For the purposes of this Recommendation | International Standard, relevant abbreviations and acronyms specified in clause 4 of Rec. ITU-T T.261 | ISO/IEC 23003-8 and the following abbreviations apply.

XXX	XXX XXX XXX XXX

# Conventions

For the purposes of this Recommendation | International Standard, relevant conventions are specified in clause 5 of Rec. ITU-T T.261 | ISO/IEC 23003-8.

# Conformance testing for ITU-T T.261 | ISO/IEC 23003-8

## Introduction

The following clauses specify normative tests for verifying conformance of waveform bitstreams as well as decoders. These normative tests make use of the reference software decoder specified in Rec. ITU-T T.261.2 | ISO/IEC 23003-XX, and the normative tests for decoder conformance also make use of test data (bitstream test suites) provided as an electronic annex to this Recommendation | International Standard.

## Bitstream conformance

Bitstream conformance for Rec. ITU-T T.261 | ISO/IEC 23003-8 is specified by clause C.4 of Rec. ITU-T T.261 | ISO/IEC 23003-8.

## Decoder conformance

Decoder conformance for Rec. ITU-T T.261 | ISO/IEC 23003-8 is specified by clause C.5 of Rec. ITU-T T.261 | ISO/IEC 23003-8.

## Procedure to test bitstreams

A bitstream that is claimed to conform to Rec. ITU-T T.261 | ISO/IEC 23003-8 shall pass the following normative test. This test should not be applied to bitstreams that are known to contain errors introduced by transmission, as such errors are highly likely to result in bitstreams that lack conformance to Rec. ITU-T T.261 | ISO/IEC 23003-8.

The bitstream under test shall be decoded by processing it with the reference software decoder. When processed by the reference software decoder, the bitstream shall not cause any error or non-conformance messages to be reported by the reference software decoder.

Successfully passing this test provides only a strong presumption that the bitstream under test does indeed meet all the requirements specified in Rec. ITU-T T.261 | ISO/IEC 23003-8 that are tested by the reference software decoder.

Additional tests may be necessary to more thoroughly check that the bitstream properly meets all the requirements specified in Rec. ITU-T T.261 | ISO/IEC 23003-8. Such complementary tests may be performed using other bitstream verifiers that perform more complete tests than those implemented by the reference software decoder.

Rec. ITU-T T.261 | ISO/IEC 23003-8 contains several informative recommendations that are not an integral part of that Recommendation | International Standard. When testing a bitstream for conformance, it may also be useful to test whether or not the bitstream follows those recommendations.

To check the correctness of a bitstream, it is necessary to parse the entire bitstream and to extract all the syntax elements and other values derived from those syntactic elements and used by the decoding process specified in Rec. ITU-T H.261 | ISO/IEC 23003-8.

A bitstream verifier may not necessarily perform all stages of the decoding process specified in Rec. ITU-T T.261 | ISO/IEC 23003-8 in order to verify bitstream correctness. Many tests can be performed on syntax elements in a state prior to their use in some processing stages.

## Procedure to test decoder conformance

### Conformance bitstreams

A bitstream that conforms to Rec. ITU-T T.261 | ISO/IEC 23003-8 has values of wsp_profile_indicator and wsp_level_indicator corresponding to a set of specified constraints on a bitstream for which a decoder conforming to the corresponding specified profile and level is required to properly perform the decoding process.

### Contents of the bitstream file

The associated conformance testing bitstreams are included with this Recommendation | International Standard as an electronic attachment. The following information is included in a single zipped file for each such bitstream.

- *.bit – bitstream (provided for all bitstreams)

- *.txt – description (provided for all bitstreams)

- *.md5 – MD5 checksum of the bitstream file (provided for all bitstreams)

- *.raw.md5 – MD5 checksum of the complete decoded RawH2 file (provided for all bitstreams). The decoded file *.raw, for which the MD5 check sum was calculated, was generated in the RawH2 format, i.e., the command line option “--FileFormat=RawH2” was used for the reference software decoder.

- *.log – output log file generated by the reference software decoder (provided for all bitstreams)

- *.cfg – config file used to generate bitstream with reference software encoder (not provided for all bitstreams, not applicable if a different encoder was used)

- *.out – additional output file (not provided for all bitstreams)

### Requirements on output of the decoding process

The output of the decoding process is specified in clause 7 of Rec. ITU-T H.266 | ISO/IEC 23090-3.

For output conformance, it is a requirement that samples of all channels shall be output by a conforming decoder in the specified order and that the values of the decoded samples of the channels that are output shall be (exactly equal to) the values specified in clause 7 of Rec. ITU-T T.261 | ISO/IEC 23003-8.

The display process, which ordinarily follows the output of the decoding process, is outside the scope of this Recommendation | International Standard.

### Static tests for output order conformance

Static tests of a decoder require testing of the samples of the decoded channels that are output from the decoder and can be accomplished when the decoded samples at the output of the decoding process are available. It may not be possible to perform this type of test with a production decoder (due to the lack of an appropriate accessible interface in the design at which to perform the test). In such a case this test should be performed by the manufacturer during the design and development phase. Static tests are used for testing the decoding process.

The pictures that are output by the decoder under test are checked to ensure that the following requirements are fulfilled:

- The decoded channels that are output by the decoder under test shall correspond to those that are output by the reference software decoder.

- The decoded channels that are output by the decoder under test shall be output in the same order as those that are output by the reference software decoder.

- The values of the samples of the decoded samples that are output by the decoder under test shall be identical to those that are output by the reference software decoder.

### 6.5.6	Decoder conformance test for a particular profile and level

In order for a decoder of a particular profile and level to claim output conformance to Rec. ITU-T T.261 | ISO/IEC 23003-8, the decoder shall successfully pass the static test specified in clause 6.5.4 with all bitstreams of the normative test suite specified for testing decoders of this particular profile and level combination.

Tables 1 through XX specify the normative test suites. The profile and level combinations are described in the tables or in the .txt file associated with the bitstream.

## Specification of the test bitstreams

### General

Some characteristics of each bitstream are described in this clause.

### Test bitstreams – Coding tools for Main profile

#### Feature Category (XXX)

##### Test bitstream XXX_A

Specification: XXX

Functional stage: XXX.

Purpose: XXX.

## Conformance test suites for Rec. ITU-T T.261 | ISO/IEC 23003-8

### Bitstreams for Main profile

| Table 1 – Coding tool bitstreams for Main profile |  |  |  |
| --- | --- | --- | --- |
| Categories | Tool description | Feature name | Bitstream features |
| XXX | XXX | XXX | XXX |

| Table 2 – High-level syntax features for bitstreams for Main profile |  |  |
| --- | --- | --- |
| Feature | Feature name | Bitstream features |
| Waveform set parameters | WSP | Multiple waveform set parameters present in bitstream |

SERIES OF ITU-T RECOMMENDATIONS

| Series A | Organization of the work of ITU-T |
| --- | --- |
| Series D | Tariff and accounting principles and international telecommunication/ICT economic and policy issues |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Environment and ICTs, climate change, e-waste, energy efficiency; construction, installation and protection of cables and other elements of outside plant |
| Series M | Telecommunication management, including TMN and network maintenance |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling, and associated measurements and tests |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks, open system communications and security |
| Series Y | Global information infrastructure, Internet protocol aspects, next-generation networks, Internet of Things and smart cities |
| Series Z | Languages and general software aspects for telecommunication systems |

To access the Recommendation, type the URL https://handle.itu.int/ in the address field of your web browser, followed by the Recommendation's unique ID.

1 	This Recommendation | International Standard includes an electronic attachment containing the conformance bitstreams identified within the text. The conformance bitstreams needed for this Recommendation are available in the ITU-T Test Signals Database at the following link: https://www.itu.int/myworkspace/t-signals/vectors?val=37.