ITU Rapporteur for Very	Document LBC-96-139

Low Bitrate Visual Telephony	Dallas, TX  USA - April 1996

Title:	Common Conditions for Core Experiments

Source:		Michael Zeug		Iterated Systems, Inc.
					Tel:	+1 404 264 8094
					Fax:	+1 404 264 8313
					E-mail: mzeug@iterated.com

ITU-T Common Conditions for Core Experiments

Those submitting demonstration material for H.263+ and/or H.263L are strongly encouraged to provide material using the following conditions.  These are considered as a minimum recommended set of sequences, bitrates, frame rates and spatial resolution that each submission demonstration should contain. Submitters are free to provide additional demonstration material as appropriate.

| Bitrate (kbps) | 10 | 24 | 112 |
| --- | --- | --- | --- |
| Spatial Resolution | QCIF | QCIF | CIF |
| Frame Rate (FPS) | 7.5 | 10 | 15 |
| Sequences | Mother and Daughter* Hall Objects | Mother and Daughter* Silent Voice News Foreman | Coastguard Foreman Gary** |

* For “Mother and Daughter” submissions should code 30 seconds of material (first 900 frames).

** For “Gary” submissions should code 30 seconds of material (first 900 frames).

Pre-Processing and Post-Processing

To provide a common starting point, all sequences should be downsampled to CIF or QCIF (as appropriate) by using the filter program developed for MPEG-4.  This would ensure that all submissions begin from the same source material.

Display Conditions

The demonstration material should be presented as a CIF image displayed side-by-side with a CIF image generated by the Test Model.  Images coded at QCIF resolution should be upsampled to CIF (using the MPEG-4 upsampling filter) for display.  These two images should be centered in a 601size display.

Additional Aspects

Additional sequences, bitrates,  and/or frame rates are also welcome.  Skipped frames are acceptable (so long as the sequences are realistically displayed and statistics are provided) and variable frame rate demonstration sequences are welcome.

A paper describing the taped demonstration is expected.  It should also identify any special features of the demonstration material such as frame skipping, degree of jitter, degree of delay, frame rate variability, etc.

Delay

In order to evaluate the delay introduced by submissions and the Test Model, submissions should present information on the delay of the submission relative to the original sequence.  This information could take the form of a video demonstration displaying the original, Test Model, and the proposal presented as if they were displayed by an actual system.  Alternatively, submissions could provide written data indicating the delay experienced by frames within the proposal.  In either case, those that submit proposals which have an affect on delay are strongly encourage to provide information concerning the amount of delay introduced.