ITU-TS SG15	LBC-96-293

Working Party 15/1	November 1996

Expert’s Group For Very Low Bitrate Visual Telephony

Atlanta (11-14)   November 1996

Source:	Telenor R&D

Status:	Proposal

Title:	Changes to the deblocking filter option.

Author:	G. Bjontegaard

# Introduction

It is proposed to change the definition of the deblocking filter in Annex J to get in line with the present definition of MPEG4 VM 4.0 Appendix C - with some small editorial change in the text.

Related to Annex J, the main difference is a simplified criteria for when the filter shall be used.  In addition there is a small modification to the filter taps.

# Modifications relative to the MPEG4 text.

In the MPEG4 text, it is not completely clear which QP should be used.  The following modifications are therefore proposed.

In the second paragraph replace:

"...C and D belong to neighboring block called block2."

with:

"...C and D belong to neighboring block called block2 which is to the right or

below block1."

Replace:

"QP = quantization parameter of the current macroblock"

with

"QP = quantization parameter of the macroblock where pixel C belongs (see the

figure)"

There is a simulation demo to show the effect of the filter taps (1,-3,3,-1)/8   and          (3,-8,8,-3)/16.