ITU-TS SG15	LBC-96-295

Working Party 15/1	November 1996

Expert’s Group For Very Low Bitrate Visual Telephony

Atlanta (11-14)   November 1996

Source:	Telenor R&D

Status:	Proposal.

Title:	Changes to the Advanced INTRA coding mode.

Author:	G. Bjontegaard

# Proposal of a INTRA VLC for coefficient coding.

This document defines a VLC suited for coding of DCT coefficients in INTRA blocks.  The VLC is constructed by using the same codewords as in the present VLC table for coefficient coding in H.263+ (referred to as INTER VLC).  The codewords also have connected to them the same value of LAST as in the previous table.  (This makes it possible to decode the codewords before interpreting the values of RUN and LEVEL).

The values of RUN and LEVEL connected to a codeword are different if used in connection with the INTER or INTRA VLC.

The proposed VLC is given in the table below.  Simulations results with the new INTRA VLC  compared with the old one is given in LBC-96-294.

LBC-96-296 defines a method where the proposed INTRA VLC also may be used for INTER coding.

| INDEX | LAST | RUN | LEVEL | BITS | VLC CODE |  | INDEX | LAST | RUN | LEVEL | BITS | VLC CODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 3 | 10s |  | 58 | 1 | 0 | 1 | 5 | 0111 s |
| 1 | 0 | 1 | 1 | 5 | 1111 s |  | 59 | 1 | 14 | 1 | 10 | 0000 1100 1s |
| 2 | 0 | 3 | 1 | 7 | 0101 01s |  | 60 | 1 | 20 | 1 | 12 | 0000 0000 101s |
| 3 | 0 | 5 | 1 | 8 | 0010 111s |  | 61 | 1 | 1 | 1 | 7 | 0011 11s |
| 4 | 0 | 7 | 1 | 9 | 0001 1111 s |  | 62 | 1 | 19 | 1 | 12 | 0000 0000 100s |
| 5 | 0 | 8 | 1 | 10 | 0001 0010 1s |  | 63 | 1 | 2 | 1 | 7 | 0011 10s |
| 6 | 0 | 9 | 1 | 10 | 0001 0010 0s |  | 64 | 1 | 3 | 1 | 7 | 0011 01s |
| 7 | 0 | 10 | 1 | 11 | 0000 1000 01s |  | 65 | 1 | 0 | 2 | 7 | 0011 00s |
| 8 | 0 | 11 | 1 | 11 | 0000 1000 00s |  | 66 | 1 | 5 | 1 | 8 | 0010 011s |
| 9 | 0 | 4 | 3 | 12 | 0000 0000 111s |  | 67 | 1 | 6 | 1 | 8 | 0010 010s |
| 10 | 0 | 9 | 2 | 12 | 0000 0000 110s |  | 68 | 1 | 4 | 1 | 8 | 0010 001s |
| 11 | 0 | 13 | 1 | 12 | 0000 0100 000s |  | 69 | 1 | 0 | 3 | 8 | 0010 000s |
| 12 | 0 | 0 | 2 | 4 | 110s |  | 70 | 1 | 9 | 1 | 9 | 0001 1010 s |
| 13 | 0 | 1 | 2 | 7 | 0101 00s |  | 71 | 1 | 10 | 1 | 9 | 0001 1001 s |
| 14 | 0 | 1 | 4 | 9 | 0001 1110 s |  | 72 | 1 | 11 | 1 | 9 | 0001 1000 s |
| 15 | 0 | 1 | 5 | 11 | 0000 0011 11s |  | 73 | 1 | 12 | 1 | 9 | 0001 0111 s |
| 16 | 0 | 1 | 6 | 12 | 0000 0100 001s |  | 74 | 1 | 13 | 1 | 9 | 0001 0110 s |
| 17 | 0 | 1 | 7 | 13 | 0000 0101 0000s |  | 75 | 1 | 8 | 1 | 9 | 0001 0101 s |
| 18 | 0 | 0 | 3 | 5 | 1110 s |  | 76 | 1 | 7 | 1 | 9 | 0001 0100 s |
| 19 | 0 | 3 | 2 | 9 | 0001 1101 s |  | 77 | 1 | 0 | 4 | 9 | 0001 0011 s |
| 20 | 0 | 2 | 3 | 11 | 0000 0011 10s |  | 78 | 1 | 17 | 1 | 10 | 0000 1100 0s |
| 21 | 0 | 3 | 4 | 13 | 0000 0101 0001s |  | 79 | 1 | 18 | 1 | 10 | 0000 1011 1s |
| 22 | 0 | 0 | 5 | 6 | 0110 1s |  | 80 | 1 | 16 | 1 | 10 | 0000 1011 0s |
| 23 | 0 | 4 | 2 | 10 | 0001 0001 1s |  | 81 | 1 | 15 | 1 | 10 | 0000 1010 1s |
| 24 | 0 | 3 | 3 | 11 | 0000 0011 01s |  | 82 | 1 | 2 | 2 | 10 | 0000 1010 0s |
| 25 | 0 | 0 | 4 | 6 | 0110 0s |  | 83 | 1 | 1 | 2 | 10 | 0000 1001 1s |
| 26 | 0 | 5 | 2 | 10 | 0001 0001 0s |  | 84 | 1 | 0 | 6 | 10 | 0000 1001 0s |
| 27 | 0 | 5 | 3 | 13 | 0000 0101 0010s |  | 85 | 1 | 0 | 5 | 10 | 0000 1000 1s |
| 28 | 0 | 2 | 1 | 6 | 0101 1s |  | 86 | 1 | 4 | 2 | 11 | 0000 0001 11s |
| 29 | 0 | 6 | 2 | 11 | 0000 0011 00s |  | 87 | 1 | 3 | 2 | 11 | 0000 0001 10s |
| 30 | 0 | 0 | 25 | 13 | 0000 0101 0011s |  | 88 | 1 | 1 | 3 | 11 | 0000 0001 01s |
| 31 | 0 | 4 | 1 | 7 | 0100 11s |  | 89 | 1 | 0 | 7 | 11 | 0000 0001 00s |
| 32 | 0 | 7 | 2 | 11 | 0000 0010 11s |  | 90 | 1 | 2 | 3 | 12 | 0000 0100 100s |

TABLE 12/H.263

VLC table for TCOEF (concluded)

| INDEX | LAST | RUN | LEVEL | BITS | VLC CODE |  | INDEX | LAST | RUN | LEVEL | BITS | VLC CODE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33 | 0 | 0 | 24 | 13 | 0000 0101 0100s |  | 91 | 1 | 1 | 4 | 12 | 0000 0100 101s |
| 34 | 0 | 0 | 8 | 7 | 0100 10s |  | 92 | 1 | 0 | 9 | 12 | 0000 0100 110s |
| 35 | 0 | 8 | 2 | 11 | 0000 0010 10s |  | 93 | 1 | 0 | 8 | 12 | 0000 0100 111s |
| 36 | 0 | 0 | 7 | 7 | 0100 01s |  | 94 | 1 | 21 | 1 | 13 | 0000 0101 1000s |
| 37 | 0 | 2 | 4 | 11 | 0000 0010 01s |  | 95 | 1 | 22 | 1 | 13 | 0000 0101 1001s |
| 38 | 0 | 0 | 6 | 7 | 0100 00s |  | 96 | 1 | 23 | 1 | 13 | 0000 0101 1010s |
| 39 | 0 | 12 | 1 | 11 | 0000 0010 00s |  | 97 | 1 | 7 | 2 | 13 | 0000 0101 1011s |
| 40 | 0 | 0 | 9 | 8 | 0010 110s |  | 98 | 1 | 6 | 2 | 13 | 0000 0101 1100s |
| 41 | 0 | 0 | 23 | 13 | 0000 0101 0101s |  | 99 | 1 | 5 | 2 | 13 | 0000 0101 1101s |
| 42 | 0 | 2 | 2 | 8 | 0010 101s |  | 100 | 1 | 3 | 3 | 13 | 0000 0101 1110s |
| 43 | 0 | 1 | 3 | 8 | 0010 100s |  | 101 | 1 | 0 | 10 | 13 | 0000 0101 1111s |
| 44 | 0 | 6 | 1 | 9 | 0001 1100 s |  | 102 | ESCAPE |  |  | 7 | 0000 011 |
| 45 | 0 | 0 | 10 | 9 | 0001 1011 s |  |  |  |  |  |  |  |
| 46 | 0 | 0 | 12 | 10 | 0001 0000 1s |  |  |  |  |  |  |  |
| 47 | 0 | 0 | 11 | 10 | 0001 0000 0s |  |  |  |  |  |  |  |
| 48 | 0 | 0 | 18 | 10 | 0000 1111 1s |  |  |  |  |  |  |  |
| 49 | 0 | 0 | 17 | 10 | 0000 1111 0s |  |  |  |  |  |  |  |
| 50 | 0 | 0 | 16 | 10 | 0000 1110 1s |  |  |  |  |  |  |  |
| 51 | 0 | 0 | 15 | 10 | 0000 1110 0s |  |  |  |  |  |  |  |
| 52 | 0 | 0 | 14 | 10 | 0000 1101 1s |  |  |  |  |  |  |  |
| 53 | 0 | 0 | 13 | 10 | 0000 1101 0s |  |  |  |  |  |  |  |
| 54 | 0 | 0 | 20 | 12 | 0000 0100 010s |  |  |  |  |  |  |  |
| 55 | 0 | 0 | 19 | 12 | 0000 0100 011s |  |  |  |  |  |  |  |
| 56 | 0 | 0 | 22 | 13 | 0000 0101 0110s |  |  |  |  |  |  |  |
| 57 | 0 | 0 | 21 | 13 | 0000 0101 0111s |  |  |  |  |  |  |  |