ITU Telecommunication Standardization Sector

Expert’s Group on Very Low Bitrate Video Telephony

LBC-97-022

February 1997

Source	: JAPAN

Title	: System Related Issues for H.263+ Annex N

Purpose	: Proposal and Discussion

# Introduction

The usage of Annex N of H.263+ “Error Resilient Mode by Backward Channel Operation” needs the backward channel. How to set up the backward channel is very related to the system recommendations. This contribution describes the following 2 kinds of scenarios to set up the backward channel and required text in H.263+ as well as H.245 to implement them.

- Separate extra logical channel for carrying the backward channel message

- Provision of the multiplex mechanism at the video layer

Both of these 2 approaches do not require any modifications of the existing system recommendations except for H.245 to implement the functionality described in Annex N. We propose to change H.263+ and H.245 so that both of these scenarios can be implemented in the H series recommendations.

# Scenarios

## Separate Logical Channel

In order to set up a separate logical channel for the backward channel message, we need some association mechanism between the forward video channel which delivers the video data and the backward channel which delivers the backward channel message. The association mechanism can be implemented by adding  a new code point, VideoBackChannelData, in the part of Datatype of H.245, which indicates the forward video logical channel number as its parameter.

## Multiplex at Video layer

Some systems do not allow to use an extra logical channel for the video backward message due to the restriction of the usable number of logical channels’ combinations. The multiplex mechanism at the video layer enables to deliver the backward channel message without increasing the number of logical channels in the case of bi-directional video communication although it may degrade the performance a little due to the delay in the multiplex mechanism. The message flow is shown in Figure 1. This scenario can be realized by adding a capability codepoint in H.245 and BSC (Back channel data Start Code) in H.263.

Figure 1

Message Flow of Video Layer Multiplex Scenario

# Proposed text in Annex N

N.5 Channel for backward channel messages

This mode has 2 submodes in terms of the channel for backward channel messages. One is the separate logical channel mode and the other is the videomux mode. The separate logical channel mode is a preferred mode and delivers the backward channel message defined in N.3.2 through the dedicated logical channel. The videomux mode is prepared for the system which cannot set up the separate extra channel for the backward channel messages due to the restriction of the number of channels’ combinations. The videomux mode delivers the backward channel messages through the same logical channel with the forward video data in the opposite direction.

N.5.1 Separate Logical Channel Mode

The separate logical channel mode delivers backward channel messages through a dedicated logical channel opened only for the purpose of the backward channel messages. The association mechanism with the forward channel which delivers video data is provided by external means (for example Recommendation H.245).

N.5.2 VideoMux Mode

The videomux mode delivers the backward channel messages through the same logical channel with the forward video data in the opposite direction. The syntax of the multiplexed bitstream is illustrated in FIGURE ?. The backward channel message shall be inserted at the point of the start code.

FIGUURE ?

Syntax of the multiplexed video bitstream

N.5.2.1 Back Channel Start Code (BSC) (22bits)

BSC is a word of 22 bits. Its value is 0000 0000 0000 0000 1 11110. All backchannel start codes shall be byte aligned

N.5.2.2 Stuffing(BSTUF) (Variable Length)

BSTUF is a codeword of variable length consisting of less than 8 bits. BSTUF may be inserted before an BSC codeword. If BSTUF is present, the last bit of BSTUF shall be the last bit of an octet, so that the start of the BSC codeword is byte aligned. Notice that 0 is used for stuffing within BSTUF.

# Required H.245 Additional Codepoint

DataType		::=CHOICE

{

.

.

.

encryptionData	EncryptionMode,

...,

videoBackChannelData	LogicalChannelNumber,

}

H263VideoCapability	::=SEQUENCE

{

sqcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

qcifMPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cifMPI		INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif4MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

cif16MPI	INTEGER (1..32) OPTIONAL,	-- units 1/29.97 Hz

maxBitRate	INTEGER (1..192400),	-- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

temporalSpatialTradeOffCapability	BOOLEAN,

hrd-B		INTEGER (0..524287) OPTIONAL,	-- units 128 bits

bppMaxKb	INTEGER (0..65535) OPTIONAL,	-- units 1024 bits

...,

slowSqcifMPI	INTEGER (1..3600) OPTIONAL,	-- units seconds/frame

slowQcifMPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCifMPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCif4MPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

slowCif16MPI	INTEGER (1..3600) OPTIONAL, --units seconds/frame

errorCompensation	BOOLEAN,

backChannelErrorRes	BackChannelErrorResCapability OPTIONAL,

}

BackChannelErrorResCapability	::=SEQUENCE

{

sqcifAdditionalFrameMemory	INTEGER (1..256) OPTIONAL,	-- units frame

qcifAdditionalFrameMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cifAdditionalFrameMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cif4AdditionalFrameMemory	INTEGER (1..256) OPTIONAL,	-- units frame

cif16AdditionalFrameMemory	INTEGER (1..256) OPTIONAL,	-- units frame

backChannelMessageType	SEQUENCE

{

ackMessage	BOOLEAN,

nackMessage	BOOLEAN,

ackAndNackMessage	BOOLEAN,

...

}

backChannelMuxType	SEQUENCE

{

separateChannel	BOOLEANL,

videoMux	BOOLEAN,

...

}

}

H263VideoMode	::=SEQUENCE

{

resolution	CHOICE

{

sqcif	NULL,

qcif	NULL,

cif		NULL,

cif4	NULL,

cif16	NULL,

...

},

bitRate	INTEGER (1..19200),	-- units 100 bit/s

unrestrictedVector	BOOLEAN,

arithmeticCoding	BOOLEAN,

advancedPrediction	BOOLEAN,

pbFrames	BOOLEAN,

...,

errorCompensation	BOOLEAN

backChannelErrorRes	BackChannelErrorResMode OPTIONAL,

}

BackChannelErrorResMode	::=SEQUENCE

{

backChannelMessageMode	CHOICE

{

ackMode	NULL,

nackMode	NULL,

ackAndNackMode	NULL,

...

}

backChannelMuxMode	CHIOCE

{

separateChannelMode	NULL,

videoMuxMode	NULL,

...

}

}

# Conclusion

We described 2 scenarios to implement Annex N of H.263+ without modifying system recommendations except for H.245, and proposed additional text of H.263+ and H.245.

Contact Person:	Toshihisa Nakai	Yasuhiro Tomita

Oki Electric Ind. Co., Ltd.	NTT

Phone	+81 6  949 5105	+81 468 59 3749

Fax	+81 6  949 5108 	+81 468 59 2829

E-mail	nakai@kansai.oki.co.jp	tomita@nttvdt.hil.ntt.co.jp