ITU Telecommunication Standardization Sector

Expert’s Group on Very Low Bitrate Video Telephony

LBC-97-023

February 1997

Source	: JAPAN

Title	: Overhead of Backward Messages for H.263+ Annex N

Purpose	: Information

# Introduction

On Annex N “Error Resilient Mode by Backward Channel Operation”, we discussed the overhead of backward signals in the Atlanta meeting. If the decoder returns backward signals at each GOB to renew the reference picture per GOB as described in the draft, the overhead of the backward channel could be serious at very low bit rate coding. At the meeting, it is suggested to study the method that the decoder returns some backward signals together (grouping method) to reduce the overhead.

Then we had the experiment to compare the grouping method with conventional one that the decoder sends a backward channel signal whenever it receives a GOB. From the results of this experiment, we got to the conclusion that the grouping method does not show the significant improvement compared with the conventional one due to the delay of the backward channel signals.

In this contribution, we define the backward channel syntax for grouping method, and show the simulation results and our analysis of the results.

# Consideration Points

In this experiment, we considered the following two points.

(1) In the case of grouping method, the timing of returning signals are delayed. By this delay, renewal of the reference picture can be delayed one frame in the encoder, and the SNR will be degraded.

(2) To compare the amount of overhead, we added the bits of backward signals to the encoder's video buffer after the time of round trip delay. By rate control of the encoder, more bits of backward signal make the SNR of the encoded video signal degraded.

It means that the grouping method has the advantage from the viewpoint of the overhead and the disadvantage from the viewpoint of the degradation caused by the delay.

# Syntax of Backward Signal

Now we define the new structure of backward signal for grouping method.

| BT | RF | TR | GN | GF | NS | SP | RTR | BSTUF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

1           1           8           5            1          5            n        8*m

Fig.1 Structure of Backward Signal

- Backward Message Type (BT) indicates ACK(1) or NACK(0).

- TR and GN indicate the number of the first (or last) signal.

- Grouping Flag (GF) indicates that some signals are grouped or not. When GF is 0 (means not grouped), NS, SP, STUFF are not usable.

- Number of Backward Signal (NS) indicate the number of grouped signals.

- Signal Pattern (SP) indicates the raw of ACK(1) and NACK(0). The number of bits (n) is equal to NS.

- RTRs are added for NACKed GOBs that SP is 0, only when BT indicates NACK. The number of RTR (m) is equal to the number of 0 in SP.

- BSTUF is used for byte align.

Table 1 shows the number of bits for the backward channel signals. (In the case of using H.223 AL2.)

Table 1. The number of bits for backward signals

|  | ACK |  | NACK |  |
| --- | --- | --- | --- | --- |
|  | 3GOB | 9GOB | 3GOB | 9GOB |
| 1SIG | 72 | 216 | 96 | 288 |
| 3SIG | 32 | 96 | 56 | 168 |
| 9SIG | - | 40 | - | 112 |

In this table, "9SIG" and "3SIG" indicates the case of grouped 9 and 3 signals respectively, and "1SIG" indicates the case of not grouped. 3SIG needs half bits of 1SIG. And 9SIG need about 1/3 - 1/5.

# Simulation

## grouping strategy

In this simulation, the grouping strategy is the following.

(1) The decoder groups a series of the same kind of signals.

(2) The maximum number of grouped signals is set to 3 and 9.

(3) When the series of signals are broken, the decoder sends the signals even if the grouped  number is lower than the maximum.

## Test Conditions

Fig.1 - 8 shows the SNR for MaD900 on 24kbps (10Hz). Table 2 shows each test condition.

Table 2. Test Conditions

|  | NEWPRED mode | error pattern | round trip delay |
| --- | --- | --- | --- |
| Fig. 1 | ACK | DECT1 | 300msec |
| Fig. 2 | NACK | DECT1 | 300msec |
| Fig. 3 | ACK | DECT2 | 300msec |
| Fig. 4 | NACK | DECT2 | 300msec |
| Fig. 5 | ACK | DECT1 | 250msec |
| Fig. 6 | NACK | DECT1 | 250msec |
| Fig. 7 | ACK | DECT2 | 250msec |
| Fig. 8 | NACK | DECT2 | 250msec |

## Results

In Fig.1 - 4 (round trip delay = 300msec), 1SIG is a little better than other methods, and 3SIG and 9SIG are almost same.

In Fig.5 - 8 (round trip delay = 250msec), 3SIG is a little better than other methods.

Table 3 shows the average rate of the overhead of backward signals against the forward video bit stream in each condition. By grouping signals, the overhead is definitely reduced.

Table 3. The average rate of the overhead of backward signals

|  | DECT1 |  | DECT2 |  |
| --- | --- | --- | --- | --- |
|  | ACK | NACK | ACK | NACK |
| 1SIG | 5.6% | 6.7% | 8.1% | 1.4% |
| 3SIG | 2.6% | 2.3% | 3.5% | 0.3% |
| 9SIG | 1.3% | 1.7% | 1.5% | 0.3% |

However, the NEWPRED effects are reduced by the delay of grouping signals. Table 4 shows the average distance between coded frame and reference frame in each round trip delay (RTD).

Table 4. The average NEWPRED distance

|  | RTD=300msec | RTD=250msec |
| --- | --- | --- |
| 1SIG | 3 | 3 |
| 3SIG | 4 | 3 |
| 9SIG | 4 | 4 |

As the round trip delay (300msec) is just tree times long of the coding interval (100msec), the encoder of 1SIG can renew reference frames just in time after 3 frames, but the encoder of 3SIG and 9SIG cannot renew reference frames in time after 3 frames. So the SNRs of 3SIG and 9SIG are degraded.

The encoder of 3SIG can renew reference frames in time after 3 frames, when the round trip delay is 250msec. So, in this case, the SNR of only 9SIG is degraded.

These results show that we cannot get the significant improvement by the grouping method due to the delay of renewal of reference frames.

## In the Case of Very Low Bit Rate Coding

When the bit rate is 10kbps, the encoded frame rate becomes down by the rate control in the case of TMN6, and the number of backward signals also decreases. Therefore the overhead of backward signals is not always serious in the low bit rate coding if we take the rate control into account.

# Conclusions

We simulated the overhead of backward channel signals of Annex N, and showed that the grouping method does not improve the performance of the NEWPRED system significantly.

Fig.1 NEWPRED-ACK (24kbps, 10Hz, DECT1, round trip delay = 300msec)

Fig.2 NEWPRED-NACK (24kbps, 10Hz, DECT1, round trip delay = 300msec)

Fig.3 NEWPRED-ACK (24kbps, 10Hz, DECT2, round trip delay = 300msec)

Fig.4 NEWPRED-NACK (24kbps, 10Hz, DECT2, round trip delay = 300msec)

Fig.5 NEWPRED-ACK (24kbps, 10Hz, DECT1, round trip delay = 250msec)

Fig.6 NEWPRED-NACK (24kbps, 10Hz, DECT1, round trip delay = 250msec)

Fig.7 NEWPRED-ACK (24kbps, 10Hz, DECT2, round trip delay = 250msec)

Fig.8 NEWPRED-NACK (24kbps, 10Hz, DECT2, round trip delay = 250msec)

Contact Person:	Toshihisa Nakai and Shigeru Fukunaga

Oki Electric Industry Co., Ltd.

Phone	+81 6  949 5105

Fax	+81 6  949 5108

E-mail	{nakai,fukunaga}@kansai.oki.co.jp