ITU - Telecommunications Standardization Sector	Document: LBC-97-041

STUDY GROUP 16

Nice, 25-28 February 1997  (Rapporteur's Meeting)

Question:		11/16

Source:			PictureTel Corporation

Contact:			Dave Lindbergh							Voice: 	+1 508-623-4351

PictureTel Corporation					Fax:	+1 508-749-2804

100 Minuteman Road - M/S 635			Email:	lindbergh@pictel.com

Andover, MA 01810 USA

Title:	Proposed draft of H.MULTILINK – CHANNEL AGGREGATION PROTOCOL FOR MULTILINK OPERATION

1. Overview

2. References

3. Definitions

4. Format Conventions

4.1 Numbering Convention

4.2 Order of Bit Transmission

4.3 Field Mapping Convention

5. Protocol Definition

5.1 Protocol Overview

5.2 Header Format

5.2.1 FLAG

5.2.2 CONTROL

5.2.3 SEQUENCE NUMBER

5.2.4 CHANNEL TAG

5.2.5 CHANNEL PROPORTION

5.2.6 DATA CRC

5.2.7 HEADER CRC

5.3 Procedures

5.3.1 Data Transmission

5.3.1.1 Transmitter Model

5.3.1.2 Data Set Transmission

5.3.1.3 Channel Reference Model

5.3.1.4 Fill Insertion

5.3.1.5 Transparency

5.3.1.6 Buffer Management

5.3.1.7 Adding and Removing Channels

5.3.2 Data Reception

5.3.2.1 Receiver Model

5.3.2.2 Transparency

5.3.2.3 Data Set Reception

5.3.2.4 Fill Removal

# Overview

This document describes a protocol for aggregation of data over multiple independent channels.  The protocol is designed to achieve the following goals:

- Low latency transmission even at very low bit rates (including POTS modem rates)

- Low overhead even at very low bit rates (including POTS modem rates)

- The ability to accommodate an arbitrary number of channels

- The ability to accommodate channels of arbitrary bit-rate

- The ability to accommodate channels of differing bit-rate

- The ability to accommodate channels of continuously varying bit-rate

- The ability to accommodate unidirectional or bi-directional channels

- Independence from channel order

- The ability for channels to drop out or be added arbitrarily

- Low implementation complexity

# References

[1]	ISO/IEC 3309 – Information Technology – Telecommunications and Information Exchange Between Systems -- High-Level Data Link Control (HDLC) Procedures – Frame Structure

# Definitions

Channel:  A single unidirectional data stream over which data is transmitted.  Channel aggregation involves transmission of data over more than one channel.

Channel Reference Model:  A model used by the transmitter and receiver to maintain synchronization of the channel aggregation rule.

Channel Set:  The set of Channels over which a given Data Set is transmitted.

Data Set:  A portion of the input data stream to be transmitted to which a single channel aggregation rule is applied.  Transmission of a Header Set delineates a given Data Set from the previous Data Set and specifies the channel aggregation rule to be used.

Header:  A collection of parameters whose beginning is marked by one or more Flags.  A Header may either be part of a Header Set, delineating the beginning of a Data Set, or may be used on one or more channels for fill insertion.

Header Set:  The set of Headers that delineate the beginning of a Data Set.  Each Header in a Header Set is transmitted over a different Channel in the Channel Set.

Sample:  The smallest unit of data which is always kept contiguous when distributing data among multiple channels.  The size of the Sample is 8 bits.

# Format Conventions

## Numbering Convention

The numbering convention used in H.MULTILINK is shown in Figure 1.  Bits are shown grouped into octets.  The bits of a single octet are shown horizontally and numbered from 1 through 8.  Multiple octets are shown vertically.

Figure 1 – Numbering convention

## Order of Bit Transmission

The octets are transmitted in ascending numerical order.  Within each octet, bit 1 is the first bit to be transmitted.

## Field Mapping Convention

When a field is contained within a single octet, the lowest-numbered bit of the field represents the lowest-order value (or the least significant bit).

When a field spans more than one octet, the highest-numbered bit of the first octet represents the highest-order value, and the lowest-numbered bit of the last octet represents the lowest-order value.

An exception to the preceding field-mapping convention are the CRC parameters. In these cases, the lowest-numbered bit of the first octet is the highest-order term of the polynomial representing the CRC, and the highest-numbered bit of the last octet is the lowest-order term of the polynomial representing the CRC.

# Protocol Definition

## Protocol Overview

H.MULTILINK acts as an additional protocol layer inserted below any protocol that would normally transmit data over a single data channel.  The interface from the higher layer protocol is a stream of bits, emulating a synchronous serial channel.

The result of the channel aggregation operation is to produce data to be transmitted on a number of separate output Channels.  At the receiver, the data from each of these Channels is recombined to reconstruct the original data stream presented to the channel aggregation layer at the transmitter.

The channel aggregation operation divides the input data stream into Samples of 8 bits each.  Each sample is transmitted on one of the available Channels.  The channel aggregation operation can proportionally distribute Samples among all available Channels — accounting for their relative data rates — so that as much as possible of the total available bandwidth is used and latency can be kept to a minimum.

In order to reconstruct the original data stream at the receiver, the receiver must be able to determine which Samples were placed on which Channel.  This is done by means of a Channel Reference Model that operates in synchrony at both the transmitter and receiver.

For the Channel Reference Model to operate effectively, it must depend on the relative rates of all Channels in use.  If the relative rates vary (or if the estimate of the relative rates vary) or if the number of Channels in use changes the model must be updated with this new information.  The Channel Reference Model is updated by transmission of overhead information.

A portion of the input data stream over which the Channel Reference Model operates autonomously without any updates is called a Data Set (see Figure 2).  A Data Set may be of arbitrary length — a Data Set continues indefinitely until another Data Set begins in its place.

The beginning of a Data Set is marked by transmission of a Header Set.  A Header Set consists of a Header transmitted on each Channel to be used for this Data Set (the set of all Channels to be used by a Data Set is referred to as its Channel Set).  The Header Set includes all of the information needed to synchronize the Channel Reference Models at the transmitter and receiver for the duration of the Data Set.

A Data Set may continue as long as the current Channel Reference Model accurately reflects the actual rates of the Channels in use (new Data Sets should also be started periodically to avoid significant error propagation).

Note – No explicit mechanism for extensibility of H.MULTILINK is provided within the protocol itself.  It is assumed that the negotiation into H.MULTILINK is done using an out-of-band mechanism, and that this mechanism would also be able to negotiate the use of any future versions of H.MULTILINK by mutual agreement between end-points.

Figure 2 – Transmission of channel aggregated data

## Header Format

The Header contains the following parameters:

- FLAG

- CONTROL

- SEQUENCE NUMBER

- CHANNEL TAG

- CHANNEL PROPORTION

- DATA CRC

- HEADER CRC

The format of the Header is shown in Figure 3.  The shaded octets are mandatory in all Headers, while the presence of the unshaded octets depends on the use of the Header.

Figure 3 – Header format

### FLAG

The FLAG parameter of the Header shall contain one or more FLAG PATTERNs in contiguous succession.

Each FLAG PATTERN is 16 bits in length and shall be set to the binary value 0111111111111110.

### CONTROL

The CONTROL parameter contains a series of fields defined in Figure 4 and Table 1.

Figure 4 – Format of the CONTROL parameter

Table 1 – Definition of fields in the CONTROL parameter

| Field | Value | Meaning |
| --- | --- | --- |
| SN | 00 | This Header does not include a SEQUENCE NUMBER parameter. This value shall only be used when the Header is being used for fill insertion. This value shall not be used when the Header is part of a Header Set. When this value is used, all remaining bits of the CONTROL parameter shall be zero and the Header shall not include a CHANNEL TAG nor a CHANNEL PROPORTION parameter. |
|  | 01 | A 1 octet SEQUENCE NUMBER parameter is included in this Header. |
|  | 10 | A 2 octet SEQUENCE NUMBER parameter is included in this Header. |
|  | 11 | A 4 octet SEQUENCE NUMBER parameter is included in this Header. |
| CT | 00 | This Header does not include a CHANNEL TAG parameter. This value shall not be used unless there is exactly one Header in the Header Set. |
|  | 01 | A 1 octet CHANNEL TAG parameter is included in this Header. |
|  | 10 | A 2 octet CHANNEL TAG parameter is included in this Header. |
|  | 11 | A 4 octet CHANNEL TAG parameter is included in this Header. |
| CP | 00 | This Header does not include a CHANNEL PROPORTION parameter. If this value is used, the receiver shall use a CHANNEL PROPORTION value of one for this Channel. |
|  | 01 | A 1 octet CHANNEL PROPORTION parameter is included in this Header. |
|  | 10 | A 2 octet CHANNEL PROPORTION parameter is included in this Header. |
|  | 11 | A 4 octet CHANNEL PROPORTION parameter is included in this Header. |
| L | 1 | The CHANNEL TAG in this Header is the largest valued CHANNEL TAG over all Channels in the Channel Set. |
|  | 0 | All other cases. |
| D | 1 | This Header includes a DATA CRC parameter. |
|  | 0 | This Header does not include a DATA CRC parameter. |

### SEQUENCE NUMBER

The SEQUENCE NUMBER parameter contains a numerical value that represents the temporal order of the Header Set relative to all other Header Sets that have been or will be transmitted.

The value of the SEQUENCE NUMBER parameter for all Headers in a Header Set shall be identical.

For each successive Header Set, the SEQUENCE NUMBER shall be incremented by one, modulo 2N, where N is the number of bits in the SEQUENCE NUMBER parameter for the Header Set, as indicated by the value of the SN field of the CONTROL parameter, and the SEQUENCE NUMBER is represented as an unsigned binary integer.

The number of bits, N, in the SEQUENCE NUMBER parameter shall be identical for all Headers in a Header set, and shall be chosen such that the difference between the maximum and minimum number of  Header Sets that could be in transit over any channel is less than 2N.  An estimate of the possible number of Header Sets in transit should account for the maximum end-to-end skew between channels, the maximum allowed skew between transmit buffers for each channel (see Section 5.3.1.6), and the range of temporal spacing allowed between Header Sets.

### CHANNEL TAG

The CHANNEL TAG parameter contains a unique identifier for each Header in the Header Set.

For a Header Set containing M Headers, each Header shall contain a unique CHANNEL TAG whose value shall be in the range 0 through M-1, represented as an unsigned binary integer.  The particular CHANNEL TAG chosen for a given Header is arbitrary, but all values in the range 0 through M-1 shall be used for some Header in the Header Set.

If there is exactly one Header in the Header Set, the CHANNEL TAG parameter may be omitted (indicated by a value of 00 for the CT field of the CONTROL parameter).

### CHANNEL PROPORTION

The CHANNEL PROPORTION parameter indicates the relative rate at which data is to be transmitted over the Channel corresponding to a given Header.  This value is used in determining the allocation of data between Channels as defined by the Channel Reference Model.

The CHANNEL PROPORTION represents an unsigned binary integer.  The value of this integer indicates the ratio of the total data that is to be transmitted on the corresponding Channel.  This ratio is equal to P/T, where P is the CHANNEL PROPORTION for this Channel, and T is the total of the CHANNEL PROPORTION parameters for all Channels in the Channel Set.

If the CHANNEL PROPORTION is equal to one for a given Header, the CHANNEL PROPORTION parameter may be omitted from that Header (indicated by a value of 00 for the CP field of the CONTROL parameter).

### DATA CRC

The DATA CRC is an optional 16-bit CRC to allow detection of errors in the data from the previous Data Set on the corresponding channel.  The presence of this parameter is indicated by a value of one in the D bit of the CONTROL parameter.

The DATA CRC shall be computed according to the procedures defined for computing the 16-bit FCS field (Frame Check Sequence) in [1].  Unlike [1], the CRC shall be computed only on the Samples of the Data Set on the corresponding Channel between the Header marking the beginning of the previous Data Set and the current Header.  Any intervening fill Headers shall not be included in the CRC computation.

Use of this optional parameter by the receiver is beyond the scope of this Recommendation.

### HEADER CRC

The HEADER CRC is a 16-bit CRC to allow detection of errors in the Header contents.

The HEADER CRC shall be computed according to the procedures defined for computing the 16-bit FCS field (Frame Check Sequence) in [1].  Unlike [1], the CRC shall be computed only on the Header data.  The CRC computation shall include all Header parameters between the FLAG and the HEADER CRC.

## Procedures

### Data Transmission

#### Transmitter Model

The input to the transmitter is assumed to be a continuously running stream of bits, where one new bit becomes available at a time (an actual implementation may supply bits in larger groups than this).  The input data stream is placed on the Input Queue of the transmitter.

The operation of the transmitter can be divided into the transmit aggregator and the transparency control mechanism (see Figure 5).  The results from the transmit aggregator are placed in Intermediate Transmit Queues — one for each Channel.  Note that the Intermediate Transmit Queues must include means to distinguish between FLAGs intentionally transmitted as part of Headers, and emulated FLAGs that may be contained within the other data.

Transparency control is performed on the data in the Intermediate Transmit Queues (see Section 5.3.1.5) and the results are placed in the Transmit Queues — again, one for each Channel.  These queues are considered the final output of the transmitter.

There may be some delay between the time that a particular group of bits is placed on a channel’s Transmit Queue and the time that it is actually transmitted over the communications link.  The transmitter shall have a means to determine the amount of this delay at any time.

The transmitter is responsible for ensuring that the Transmit Queues do not underrun.  Procedures to ensure this are given in Sections 5.3.1.2 and 5.3.1.4.

Processing of data for transmission is intended to be done continuously as new input data is supplied.  It is not intended that a transmitter wait for an entire Data Set to arrive prior to processing the data for transmission.

Figure 5 – Transmitter Model

#### Data Set Transmission

The beginning of a new Data Set is marked by transmission of a Header Set.

To begin a new Data Set, a transmitter shall determine the corresponding Channel Set — which Channels it intends to transmit the Data Set over.  The Channel Set may include one or more Channels.  Note that the Channel Set would normally include all available Channels.

The transmitter shall then determine the content of each Header in the Header Set — one Header for each Channel in the Channel Set — based on the definition of the Header parameters in Section 5.2.

For each Channel, the corresponding Header shall be placed on the Intermediate Transmit Queue of that Channel immediately following the last Sample of the previous Data Set to be transmitted on this Channel.  Since the size of a Data Set is not defined a priori, transmission of the previous Data Set may be terminated at any Sample boundary.  If there is a need to delay transmission of the new Data Set after termination of the previous one, additional FLAG PATTERNs may be included in the Header to take up the extra space and avoid underrun.

Immediately following the Header on each Channel in the Channel Set, the data from the Data Set shall be transmitted.  The rules for determining which portion of the Data Set is transmitted on which Channel is given by the Channel Reference Model (see Section 5.3.1.3).

At the beginning of a Data Set, the Channel Reference Model shall be reset to its initial state.  The Channel Reference Model shall also be given the number of Channels in the Channel Set as well as the CHANNEL TAG and CHANNEL PROPORTION value for each Channel.

The data in the Data Set is divided into Samples of 8 bits each.  The Channel Reference Model is used to determine which Channel each Sample should be transmitted over.  For each Sample, the Channel Reference Model shall be incremented by one step.  Each step of the Channel Reference Model results in a CHANNEL TAG.  The transmitter shall place the Sample on the Intermediate Transmit Queue of the Channel corresponding to the resulting CHANNEL TAG — the Channel over which the Header containing that CHANNEL TAG was transmitted for this Data Set.

#### Channel Reference Model

The Channel Reference Model is a finite state machine that accepts the following parameters as input:

- The number of Channels in the Channel Set

- The CHANNEL TAG for each Channel in the Channel Set

- The CHANNEL PROPORTION for each Channel in the Channel Set

The Channel Reference Model contains a series of separate state variables, one for each Channel in the Channel set, indexed by the CHANNEL TAG for that Channel.  Each state variable is a signed integer value.

The following state transition operations are defined by the Channel Reference Model:

- Reset to the initial state

When the Channel Reference Model is reset to its initial state, the value of all of its state variables shall exactly equal zero.

- Increment the state by one step

For each increment of the state by one step, the following procedure shall be applied in the order stated:

- The value of the state variable for each channel shall be incremented by the value of the CHANNEL PROPORTION for that channel.

- For all Channels, the state variable with the largest value is chosen.  If there is more than one state variable with exactly the same value, then among those, the state variable corresponding to the smallest valued CHANNEL TAG is chosen.

- The value T shall be subtracted from the value of the chosen state variable, where T is the sum of all of the CHANNEL PROPORTION values for all Channels in the Channel Set.

- The CHANNEL TAG value corresponding to the chosen state variable shall be indicated as the result.

All arithmetic operations in the above procedure shall result in no loss of precision.

#### Fill Insertion

The transmit aggregator shall prevent underrun of the Transmit Queues (the potential for underrun can occur as a result of a mismatch between the Channel Reference Model and the actual channel rates).  If a Channel is at risk of underruning, the transmit aggregator shall insert fill data into the Intermediate Transmit Queue of that Channel of an amount sufficient to prevent such an underrun.

The fill data shall consist of a Header that contains one or more FLAG PATTERNs followed by a CONTROL parameter with 00 as the value of the SN field (see Section 5.2.2 for resulting restrictions on the Header format).  A Header used for fill shall only include the FLAG, CONTROL, and HEADER CRC parameters.

Fill data shall be inserted into the Intermediate Transmit Queue of a Channel between successive Samples of the Data Set and shall not interrupt an individual Sample.  Fill data shall not be inserted inside another Header.

Fill insertion shall not affect the operation of the Channel Reference Model.  The content of the Samples before and after the fill data shall not depend on the presence or length of the fill data.  In effect, the fill insertion occurs after and independently from the operation of the transmit aggregator.

#### Transparency

Data being transmitted on each Channel between successive FLAGs shall be processed using transparency control to eliminate the possibility of flag emulation in the data stream.  For each Channel, for all data in the Intermediate Transmit Queue between FLAGs, the transmitter shall insert a “0” bit after all sequences of 13 contiguous “1” bits, and then place the data in the Transmit Queue of that Channel.

#### Buffer Management

A Data Set may continue indefinitely until one of the following criteria is reached:

- When the skew between any two Transmit Queues for all Channels exceeds a locally specified threshold.  The skew is defined as the difference between the maximum and minimum values of the estimated amount of time in the Transmit Queue across all Channels in the Channel Set.  The amount of time in the Transmit Queue is measured based on the fullness of the queue and the estimated rate of that Channel.

- When a certain locally defined time period has passed.  New Data Sets should be started periodically in order to avoid indefinite propagation of errors.

- At any other time as determined by the transmitting system.

#### Adding and Removing Channels

Channels may be added or removed during transmission at any time.

When a new Channel becomes available, no change is made to the operation of the Data Set being transmitted.  Any time after the Channel becomes available, the transmitter may include the new Channel in the Channel Set associated with any subsequent Data Set.

If a Channel is to become unavailable, the transmitter shall begin a new Data Set which does not include this Channel in its associated Channel Set, and shall not include this Channel in any subsequent Channel Sets unless this Channel later becomes available.  If the loss of the Channel is under the control of the transmitter, the transmitter shall not make the Channel unavailable until it has begun the new Data Set and until all data from previous Data Sets have been transmitted over the Channel.  If a Channel becomes unavailable without prior knowledge by the transmitter, data from the current Data Set not yet transmitted over that Channel prior to the beginning of the new Data Set will be lost.

### Data Reception

#### Receiver Model

The receiver receives data from all available Channels.  The received data from each Channel is placed on a Receive Queue for that Channel.  The receiver processes the data in each Receive Queue according to the procedures defined in this section.

The operation of the receiver can be divided into the transparency control mechanism and the receive recombiner (see Figure 6).  FLAG detection and transparency control (see Section 5.3.2.2) is performed on the data from each of the Receive Queues and the results are placed in Intermediate Receive Queues.  Note that the Intermediate Receive Queues must include means to distinguish between FLAGs received as part of Headers, and emulated FLAGs that may be contained within payload data.

The receive recombiner operates on the data in the Intermediate Receive Queues and the result is placed in the Output Queue.  This queue is considered the final output of the receiver.

Processing of data for reception is intended to be done continuously as new input data is supplied.  It is not intended that a receiver wait for an entire Data Set to arrive prior to processing the data from that Data Set.

Figure 6 – Receiver Model

#### Transparency

A receiver shall continuously examine the data received on the Receive Queue of each available Channel, searching for FLAGs.  If a FLAG is detected, this signifies the presence of a Header.

Data being received on each Channel between successive FLAGs shall be processed using transparency control to reverse the transparency control process used by the transmitter.  For each Channel, for all data in the Receive Queue between FLAGs, the receiver shall discard any “0” bit which directly follows 13 contiguous “1” bits and place the resulting data on the Intermediate Receive Queue for that Channel.

#### Data Set Reception

Reception of data from a particular Data Set begins with the reception of all Headers in the corresponding Header Set.

A receiving system determines when a complete Header Set has been received by examining the CONTROL, SEQUENCE NUMBER, and CHANNEL TAG parameters in the received Headers (see Section 5.2).  Headers are considered to be part of the same Header Set if they contain identical Sequence Numbers.  A receiver determines the number of Headers in the Header Set (which is equal to the number of Channels in the Channel Set) when it receives a Header containing either no CHANNEL TAG (indicated by a value of 00 for the CT field of the CONTROL parameter), in which case it knows that the number of Headers is exactly one, or when it receives a Header with the value of one for the L bit CONTROL parameter, in which case the number of Headers is one plus the value of the CHANNEL TAG contained in this Header.  A complete Header Set consists of all Headers with CHANNEL TAG values between zero and the value contained in the Header with the L bit set to one.

All Headers shall be examined to determine if there are errors in the contents of the Header.  The HEADER CRC shall be used to determine whether errors have occurred according to the procedures defined in Section 5.2.6.  A Header received in error shall be ignored.  If this Header is part of a Header Set, this may mean that the complete Header Set is never fully received.  If this is the case, the receiver shall wait until a complete Header Set for a subsequent Data Set before continuing to process any data for reception.  In this case, the complete Data Set with the erroneous Header is lost.

During the time that the receiver is waiting for all Headers in a Header Set to be received, data following any Headers in that Header Set shall be held until all Headers have been received.  Only when all Headers have been received can recombination of the Data Set begin.

Once all Headers have been received for a given Header Set, the data following each of the headers can begin to be processed in order to reconstruct the transmitted Data Set.  The rules for determining which portion of the Data Set is to be extracted from which Channel is given is given by the Channel Reference Model (see Section 5.3.1.3).

At the beginning of a Data Set, the Channel Reference Model is reset to its initial state.  It is also given the number of Channels in the Channel Set as well as the CHANNEL TAG and CHANNEL PROPORTION values received in the Header for each Channel.

The Channel Reference Model is used to determine which Channel each Sample is to be received over.  For each Sample to be extracted into the reconstructed Data Set, the Reference Model is incremented by one step.  Each step of the Channel Reference Model results in a CHANNEL TAG.  The receiver shall extract the Sample on the Intermediate Receive Queue of the Channel corresponding to the resulting CHANNEL TAG — the Channel over which the Header containing that CHANNEL TAG was received for this Data Set.

If there have been no errors in a given Data Set, the receive recombiner will extract the complete Data Set from the Intermediate Receive Queue prior to the extraction of any Headers for the subsequent Data Set.  However, if errors have occurred on one or more Channels, it is possible that the Channel Reference Model indicates that data is to be extracted from a Channel in which there is no more data prior to the subsequent Header, while on other Channels, data from the current Data Set still remains.  If this condition occurs the receiver shall immediately discontinue processing of the current Data Set and begin processing of the subsequent Data Set.

#### Fill Removal

Reception of a Header with the value 00 as the value of the SN field (and the remainder of the Header set according to Section 5.2.2) indicates that the Header is being used for fill insertion on that Channel.

When the receive recombiner receives such a Header, it shall remove the complete Header from the Intermediate Receive Queue and discard it.  It shall continue processing subsequent data on that Channel as if the fill Header had never been received.

[end]