ITU-TS SG15	LBC-97-048

Working Party 15/1	February 1997

Expert’s Group For Very Low Bitrate Visual Telephony

Nice (25-28)   February 1997

Source:	Telenor Satellite Services

Title:	Comments to H.263 Annex N

Author:	G. Bjontegaard

# Introduction

The content of this document has previously been distributed by E-mail.

In section N.4 it is stated that: ‘The decoder of this mode has additional plural number of picture memories....’

When I worked with NEWPRED based on NACK, I had in mind that this added memory on the decoder should not be necessary.  I still think it is possible to obtain this.

# Possible changes to Annex N.

To avoid additional picture memories, N.3.2.2 and N.3.2.3 would need to be changed:

## N.3.2.2

Remove RF from the syntax.  (It is intended for NACK only?)

## N.3.2.3

For ACK, the definition of TR is as already stated.  For NACK, TR means the temporal reference of the last correctly decoded data - that we ask the encoder to use for prediction.  (This TR is reliable and we therefore do not need RF).

In case of NACK, the decoder may then choose to operate in the following way:

When an error is detected and a NACK with TR is sent to the encoder for a certain GOB, the decoder may choose to stop all decoding of that GOB until the encoder has reacted to the NACK (or sent INTRA update).  The decoder will know that the encoder has reacted when: TRPI=1 and TRP=TR.

Have I misunderstood something?  In case it is possible to avoid additional memory at the decoder with the changes I just proposed, I think such a change would be worth wile.