ITU-Telecommunication Standardization Sector		LBC-97-050

Working Party 15/1

Expert’s Group on Very Low Bitrate Visual Telephony

Nice, February 25-28, 1997

Source:	Ericsson1

Title:	Results from CE with Robust FLC (LBC-97-037)

Purpose:	Information

Introduction.

Core Experiment ”Evaluation of Error Resilient FLC for the DC Coefficient” has been carried out as described in LBC-97-037 and MPEG-4 WG11/N1383. A robust FLC for Intra DC components is compared to the ”linear” FLC in H.263 and VM4.  The comparison is done with 11 macro-blocks per VOP (qcif) coded as Intra. Adaptive slice is used. (Valid in MPEG-4, meaning that the number of bits per slice is ³ 480 for Class A and³ 736 for Class B).

The comparison is done for two different coding modes. In both modes the  Intra DC is included in ‘hdr’:

- Combined mode:	sync, hdr,cof,hdr,cof, ...., sync

- Separate mode:	sync, hdr,hdr, ..., cof,cof, ...., sync

Simulation results.

In tables 1-14 SNR values averaged over 50 iterations are shown for 7 different channel conditions and 5 different sequences.

Conclusion.

The simulation results shows that the proposed RFLC is robust to random errors in chrominance Intra DC with

0.9 - 4.5 dB improvement depending on sequence.

For the luminance Intra DC the gain is 0 to 0.9 dB for random errors.

For other channel conditions than random errors no significant difference between the current FLC and the proposed FLC is observed.

1Contact person: Göran Bång      Email: goran.bang@era-t.ericsson.se

Table 1. Combined mode, Random errrors, ber=1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 22.68 | 20.98 | 36.49 | 39.33 | 22.60 | 20.97 | 34.56 | 35.20 | 0.08 | 0.01 | 1.93 | 4.13 |
| contain | 28.68 | 27.15 | 37.40 | 36.79 | 28.06 | 26.79 | 33.99 | 34.10 | 0.62 | 0.36 | 3.41 | 2.69 |
| foreman | 20.58 | 18.93 | 32.18 | 31.85 | 20.53 | 18.92 | 31.15 | 30.94 | 0.05 | 0.01 | 1.03 | 0.91 |
| mad | 29.50 | 28.09 | 36.01 | 37.09 | 28.64 | 27.49 | 33.86 | 34.73 | 0.86 | 0.60 | 2.15 | 2.36 |
| silent | 26.92 | 25.44 | 33.48 | 35.18 | 26.55 | 25.24 | 32.39 | 32.95 | 0.37 | 0.20 | 1.09 | 2.23 |

Table 2. Combined mode, Burst errors 1 ms, ber=1e-2

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 23.68 | 22.05 | 34.55 | 37.17 | 23.66 | 22.05 | 34.23 | 36.04 | 0.02 | 0.00 | 0.32 | 1.13 |
| contain | 27.07 | 25.81 | 32.71 | 33.47 | 26.95 | 25.80 | 31.82 | 32.11 | 0.12 | 0.01 | 0.89 | 1.36 |
| foreman | 21.65 | 20.05 | 31.70 | 31.82 | 21.62 | 20.02 | 31.76 | 31.52 | 0.03 | 0.03 | -0.06 | 0.30 |
| mad | 28.01 | 26.86 | 32.70 | 34.06 | 27.72 | 26.76 | 31.32 | 32.30 | 0.29 | 0.10 | 1.38 | 1.76 |
| silent | 25.54 | 24.25 | 30.32 | 32.26 | 25.45 | 24.29 | 29.75 | 30.60 | 0.09 | -0.04 | 0.57 | 1.66 |

Table 3. Combined mode, Burst errors 10 ms, ber=1e-2

|  | RFLC |  |  |  | VM4-FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 28.17 | 26.50 | 39.69 | 42.59 | 28.16 | 26.49 | 39.74 | 42.59 | 0.01 | 0.01 | -0.05 | 0.00 |
| contain | 33.02 | 31.72 | 38.74 | 38.07 | 33.01 | 31.71 | 38.71 | 38.04 | 0.01 | 0.01 | 0.03 | 0.03 |
| foreman | 27.83 | 26.29 | 36.35 | 36.35 | 27.83 | 26.29 | 36.43 | 36.37 | 0.00 | 0.00 | -0.08 | -0.02 |
| mad | 34.41 | 33.15 | 39.38 | 40.06 | 34.44 | 33.17 | 39.51 | 40.19 | -0.03 | -0.02 | -0.13 | -0.13 |
| silent | 30.21 | 28.95 | 34.61 | 36.10 | 30.19 | 28.92 | 34.56 | 36.23 | 0.02 | 0.03 | 0.05 | -0.13 |

Table 4. Combined mode, Burst errors 1 ms, ber=1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 29.18 | 27.53 | 39.44 | 42.34 | 29.16 | 27.52 | 39.34 | 42.22 | 0.02 | 0.01 | 0.10 | 0.12 |
| contain | 32.93 | 31.69 | 38.21 | 37.78 | 32.91 | 31.69 | 38.01 | 37.79 | 0.02 | 0.00 | 0.20 | -0.01 |
| foreman | 29.15 | 27.69 | 36.40 | 36.46 | 29.15 | 27.69 | 36.41 | 36.54 | 0.00 | 0.00 | -0.01 | -0.08 |
| mad | 34.80 | 33.65 | 38.92 | 40.13 | 34.61 | 33.58 | 38.57 | 39.46 | 0.19 | 0.07 | 0.35 | 0.67 |
| silent | 30.71 | 29.53 | 34.56 | 36.02 | 30.67 | 29.50 | 34.39 | 36.02 | 0.04 | 0.03 | 0.17 | 0.00 |

Table 5. Combined mode, Burst errors 10 ms, ber=1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 30.57 | 28.93 | 40.16 | 43.09 | 30.57 | 28.93 | 40.16 | 43.09 | 0.00 | 0.00 | 0.00 | 0.00 |
| contain | 34.03 | 32.79 | 39.10 | 38.42 | 34.02 | 32.78 | 39.11 | 38.43 | 0.01 | 0.01 | -0.01 | -0.01 |
| foreman | 31.68 | 30.34 | 37.29 | 37.36 | 31.68 | 30.34 | 37.32 | 37.40 | 0.00 | 0.00 | -0.03 | -0.04 |
| mad | 36.35 | 35.17 | 40.25 | 40.96 | 36.35 | 35.17 | 40.26 | 40.97 | 0.00 | 0.00 | -0.01 | -0.01 |
| silent | 31.94 | 30.82 | 35.11 | 36.74 | 31.94 | 30.82 | 35.13 | 36.73 | 0.00 | 0.00 | -0.02 | 0.01 |

Table 6. Combined mode, Packet loss 1e-2

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 28.72 | 27.05 | 39.74 | 42.59 | 28.69 | 27.04 | 39.53 | 42.39 | 0.03 | 0.01 | 0.21 | 0.20 |
| contain | 33.56 | 32.31 | 38.84 | 38.08 | 33.58 | 32.32 | 38.87 | 38.22 | -0.02 | -0.01 | -0.03 | -0.14 |
| foreman | 28.22 | 26.69 | 36.47 | 36.51 | 28.23 | 26.69 | 36.48 | 36.51 | -0.01 | 0.00 | -0.01 | 0.00 |
| mad | 35.56 | 34.30 | 40.16 | 40.91 | 35.50 | 34.24 | 40.16 | 40.83 | 0.06 | 0.06 | 0.00 | 0.08 |
| silent | 31.06 | 29.83 | 35.03 | 36.56 | 31.01 | 29.79 | 34.96 | 36.41 | 0.05 | 0.04 | 0.07 | 0.15 |

Table 7. Combined mode, Packet loss 1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 30.53 | 28.90 | 40.07 | 42.92 | 30.53 | 28.90 | 40.01 | 42.97 | 0.00 | 0.00 | 0.06 | -0.05 |
| contain | 34.06 | 32.82 | 39.10 | 38.40 | 34.06 | 32.82 | 39.06 | 38.43 | 0.00 | 0.00 | 0.04 | -0.03 |
| foreman | 31.60 | 30.25 | 37.30 | 37.38 | 31.60 | 30.25 | 37.32 | 37.38 | 0.00 | 0.00 | -0.02 | 0.00 |
| mad | 36.36 | 35.18 | 40.25 | 41.04 | 36.35 | 35.17 | 40.28 | 40.99 | 0.01 | 0.01 | -0.03 | 0.05 |
| silent | 31.96 | 30.83 | 35.14 | 36.84 | 31.92 | 30.80 | 35.09 | 36.70 | 0.04 | 0.03 | 0.05 | 0.14 |

Table 8. Separate mode, Random errrors, ber=1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 23.36 | 21.64 | 37.87 | 40.94 | 23.24 | 21.60 | 35.41 | 36.43 | 0.12 | 0.04 | 2.46 | 4.51 |
| contain | 28.59 | 27.06 | 37.39 | 36.80 | 27.95 | 26.72 | 34.06 | 34.00 | 0.64 | 0.34 | 3.33 | 2.80 |
| foreman | 21.53 | 19.89 | 32.84 | 32.70 | 21.49 | 19.88 | 31.83 | 31.71 | 0.04 | 0.01 | 1.01 | 0.99 |
| mad | 29.79 | 28.36 | 36.48 | 37.48 | 29.07 | 27.87 | 34.93 | 35.37 | 0.72 | 0.49 | 1.55 | 2.11 |
| silent | 26.95 | 25.46 | 33.70 | 35.38 | 26.58 | 25.27 | 32.33 | 32.82 | 0.37 | 0.19 | 1.37 | 2.56 |

Table 9. Separate mode, Burst errors 1 ms, ber=1e-2

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 24.43 | 22.76 | 36.84 | 40.33 | 24.42 | 22.77 | 36.32 | 39.31 | 0.01 | -0.01 | 0.52 | 1.02 |
| contain | 26.94 | 25.57 | 33.27 | 34.24 | 26.80 | 25.56 | 32.40 | 33.48 | 0.14 | 0.01 | 0.87 | 0.76 |
| foreman | 22.60 | 20.99 | 32.83 | 33.19 | 22.60 | 21.00 | 32.86 | 32.48 | 0.00 | -0.01 | -0.03 | 0.71 |
| mad | 28.38 | 27.14 | 33.24 | 35.46 | 28.16 | 27.06 | 32.13 | 33.97 | 0.22 | 0.08 | 1.11 | 1.49 |
| silent | 25.61 | 24.29 | 30.64 | 32.95 | 25.47 | 24.26 | 29.88 | 31.46 | 0.14 | 0.03 | 0.76 | 1.49 |

Table 10. Separate mode, Burst errors 10 ms, ber=1e-2

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 28.31 | 26.63 | 39.65 | 42.67 | 28.32 | 26.65 | 39.82 | 42.71 | -0.01 | -0.02 | -0.17 | -0.04 |
| contain | 32.44 | 31.11 | 38.45 | 37.84 | 32.46 | 31.15 | 38.45 | 37.66 | -0.02 | -0.04 | 0.00 | 0.18 |
| foreman | 28.15 | 26.62 | 36.54 | 36.50 | 28.16 | 26.63 | 36.58 | 36.54 | -0.01 | -0.01 | -0.04 | -0.04 |
| mad | 34.36 | 33.03 | 39.86 | 40.45 | 34.36 | 33.05 | 39.71 | 40.41 | 0.00 | -0.02 | 0.15 | 0.04 |
| silent | 30.21 | 28.91 | 34.81 | 36.40 | 30.23 | 28.93 | 34.80 | 36.34 | -0.02 | -0.02 | 0.01 | 0.06 |

Table 11. Separate mode, Burst errors 1 ms, ber=1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 29.53 | 27.88 | 39.74 | 42.71 | 29.51 | 27.87 | 39.69 | 42.66 | 0.02 | 0.01 | 0.05 | 0.05 |
| contain | 32.60 | 31.34 | 37.88 | 37.78 | 32.57 | 31.37 | 37.74 | 37.37 | 0.03 | -0.03 | 0.14 | 0.41 |
| foreman | 29.71 | 28.26 | 36.68 | 36.90 | 29.71 | 28.26 | 36.58 | 36.85 | 0.00 | 0.00 | 0.10 | 0.05 |
| mad | 34.83 | 33.67 | 39.00 | 40.11 | 34.76 | 33.69 | 38.84 | 39.67 | 0.07 | -0.02 | 0.16 | 0.44 |
| silent | 30.76 | 29.55 | 34.54 | 36.37 | 30.75 | 29.58 | 34.49 | 36.17 | 0.01 | -0.03 | 0.05 | 0.20 |

Table 12. Separate mode, Burst errors 10 ms, ber=1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 30.61 | 28.97 | 40.12 | 43.00 | 30.61 | 28.98 | 40.16 | 43.07 | 0.00 | -0.01 | -0.04 | -0.07 |
| contain | 33.97 | 32.72 | 39.10 | 38.44 | 33.97 | 32.72 | 39.10 | 38.44 | 0.00 | 0.00 | 0.00 | 0.00 |
| foreman | 31.74 | 30.41 | 37.33 | 37.42 | 31.74 | 30.41 | 37.33 | 37.39 | 0.00 | 0.00 | 0.00 | 0.03 |
| mad | 36.29 | 35.10 | 40.22 | 41.03 | 36.32 | 35.14 | 40.23 | 41.01 | -0.03 | -0.04 | -0.01 | 0.02 |
| silent | 31.95 | 30.82 | 35.13 | 36.80 | 31.95 | 30.82 | 35.14 | 36.77 | 0.00 | 0.00 | -0.01 | 0.03 |

Table 13. Separate mode, Packet loss 1e-2

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 28.92 | 27.26 | 39.73 | 42.47 | 28.91 | 27.26 | 39.70 | 42.46 | 0.01 | 0.00 | 0.03 | 0.01 |
| contain | 33.11 | 31.83 | 38.65 | 37.99 | 33.22 | 31.92 | 38.81 | 38.16 | -0.11 | -0.09 | -0.16 | -0.17 |
| foreman | 28.70 | 27.18 | 36.68 | 36.68 | 28.68 | 27.17 | 36.68 | 36.70 | 0.02 | 0.01 | 0.00 | -0.02 |
| mad | 35.37 | 34.12 | 39.98 | 40.70 | 35.38 | 34.12 | 40.07 | 40.68 | -0.01 | 0.00 | -0.09 | 0.02 |
| silent | 30.98 | 29.75 | 34.95 | 36.55 | 31.00 | 29.77 | 35.00 | 36.65 | -0.02 | -0.02 | -0.05 | -0.10 |

Table 14. Separate mode, Packet loss 1e-3

|  | RFLC |  |  |  | VM-4 FLC |  |  |  | Gain |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P | Y | U | V | P | Y | U | V | P | Y | U | V |
| coastg | 30.59 | 28.96 | 40.12 | 42.99 | 30.59 | 28.96 | 40.06 | 42.96 | 0.00 | 0.00 | 0.06 | 0.03 |
| contain | 33.97 | 32.73 | 39.07 | 38.40 | 34.01 | 32.76 | 39.10 | 38.44 | -0.04 | -0.03 | -0.03 | -0.04 |
| foreman | 31.70 | 30.36 | 37.34 | 37.41 | 31.70 | 30.35 | 37.34 | 37.40 | 0.00 | 0.01 | 0.00 | 0.01 |
| mad | 36.33 | 35.14 | 40.29 | 41.04 | 36.35 | 35.16 | 40.29 | 41.05 | -0.02 | -0.02 | 0.00 | -0.01 |
| silent | 31.93 | 30.80 | 35.10 | 36.73 | 31.97 | 30.84 | 35.15 | 36.81 | -0.04 | -0.04 | -0.05 | -0.08 |