| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Second Meeting: Sunriver, Oregon, 8-11 September 1997 | Document Q15-B-04 Filename: q15b04.doc Date generated: 09/01/97 |
| --- | --- |

| Question: | Q.15/16 |  |
| --- | --- | --- |
| Source: | Stephan Wenger TU Berlin / TELES AG | Tel: +49 30 314 73160 Fax: +49 30 314 25156 Email: stewe@cs.tu-berlin.de |
| Title: | Report of the Ad Hoc Committee on H.263+ Recommended Modes and Integration with Terminal Standards (Joint AHC with Q11) |  |
| Purpose: | Information |  |

_____________________________

## Introduction

During the finishing session of the Portland meeting of Q.15/16, an Ad Hoc committee was created to work on Recommended Mode Combinations and Integration with Terminal Standard. There were no meetings of this AHC planned; all of the work was done on the Q15 reflector. Some of the issues mandated to the AHC were intensively discussed on the reflector and progress can be reported on them; others showed little or no activity. This document summarizes the results of the discussion process and identifies areas which need further work.

## Mandate

The mandate of the AHC was defined as follows (copied from the draft meeting report Q15a63d1.doc):

For every transport stack including H.323, H.324, and H.320

- work on a set of recommended mode combinations

- work on signaling of these mode combinations in H.245 and H.242

- discuss suggestions for reducing the number of optional mode combinations

In addition to the general issues above

- work on the Tandberg proposal regarding H.263 Annex C [Q11a57/Q15a46]

- work on H.245 code points for H.263 Draft Annex N [Q11a55/Q15a29, Q11a56/Q15a30]

Work on preferred mode combinations of the modes affecting the complexity (MIPS) / quality tradeoff will be based on the accepted proposal Q15a44 as refined in Q15a57.  Base on this document, a small number of levels (probably 3 or 4) should be identified.

In addition, the AHC will discuss the impact of the more system-related optional modes, especially H.263 Draft Annexes K, N, O, and R for various applications, which will be listed by their recommendation family numbers (e.g., H.320, H.323, and H.324).  Based on Mr. Okubo’s input [Q11a59/Q15a53], care will be taken to ensure easy interoperability between the different network worlds to allow gateways without transcoding.  However, an optional adaption to the network characteristics of the widely accepted networks (ISDN H.320, PSTN H.324, Internet and LAN H.323) will be given priority.

## Issues discussed

For threads on the Email reflector can be identified as belonging to the subject of the AHC:

- Preferred H.263+ Mode combinations regarding the MIPS vs. Quality modes

- Annex C (CPM) vs. H.263+,

- Annex N support in H.245

- Annex L in packet lossy environments.

It should be noted here, that the last topic also has strong relationship to the packetization and H.263+ Ad Hoc committees, because the packetization scheme has a strong impact on the problem solutions and the solutions themselves have impact to the language of H.263+.

## Reflector Discussion

The results of the Email discussions will be summarized here thread by thread.

### Preferred H.263+ mode combinations regarding the MIPS vs. Quality modes

It seemed to be consensus both during the Portland meeting and on the reflector, that the basic approach of dividing the optional modes into a MIPS vs. Quality tradeoff category and a Transport related category is useful for the further discussion. Some of the modes, especially Annexes H, K, N, and R belong clearly into the transport related category, others like D, E, F, I, J, P, Q, R, S and T belong to the MIPS vs. Quality category. The rest of the modes cannot easily assigned to one of the two groups, because they all have some impact on both aspects.

Shortly after the Portland meeting, the document Q15a57 (Matt Bace, PictureTel) proposed a set of three „levels“ of support of the MIPS vs. Quality modes F, I, J, P, S, and T. The reasons for the limitation to those modes can be found in the document. This document found some support.

Kevin O’Connell pointed out in two Emails, that, although he agreed to the basic idea of the level idea, he would like to see a different assignment of the modes to the „levels“. There was some discussion on this assignment, but no consensus was reached.

Kevin O’Connell also suggested two additional mode combinations, which were „baseline“ and „maximum coding efficiency“. Stephan Wenger proposed to change the somewhat limited „maximum coding efficiency“ to a „everything can be negotiated“ mode combination, which allows vendors to use all advanced features of their codecs. Kevin responded that this might too much.

Samson Cheung came up with the idea of having two baseline modes: one H.263 of year 1995 baseline and one H.263+ baseline. Gary Sullivan pointed out, that for several (system oriented) reasons H.263+ expressively forbids a H.263+ baseline mode, and that to his opinion, this policy should not be changed because of the added complexity of the negotiation mechanism. No consensus was reached.

The outcome of this discussion process looks like the following

| Level | Bace/Wenger | O’Connell | Remarks |
| --- | --- | --- | --- |
| 0 0a | H.263 Baseline (H.263+ Baseline) | H.263 Baseline (H.263+ Baseline) | Objections Sullivan |
| 1 | I, J, T | J, T |  |
| 2 | S, limited P | limited P | limited means implicit factor of 4 only |
| 3 | F | I, F |  |
| 4 | (all) | M, S |  |

### Annex C (CPM) in H.263+

During the Portland meeting, a proposal (Q15a46) from Tom-Ivar Johansen (Tandberg) regarding some changes in Annex C to allow CIF CPM was not adopted. Instead, various people seemed to think, that CPM will interfere with H.263+ and especially some of the optional modes of H263+. It was decided to work in the AHC on the possibility of using CPM within H.263+. It was also questioned, whether anybody makes use of Annex C of H.263 version 1995 and so whether additional work is needed in this field.

Mr. Johansen pointed out, that

- The combination of Slices and Independently Segmented Decoding might be also a solution for his problems, but is less flexible, lightweight and simple as Annex C and also burdens the transport stack with additional effort. This might be not acceptable for terminals vendors.

- CPM within H.263 is used in terminal products of his company and within VideoServer MCUs.

- Minor syntax changes could fix the problems of the combination of CPM/OPPTYPE and similar problems regarding Annex K and N, (this after Gary Sullivan has spotted the problem).

- Minor changes in H.242 will allow H.320 terminals to use larger picture format CPM.

- There will be little or no impact to the non Multipoint/H.320 applications, e. g. H.324 systems.

- He prepared the document Q15a34 containing the proposed changes for the Sunriver meeting.

Toshihisa Nakai pointed out, that a simple solution to avoid any syntax changes would be to disallow the usage of Annex C whenever PLUSPTYPE is present.

There was no further Email traffic on these two proposals; so the author believes, that no consensus was reached here.

### Annex N support in H.245

Generally speaking, little work was done in terms of adding/enhancing the terminal control protocols H.245 and H.242 with regards to new features/modes. The noticeable exception is Yasuhiro Tomitas Emails to the reflector concerning syntax and semantics of H.245 extensions relevant for Annex N. Since these extensions are covered in a Sunriver contribution from Japan, further discussion seems not to be necessary here.

As a side note it should be mentioned, that TELES/TU Berlin will have a contribution to the Sunriver meeting of Q.12, 13, 14/16 regarding an extension of H.245 for the support of Video Redundancy Coding, which is implemented by the use of annex N. Since syntax and semantics of these enhancements do not interfere with Mr. Tomitas proposal and since they are strictly limited to the H.225 cap exchange portion of H.245, it seems not to be necessary to burden Q.15 with this proposal. This is especially true because of the still unclear issue of the RTP payload format; see the relevant AHC report on packetization.

### Annex L in packet lossy networks

In conjunction with the packetization discussion the question was raised, whether it might necessary to forbid the use of Annex L in such environments, in which the picture header might get lost (e. g. packet lossy environments).

Stephan Wenger proposed to forbid the use of Annex L completely in packet lossy environments because of the fatal impacts of loosing information like a new chroma key or a picture release request. It was also pointed out, that the mechanisms within the Annex L specification interfere with the packetization scheme.

Gary Sullivan proposed instead the introduction of timeouts of 5 seconds or 5 frames, whichever comes later, for most of the Annex L funtionality.

Stephan Wenger responded, that this would help for one of the two packetization proposals currently discussed, but not for the other.

There seemed to be consensus on the reflector, that the introduction of timeouts is the right approach for most of the problems resulting out of the loss of Annex L information because of a packet loss. However, it was asked to carefully review especially the chroma keying features, whether the timeout mechanism is correctly defined.

Since there was consensus on the introduction of timeouts to Annex L functionality, the necessary changes were made in draft14 by Mr. Sullivan.

## Future work

A couple of workitems of the AHC need further investigation.

During the Sunriver meeting it should be hopefully possible to find consensus on

- Preferred Mode Combinations on H.263+ MPIS vs. Quality modes and

- Annex C support for H.263+

Also during the meeting, but possibly later on the reflector too,  work is needed in

- Close examination of H.245 cap exchange mechanisms for H.263+ MIPS vs. Quality optional modes (Q.11),

- Adding „shortcuts“ in H.245 for the „levels“ of mode combinations, if this seems to be useful and necessary (e. g. to simplify and speed up the negotiation process),

- Evaluate the necessity of similar mechanisms for H.242 and defining those mechanisms if necessary (Q.11)

- Work on a set of mode combinations of the Transport-related modes (H, K, N, O, R) for the different network stacks, H.324, H.323, and H.320 (probably others, too? Any opinions on that?)

The personal feeling of the author is, that the first three points above can be best done by experts of Q.11 and Q.15, because they are used to work very closely together (although Q.12, 13, 14 are also influenced by these discussions). For the H.323 part of the fourth point, a meeting of Q.15 with the experts of the Q.12-14 might be helpful. Since this discussion is very closely coupled to the design of the RTP payload format, it should be probably better done in the packetization AHC of Q.15.

END