| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Sunriver, 8 - 11 September 1997 | Document Q15-B-12 Filename: q15b12.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Kui Zhang (SCALAR Consortium) Center for Vision, Speech and Signal Processing email: K.Zhang@surrey.ac.uk University of Surrey, Guildford Tel: +44-1483-300800 Ext. 3026 Surrey GU2 5XH, UK Fax: +44-1483-534139 |
| Title: | Performance evaluation of H.263+ deblocking filtering and proposal for syntax amendment |
| Purpose: | Information |

_____________________________

# Introduction

This document gives comparison results of experiments on H.263+ deblocking filtering (Annex J). Two versions of Annex J were tested: Annex J in H263PLUS.doc (March 1997) and Annex J in Draft 13. We also give the experimental results on an erroneously implemented Draft 13 Annex J in which the clipd1() function is incorrectly implemented. To avoid such misunderstanding, we propose to amend Annex J in Draft 13 so that the definition of clipd1() can be clarified.

# Experimental Results

The experiments were carried out using CIF sequences. The frame rate is set at 15 fps, the quantization step for intra frame coding is set to 16 and quantization step for inter frame is set to 31. No other options are enabled. Table 1 gives the results without option J and with Old Annex J option as defined in March 97 H263PLUS.doc. Table 2 gives the results using Draft 13 Annex J definition as well as the results when clipd1(x,lim) function is erroneously implemented in which the clipd1(x,lim) is defined as

int clipd1(x,lim)

{

if (x>lim) return lim;

if (x<-lim) return -lim;

return x;

}

Such a definition will reverse the sign of x when lim is less than zero.

Table 1

|  | No Annex J |  |  |  |  |  | Old Annex J |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTRA |  |  | INTER |  |  | INTRA |  |  | INTER |  |
| Sequence | PSNR(dB) | bits | PSNR(dB) |  | bitrate (bps) | PSNR(dB) |  | bits | PSNR(dB) |  | bitrate (kbps) |
| akiyo | 34.59 | 38592 | 32.08 |  | 13867 | 35.19 |  | 38616 | 32.11 |  | 13767 |
| bream | 33.30 | 44128 | 28.60 |  | 78714 | 33.56 |  | 44152 | 28.50 |  | 77192 |
| children | 31.42 | 68832 | 26.65 |  | 90219 | 31.85 |  | 68856 | 26.87 |  | 89640 |
| coastguard | 29.84 | 53284 | 26.07 |  | 92582 | 30.09 |  | 53288 | 26.12 |  | 96839 |
| container | 31.51 | 57088 | 28.34 |  | 23516 | 31.68 |  | 57112 | 28.15 |  | 27266 |
| foreman | 32.16 | 44216 | 28.43 |  | 93560 | 32.64 |  | 44248 | 28.37 |  | 88862 |
| hall | 33.14 | 51584 | 30.27 |  | 21737 | 33.47 |  | 51608 | 30.43 |  | 21531 |
| mad | 34.41 | 32616 | 32.26 |  | 24203 | 34.96 |  | 32640 | 32.36 |  | 23340 |
| news | 32.59 | 54664 | 29.50 |  | 33881 | 32.97 |  | 54688 | 29.61 |  | 33490 |
| silent | 30.91 | 43360 | 29.18 |  | 38882 | 31.32 |  | 43384 | 29.36 |  | 36659 |
| weather | 29.94 | 117400 | 27.41 |  | 35149 | 30.18 |  | 117424 | 27.52 |  | 34904 |
| Average | 32.16 | 55069 | 28.98 |  | 49665 | 32.54 |  | 55092 | 29.04 |  | 49408 |

Table 2

|  | New Annex J |  |  |  |  |  | New Annex J with erroneous clipd1() |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTRA |  |  | INTER |  |  | INTRA |  |  | INTER |  |
| Sequence | PSNR(dB) | bits | PSNR(dB) |  | bitrate (bps) | PSNR(dB) |  | bits | PSNR(dB) |  | bitrate (kbps) |
| akiyo | 35.20 | 38616 | 31.79 |  | 14173 | 34.57 |  | 38616 | 31.70 |  | 14500 |
| bream | 33.51 | 44152 | 28.41 |  | 78488 | 33.00 |  | 44152 | 27.12 |  | 88642 |
| children | 31.82 | 68856 | 26.80 |  | 89878 | 30.86 |  | 68856 | 25.59 |  | 102573 |
| coastguard | 30.06 | 53288 | 26.03 |  | 96732 | 29.50 |  | 53288 | 25.53 |  | 101390 |
| container | 31.69 | 57112 | 27.92 |  | 28159 | 29.00 |  | 57112 | 26.55 |  | 37325 |
| foreman | 32.63 | 44248 | 28.19 |  | 89502 | 32.32 |  | 44248 | 27.91 |  | 91601 |
| hall | 33.56 | 51608 | 30.36 |  | 21485 | 27.95 |  | 51608 | 23.87 |  | 78336 |
| mad | 34.94 | 32640 | 32.23 |  | 23033 | 34.85 |  | 32640 | 32.23 |  | 23383 |
| news | 32.99 | 54688 | 29.47 |  | 33796 | 31.55 |  | 54688 | 28.39 |  | 36126 |
| silent | 31.27 | 43384 | 29.31 |  | 36681 | 31.04 |  | 43384 | 29.09 |  | 37233 |
| weather | 30.17 | 117424 | 27.27 |  | 36014 | 25.56 |  | 117424 | 24.23 |  | 58209 |
| Average | 32.53 | 55092 | 28.89 |  | 49813 | 30.93 |  | 55092 | 27.47 |  | 60847 |

# Discussion

From the average results, it appears that new deblocking filter performs slightly worse that that of the old deblocking filter. However, as it has been pointed out in Q15-A-50, the subject quality is improved. When compared with no filtering, the biggest bitrate increase appears to be in sequences with water background (coastguard, container). It is also noted that both deblocking filtering scheme give positive results on INTRA frame coding (higher PSNR). However, the new scheme produces a lower PSNR and higher bit rate for INTER frame coding.

It also can be seen that with erroneously implemented clipd1() function, the performance suffered significantly in terms PSNR and bit rate.

The subject quality of the new scheme is significantly better than the old one.

# Conclusion

- The new annex J in Draft 13 have a comparable objective performance as the old annex J while produces better visual quality.

- It is recommended that following line Annex J of Draft 13 be changed from

d2=clipd1((A-D)/4,d1/2)

to

d2=clipd1((A-D)/4,abs(d1/2))

to avoid confusion.

# Acknowledgments

I would like to thank Dr. Peter List who first assured me that I had done something wrong. I also would like to thank Dr. Matt Bace who pointed out that clip() function is not necessary for A1 and D1. Then I realized that the real problem is in the definition of clipd1() function.

# References

- Gisle Biontegaard, A simple modification to the loop filter in Annex J, q15a12, Portland, June 1997.

- ITU-T Study Group 16, Draft Text of “H.263+” Revisions to Recommendation H.263 for Determination, March 1997.

- H.263+ Video Group, Draft Text of H.263+, Draft 13, 14 July 1997.