| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Second Meeting: Sunriver, Oregon, 8-11 September 1997 | Document Q15-B-22 Filename: q15b22.doc Date generated: 09/03/97 |
| --- | --- |

| Question: | Q.15/16 |  |
| --- | --- | --- |
| Source: | Torbjörn Einarsson Compression Lab, Ericsson Telecom, SCALAR ACTS project S-126 25 Stockholm | Tel: +46-8-681 3627 Fax: +46-8-710 0460 Email: T.Einarsson@clab.ericsson.se |
| Title: | Changes to remove start code emulations in H.263+ |  |
| Purpose: | Proposal |  |

_____________________________

On Aug. 19, I sent an email to the reflector pointing out a number of emulation issues in Draft 13 of H.263+. Since most of them have been remedied in Draft 14, there is not so much left for this document.

The only thing that is left is the specification of which CBPY-table that shall be used for the different MBTYPES for B, EP, and EI pictures. I made some simulation and found out that the bi-directionally predicted macroblocks for EP-pictures should use INTRA CBPY.

Therefore, I suggest that the current text in O.4.4 
 “Upward predicted macroblocks in EI and EP pictures and Intra macroblocks in EI, EP, and B pictures use the CBPY definition for Intra macroblocks, and other macroblock types in EI, EP, and B pictures use the CBPY definition for Inter macroblocks.” 
is changed to

“Upward predicted macroblocks in EI and EP pictures, bi-directionally predicted macroblocks in EP pictures, and Intra macroblocks in EI, EP, and B pictures use the CBPY definition for Intra macroblocks, and other macroblock types in  EI EP and B pictures use the CBPY definition for Inter macroblocks.”