| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Second Meeting: Sunriver, Oregon, 8-11 September 1997 | Document Q15-B-27 Filename: q15b27.doc Date generated: 09/03/97 |
| --- | --- |

| Question: | Q.15/16 |  |
| --- | --- | --- |
| Source: | Gim Deisher Intel Corporation 5200 NE Elam Young Pkwy, MS JF2-78 Portland, OR 97124 | Tel: +1 503 264-3758 Fax: +1 503 264-9372 Email: Gim_L_Deisher@ccm.jf.intel.com |
| Title: | Draft RTP Payload Format for H.263+ |  |
| Purpose: | Proposal |  |

_____________________________

Revision History

Initial version, 8/7/97

Internet Engineering Task Force                Audio-Video Transport WG

INTERNET-DRAFT                                                   C. Zhu

Linda S. Cline

Gim L. Deisher

Thomas R. Gardos

Christian Maciocco

Donald Newell

Intel Corporation

Stephan Wenger

TU Berlin

RTP Payload Format for the 1998 Version of

ITU-T Rec. H.263 Video (H.263+)

Status of This Memo

This document is an Internet-Draft.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or made obsolete by other documents at

any time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

To learn the current status of any Internet-Draft, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Distribution of this document is unlimited.

1. Introduction

The 1998 version of ITU-T Recommendation H.263 added numerous coding

options to improve codec performance over the 1996 version.  We refer

to the 1998 version as H.263+ in this document.  Among the new options,

the ones with the biggest impact on the RTP payload are the slice

structured mode and the scalability mode.  This section summarizes the

impact of these new coding options on packetization.  Refer to [4] for

more information on coding options.

Slice structure was added to H.263+ for three purposes: to provide

enhanced error resilience capability, to make the bitstream more

amenable to use with an underlying packet transport such as RTP, and

to minimize video delay.  The slice structured mode supports

fragmentation at macroblock boundaries.

H.263+ also includes bitstream scalability as an optional coding mode.

Three kinds of scalability are defined: temporal, signal-to-noise ratio

(SNR), and spatial scalability.  Temporal scalability is achieved via

the disposable nature of bidirectionally predicted frames, or B-frames.

SNR scalability permits refinement of encoded video frames, thereby

improving the quality (or SNR).  Spatial scalability is similar to SNR

scalability except the refinement layer is twice the size of the base

layer in each dimension.  Section 4 describes a method in which

scalability may be used in a layered, multiple stream scenario.

2. Usage of RTP

When transmitting H.263+ video streams over the internet, the output of

the encoder can be packetized directly.  All the bits resulting from

the bitstream including the fixed length codes and variable length

codes will be included in the packet.

For H.263+ bitstreams coded with temporal, spatial, or SNR scalability,

each layer shall be transmitted to a different address in a manner

similar to that suggested by McCanne and Speer [6].  More specifically,

each layer shall use a unique IP address and port combination relative

to the base layer IP address and port.  In addition, temporal relations

between layers shall be expressed using the RTP timestamp so that they

can be synchronized at the receiving ends in multicast or unicast

applications.  It is required that addresses assigned to each layer are

in the same order as ELNUM defined in H.263+ [4].  They should also

have the same relationship; for example, if a receiver is receiving

layers 0, 1 and 3, the addresses for reception are base, base + 1, and

base + 3.

The H.263+ video streams will be carried as payload data within RTP

packets.  A new H.263+ payload header is defined in section 3, H.263+

payload header.  This section defines the usage of RTP fixed header

and H.263+ video packet structure.

2.1 RTP Header Usage

Each RTP packet starts with a fixed RTP header.  The following fields

of the RTP fixed header are used for H.263+ video streams:

Marker bit (M bit): The Marker bit of the RTP header is set to 1 when

the current packet carries the end of current frame.  0 otherwise.

Payload Type (PT): The Payload Type shall specify H.263+ video payload

format.  A dynamic payload can be used initially until a static payload

type is assigned.

Timestamp: The RTP Timestamp encodes the sampling instance of the first

video frame contained in the RTP data packet.  The RTP timestamp may be

the same on successive packets if a video frame occupies more than one

packet.  For an H.263+ video stream, the RTP timestamp is based on a

90 kHz clock, the same as that of the RTP payload for H.261 stream [5].

2.2 Video Packet Structure

An H.263+ compressed bitstream is carried as a payload within each RTP

packet.  For each RTP packet, the RTP header is followed by an H.263+

payload header, which is followed by a standard H.263+ compressed

bitstream.  The size of the H.263+ payload header is variable depending

on modes used as detailed in the next section.  The layout of the RTP

H.263+ video packet is shown as:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    RTP Header                                                 |

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.    H.263+ Payload Header                                      .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    H.263+ Stream                                              |

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

3. Design Considerations

The goal of this payload format are: 1) to provide an efficient way of

encapsulating a H.263+ standard compliant bitstream; 2) to enhance the

resiliency towards packet loss for bitstreams generated based on any of

the commonly used coding options.

Since it would be difficult to protect against packet loss in all

situations without adding excessive overhead to the payload header,

resiliency improvements for coding options that are rarely utilized or

often static on the session level are not being considered here.

The optional slice structured mode described in annex K of H.263+ [4]

can be used to enable more flexibility for packetization.  Furthermore,

packets based on a slice structure are also inherently loss resilient.

Similar to a picture segment that begins with a GOB header, the motion

vector predictors in a slice are restricted to reside within its

boundaries.

When slice structure is not applied, we recommend the use of GOB header

in every GOB to reduce the dependency on motion vector prediction

across GOBs.  Reasons justifying this choice are given in section 3.3

of [7].

The independently segmented decoding (ISD) described in annex R of [4]

does not allow any data dependency across slice or GOB boundaries.  It

can be utilized to further improve resiliency in high loss condition.

If ISD is used in conjunction with the slice structure, the dimensions

and quantity of the slices present in a frame should remain the same

between two intra-coded frames (I-frames).  The ISD slices can be

entirely intra coded from time to time to realize quick error recovery

without adding latency time associated with sending complete I-frames.

There are a few assumptions and constraints associated with this design

of the H.263+ payload header format:

. In slice structured mode, only complete slices should be included in

a packet.  In other words, slices should not be fragmented across

packets.  Optimally, a packet will contain only one slice.

. In addition, we assume that no macroblock will be too large to fit in

a packet.

. The temporal reference (TR) corresponding to a frame can also be

derived from the RTP timestamp.

. For resiliency, we recommend sending full picture header for every

frame.  In other words, always set UFEP to '001'.

. In multi-layer scenario, we assume that each layer is transmitted to

a different network address.  See section 5 for more detail on an

optional scheme to do multi-layer synchronization.  The configuration

of each layer such as the enhancement layer number (ELNUM), reference

layer number (RLNUM), and scalability type should be determined at

the start of the session and should not change during the course of

the session.

. When reference picture selection (annex N) is used, the TRI and TRPI

in the slice header or the GOB header should always be set for

resiliency purpose.  This will place TR and TRP in the slice header

or GOB header.  Also set BCI for BCM if a back channel is used.  Note

that the employment of a back channel is not practical in a multicast

environment.

. If any of the options in H.263+ such as custom picture clock

frequency, custom picture format, reference picture selection, and PB

frames are used, the receiver should wait to receive a full picture

header before starting to decode when joining a session in progress.

. Certain configurations and optional modes are assumed to be rarely or

never changed on the session level:

- Split screen indicator.

- Document camera indicator.

- Custom picture clock frequency.

- Picture aspect ratio.

- Reference picture selection mode flag.

. Furthermore, we make no provision for better resiliency against

losses which affect the coding options listed above or any of the

following:

- Full-picture freeze release.

- Reference picture resampling.

- Continuous presence multipoint.

- PB frames.  We recommend the receiver to drop the B frame if loss

occurred in receiving a PB frame.  The receiver may still recover

the P frame portion of the PB frame.

- Any supplemental enhancement information (annex L).

4. H.263+ Payload Header

For H.263+ video streams, each RTP packet carries only one H.263+ video

packet.  The H.263+ payload header is always present for each H.263+

video packet.  Four modes, A, B, C and D, are defined for the H.263+

payload.

Any packet that begins with a picture start code should use mode A.

For packets that start with either a GOB header or a slice header, mode

B is used.  Mode C may be used in place of mode B when custom picture

sizes are involved.  When slice structure coding option is not applied,

some GOBs in the bitstream may be larger than the size of one packet.

In this case, mode D should be used for packets fragmented on the

macroblock boundaries.

These modes are distinguished using bit 2 and bit 3 in the first 32-bit

word of the payload header.  The bits are set to '00' for mode A, '01'

for mode B, '10' for mode C, and '11' for mode D.

The two-bit version field common to all four payload modes serves to

identify between different releases of the H.263+ payload format.  For

this particular draft, the payload is identified as version '00'.

[Ed. Note: The version control will not take effect until a draft has

been formally submitted to the IETF.]

4.1 Mode A

The payload header in mode A is 16 bits in size.  It is used to

encapsulate the first packet in every frame which contains a picture

header with leading picture start code.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=0|0 0|SBIT |EBIT |    RR     |  H.263+ data stream ....      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Mode A Payload Header

V: 2 bits

Version number.  Set to '00' for this payload format.

SBIT: 3 bits

Start bit position specifies the number of bits that should be ignored

in the first data byte.

EBIT: 3 bits

End bit position indicates the number of bits that should be ignored in

the last data byte.

RR:

Reserved bits.

[Ed. Note: Mode B can be used to replace mode A payload header.  The

overhead of using mode B for packets that start with a

picture header is 16 bits.  Should we keep mode A and B

as separate modes or merge them?]

4.2 Mode B

Mode B is used to encapsulate packets that begin with either a GOB

header or a slice header.  In addition to all of the fields described

in the mode A payload header, mode B header also includes parameters

which represent various coding options in H.263+.

The following 1-bit fields match the corresponding annex in H.263+: D,

E, F, G, I, J, K, N, R, S, T, P, and Q.  For example, D in the header

indicates Unrestricted Motion Vector option as defined in annex D.

When custom picture format is applied, mode C should be used instead of

mode B.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=0|0 1|SBIT |EBIT | SRC |Z|D|E|F|G|I|J|K|N|R|S|T| PTC |P|Q|SSS|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Mode B Payload Header

SRC: 3 bits

Source format specifies the picture size as defined in H.263+ [4].

Z: 1 bit

Custom Picture Clock Frequency (PCF).

D: 1 bit

Unrestricted Motion Vector mode (UMV).

E: 1 bit

Syntax-based Arithmetic Coding mode (SAC).

F: 1 bit

Advanced Prediction mode (AP).

G: 1 bit

PB-frames mode (PB).

I: 1 bit

Advanced Intra Coding mode (AIC).

J: 1 bit

Deblocking Filter mode (DF).

K: 1 bit

Slice Structured mode (SS).

N: 1 bit

Reference Picture Selection mode (RPS).

R: 1 bit

Independently Segmented Decoding mode (ISD).

S: 1 bit

Alternative Inter VLC mode (AIV).

T: 1 bit

Modified Quantization mode (MQ).

PTC: 3 bits

Picture Type Code.

'000' I-picture (INTRA)

'001' P-picture (INTER)

'010' Improved PB frame (Annex M)

'011' B-picture (Annex O)

'100' EI-picture (Annex O)

'101' EP-picture (Annex O)

P: 1 bit

Reference Picture Resampling (RPR).

Q: 1 bit

Reduced-Resolution Update mode (RRU).

SSS: 2 bits

Slice Structured Submode, see H.263+ for details.  This field is not

relevant for non-slice structured bitstream.

4.3 Mode C

For bitstream that corresponds to custom picture sizes, i.e. custom

picture format is signaled, mode C should be used instead of mode B.

Mode C payload header consists of two additional fields which indicate

the picture width and height respectively.

In particular, this mode should be utilized to encapsulate packets that

start with a GOB header or a slice header when there are changes in the

custom picture size.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=0|1 0|SBIT |EBIT | SRC |Z|D|E|F|G|I|J|K|N|R|S|T| PTC |P|Q|SSS|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    CPWI     |    CPHI     |RR |  H.263+ data stream ....      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Mode C Payload Header

CPWI: 7 bits

Custom picture width indication.

CPHI: 7 bits

Custom picture height indication.

[Ed. Note: We are considering replacing mode C with a payload header

that includes a picture header (without PSC), such as is

described by Wenger, et al.  The payload header size will

vary from around 3 to 5 dwords.  Comments will be

appreciated.]

4.4 Mode D

Mode D is applicable only to bitstreams that are encoded without using

the slice structured option.

For a packet that begins with neither a GOB header nor a picture

header, additional information is added to the payload header for

resiliency.  If a preceding packet containing the GOB header or the

picture header is lost during the transmission of a video frame, a

receiver may continue to decode the remaining macroblocks based on any

mode D packet it receives.

Note that here we do not attempt to protect against loss of non-zero

motion vector predictors MV2 and MV3 [4] of the leading macroblock in a

packet.  Slice structured mode should be used to avoid potential

problems in recovering these differential motion vectors for large GOBs

consisting of more than one line of macroblocks.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=0|1 1|SBIT |EBIT | SRC |Z|D|E|F|G|I|J|K|N|R|S|T|P|Q|  QUANT  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| PTC |   GN    |       MBN       |    CPWI     |    CPHI     | |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    HMV1     |    VMV1     |    HMV1'    |    VMV1'    |  RR   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Mode D Payload Header

QUANT: 5 bits

Quantizer value in effect at the start of this packet.

GN: 5 bits

GOB number in effect at the start of this packet.

MBN: 9 bits

The macroblock number (relative to its GOB) in effect at the start of

this packet.

HMV1, VMV1: 7 bits each

Horizontal and vertical motion vector predictors for the first

macroblock coded in this packet from the macroblock to its left.

HMV1', VMV1': 7 bits each

Horizontal and vertical motion vector predictors for block 3 of the

first macroblock coded in this packet from the macroblock to its left.

Set to 0's when both advanced prediction and deblocking filter are off.

All other fields are the same as defined in the preceding sections.

[Ed. Note: Is it appropriate to require the use of slice structure with

this payload type?  If so, mode D will not be needed.]

5. Multi-layer Synchronization

The scalability option may be used to create layered encodings.

Multiple layers of H.263+ bitstreams may be transported to different

network addresses.  This section also describes an optional H.263+

specific RTCP packet format for synchronization and adaptive control

when layered bitstreams are distributed with different RTP sessions.

The information contained in these RTCP packets may also be transmitted

out-of-band using a different control protocol.

The need to synchronize different layers transmitted over different

sessions with different addresses calls for payload specific measures

to express the associations between layers, because what is suggested

by Speer and McCanne [6] is not necessarily adequate for H.263+ for

efficient operations in a loosely coupled environments.  For example,

an EP frame in layer 1 can be predicted from a preceding EI or EP frame

in the same layer in conjunction with a P frame from the base layer.

In addition, layer 1 could be an enhancement to base layer spatially,

while layer 2 may be an enhancement to the base layer temporally.  This

information is useful in that layer 2 does not need to use layer 1 in

order to contribute to overall picture.  On the other hand, if layer 2

is coded based on prediction from layer 1, it has to wait for layer 1

in order to proceed.

This section defines H.263+ specific RTCP packets for multi-layer

synchronization and adaptive control.  Layering information is

represented in 4-tuples in the form of [layer number, reference layer

number, scalability attribute, target rate info].  For example:

. [0, -1, any, 20]: base layer depends on nothing at a target rate of

20 kbps.

. [1,  0, SNR, 30]: layer 1, SNR enhancement to layer 0 at a target

rate of 30 kbps.

. [2,  0,   B, 40]: layer 2, B frames enhancement to layer 0, at a

target rate of 40 kbps.

. [3,  2,   S, 40]: layer 3, spatial enhancement to layer 2, at a

target rate of 40 kbps.

The RTCP packets defined in this section shall be sent to the address

of the base layer only.  It serves the following purposes: 1) assist

the receiver in receiving and decoding different layers out of

different addresses; 2) advise the receivers about layer distribution,

attributes, and target rate information so that they can choose the

layers to receive from.  An assumption is made here that layer

dependencies do not change during the session; otherwise an additional

mechanism to distribute this information in-band during dynamic changes

would be necessary, requiring additional bandwidth and receiver

complexity.  If a sender ceases sending one of the layers during the

session, this may be indicated by a rate information value of 0, or by

dropping the tuple for that layer.

The RTCP packet format and payload type to be used is the RTCP APP

(application defined) packet (PT = 204).

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| subtype |    PT=204     |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 SSRC of the packet sender                     |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|                       name (ASCII)                            |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

|  LN   |  RLN  |ST |                    RI                     |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

| ....                                                          |

+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+=+

V, P, and length fields are defined the same as in APP RTCP packets

[1], PT is payload type, value is 204.  Since tuples are 32 bit fields,

the number of tuples in this packet may be derived from the length

field.

subtype: 5 bits

The subtype for this APP packet: 0.

name: 4 octets

The descriptive name for this set of APP packets: "H.263+".

Layer Number (LN): 4 bits

Layer number to be described.

Referenced Layer Number (RLN): 4 bits

The number of the layer on which LN depends.

Scalability Type (ST): 2 bits

Which type of scalability is used: '00' for SNR, '01' for spatial, and

'10' for temporal; '11' for mixed, in which more than one type of

scalability is used.

Rate Information (RI): 22 bits

Date rate measured in unit of 1024 bits per second.

[Ed. Notes: We include section 5 here since it is designed to support

annex O of H.263+.  Is it more appropriate to separate this

section into a different draft?  Comments welcome.]

6. References

[1] H. Schulzrinne, S. Casner, R. Frederick, V. Jacobson, "RTP : A

Transport Protocol for Real-Time Applications", RFC 1889.

[2] "Video Codec for Audiovisual Services at px64 kbits/s", ITU-T

Recommendation H.261, 1993.

[3] "RTP Profile for Audio and Video Conference with Minimal Control",

RFC 1890.

[4] "Video Coding for Low Bitrate Communication", Draft ITU-T

Recommendation H.263, Draft 13, July 1997.

[5] T. Turletti, C. Huitema, "RTP Payload Format for H.261 Video

Streams", RFC 2032.

[6] M. F. Speer and S. McCanne, "RTP Usage with Layered Multimedia

Streams", internet-draft work-in-progress, IETF,

draft-ietf-avt-layered-video-02.txt, May 1996.

[7] C. Zhu, "RTP Payload Format for H.263 Video Streams",

internet-draft, IETF, draft-ietf-avt-rtp-payload-04.txt,

March 1997.