| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Second Meeting: Sunriver, Oregon, 8-11 September 1997 | Document Q15-B-30 Filename: q15b30.doc Date generated: 08/25/97 |
| --- | --- |

| Question: | Q.15/16 |  |
| --- | --- | --- |
| Source: | Karl O. Lillevold Intel Corporation, M/S JF2-78 2111 NE 25th Avenue Hillsboro, OR 97124-5961, USA | Tel: + 1 503 264 7923 Fax: + 1 503 264 9372 Email: klillevo@mailbox.jf.intel.com |
| Title: | # Simulation results: spatial scalability in one dimension |  |
| Purpose: | Information |  |

_____________________________

# Introduction

In document Q15-A-47r1 Thomas Gardos proposed to add spatial scalability in one dimension to H.263+. The group was favorably disposed to the concept of the proposal, but wished to defer the issue until a demonstration of the usefulness of this technique was provided. This document is mainly an accompanying document to a video tape demonstration of the proposed mode. Please refer to document Q15-A-47r1 for further details of this proposal.

# Experiment description

In order to show that scalability in one dimension can be useful, the following combination of layers and bit rates was used as the base for comparisons:

| Layer | Bit rate | Picture resolution |
| --- | --- | --- |
| Base | 30 kbit/sec | QCIF (176x144) |
| EL1 | 60 kbit/sec | ## Parameter to be tested |
| EL2 | 90 kbit/sec | CIF (352x288) |

The parameter to be tested in this experiment is the picture resolution used to encode the middle spatial enhancement layer, EL1. The following variations of the encoded picture resolution for the middle layer were tested:

| Layer | Encoded picture resolution (parameter to be tested) |  |  |  |
| --- | --- | --- | --- | --- |
| EL1 | 0: QCIF (176x144) | 1: 352x144 | 2: 176x288 | 3: CIF (352x288) |

The test sequences used were News and Silent Voice. They were encoded with a picture-level rate control equivalent to the TMN8 rate control, but with a fixed frame rate of 10 fps in each of the layers. Not counting the intra frame, this achieved the target bit rate for each layer with very good accuracy.

A filter with coefficients {-1,0,9,16,9,0,-1} was used to upsample the reconstructed sequences with lower than CIF resolution, to CIF resolution.

# Subjective Results

On the video tape, the subjective results for the four EL1 encoded picture resolutions are shown with the following screen setup:

| QCIF | 352x144 |
| --- | --- |
| 176x288 | CIF |

It is the opinion of the author that the CIF EL1 resolution has the best overall subjective quality, but the quality of the 352x144 resolution is very close. The 176x288 resolution looks worse than 352x144, which is very close to the QCIF quality. Admittedly, the sequences with higher resolutions all have more blocking artifacts than the blurrier QCIF encoded sequence. These blocking artifacts could have been reduced significantly with the new deblocking filter and its matching post-filter, but in order to compare the effect of coding resolution alone, I did not use these options.

# Objective Results

The main results for this experiment are the subjective results as given on the videotape, but a summary of the objective results has also been included. The PSNR measurements in the column marked with (*) are all relative to the CIF original, while the PSNR measurements in column (**) are relative the re-sampled original.

Table 1 Objective Results for News

| Sequence | Layer | Res. (EL1 res.) | Rate | PSNR (*) | PSNR (**) | Mean QP |
| --- | --- | --- | --- | --- | --- | --- |
| News | Base | 176x144 (0-3) | 30 | 26.7 | 32.5 | 12.1 |
|  | EL1 | 176x144 | 60 | 27.7 | 37.0 | 5.2 |
|  |  | 352x144 |  | 30.4 | 34.2 | 9.1 |
|  |  | 176x288 |  | 28.6 | 35.1 | 8.5 |
|  |  | 352x288 |  | 32.3 | 32.3 | 16.1 |
|  | EL2 | 352x288 (0) | 90 | 34.9 | 34.9 | 10.0 |
|  |  | 352x288 (1) |  | 35.2 | 35.2 | 9.2 |
|  |  | 352x288 (2) |  | 35.1 | 35.1 | 9.5 |
|  |  | 352x288 (3) |  | 35.2 | 35.2 | 9.1 |

Table 2 Objective Results for Silent Voice

| Sequence | Layer | Res. (EL1 res.) | Rate | PSNR (*) | PSNR (**) | Mean QP |
| --- | --- | --- | --- | --- | --- | --- |
| Silent Voice | Base | 176x144 (0-3) | 30 | 27.8 | 32.0 | 11.8 |
|  | EL1 | 176x144 | 60 | 29.0 | 36.4 | 5.2 |
|  |  | 352x144 |  | 29.7 | 34.0 | 8.2 |
|  |  | 176x288 |  | 30.5 | 33.7 | 8.4 |
|  |  | 352x288 |  | 31.0 | 31.0 | 14.5 |
|  | EL2 | 352x288 (0) | 90 | 33.3 | 33.3 | 8.9 |
|  |  | 352x288 (1) |  | 33.4 | 33.4 | 8.5 |
|  |  | 352x288 (2) |  | 33.3 | 33.3 | 8.6 |
|  |  | 352x288 (3) |  | 33.4 | 33.4 | 8.5 |

# Conclusions

At least for software implementations, picture re-sampling in one dimension is considerably simpler than re-sampling in both dimensions. Since the subjective and objective performance for spatial scalability in one dimension is reasonably close to spatial scalability in two dimensions, as well as better than using no spatial scalability at the same bit rate, we believe that is enough to prove its usefulness.