| ITU – Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group Sunriver, 8-11 September 1997 | Document Q15-B-54 Filename: q15b54.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Wei-ge Chen (wchen@microsoft.com) and Ming-Chieh Lee Microsoft Corporation One Microsoft Way Redmond, WA 98052 Tel: +1 425 936 0413 |
| Title: | Improved Inter-CBPY coding |
| Purpose: | Proposal |

# 1.	Introduction

CBPY are coded with variable length code as shown Table 13/H263. The assumption is that for intra macroblocks there are more coded luminance blocks than non-coded luminance blocks, while for inter macroblocks the opposite is true. When this assumption is violated, Table 13/H263 becomes inefficient. This contribution proposes a sematics change to code CBPY of inter macroblocks more efficiently.

# 2.	More Details

We found that inter macroblocks have more coded luminance blocks than non-coded ones when both chrominance blocks of are coded. Thus, we propose to use the left column of Table 13/H263 for inter macroblocks when CBPC = ‘11’. The distribution of coded luminance blocks when CBPC == ‘11’ and percentage of savings on CBPY bits are shown in the following tables. Note that except less than 0.1% loss at 10kps and under of the akiyo sequence, the gain ranges from a few percent to more than 26%.

| Sequence | QP | % of Coded Luminance Blocks (CPBC=’11’) |  |  |  |  | No. Bits for CBPY |  | %saved |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Zero | One | Two | Three | Four | Current | Proposed |  |
| Stefan | 5 | 0.02 | 0.04 | 2.01 | 5.59 | 92.34 | 39475 | 28982 | -26.6% |
| Stefan | 10 | 0.00 | 0.35 | 2.49 | 9.52 | 87.64 | 36716 | 32512 | -11.5% |
| Stefan | 20 | 0.00 | 0.78 | 6.24 | 21.06 | 71.92 | 34728 | 33728 | -2.9% |
| Weather | 5 | 0.44 | 2.67 | 20.18 | 17.96 | 58.76 | 14614 | 13130 | -10.2% |
| Weather | 10 | 1.22 | 2.10 | 12.17 | 15.49 | 24.78 | 13759 | 13248 | -3.7% |
| Weather | 20 | 1.49 | 8.46 | 36.82 | 31.34 | 21.89 | 11503 | 11375 | -1.0% |
| Akiyo | 5 | 0.00 | 5.76 | 33.51 | 24.61 | 36.13 | 13733 | 13601 | -1.0% |
| Akiyo | 10 | 7.14 | 28.57 | 57.14 | 3.57 | 3.57 | 9430 | 9439 | +0.09% |
| Akiyo | 20 | 100 | 0 | 0 | 0 | 0 | 4624 | 4626 | +0.04% |
| Foreman | 5 | 0.08 | 1.02 | 6..13 | 13.43 | 79.34 | 40853 | 38685 | -5% |
| Foreman | 10 | 0.74 | 3.69 | 13.55 | 24.88 | 57.14 | 33112 | 32568 | -2% |
| Foreman | 20 | 2.31 | 6.15 | 22.31 | 33.85 | 35.38 | 23705 | 23583 | -1% |

# 3.	Summary

The benefit of the proposed change is more “significant” for lower QP values and more complex scenes. We consider it a useful addition to Annex S. This proposal causes neither complexity nor start code emulation concerns. No new syntax element is introduced.