| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Third Meeting: Eibsee, Bavaria, Germany, 2-5 December, 1997 | Document Q15-C-17 Filename: q15c17.doc Date generated: 11/27/97 |
| --- | --- |

| Question: | Q.15/16 |  |  |
| --- | --- | --- | --- |
| Source: | Gunnar Hellström Swedish National Post and Telecom Agency c/o Omnitor Alsnögatan 7, 4tr S-161 41 Stockholm SWEDEN | Tel: Fax: Email: | +46 701 100 501 +46 8 556 002 06 gunnar.hellstrom@omnitor.se |
| Title: | Proposed sign language sequencies for testing |  |  |
| Purpose: | Discussion |  |  |

_____________________________

This paper and accompanying video sequencies presents two video clips with sign language intended to be used as test material for video coding. It contains sign language performed with natural speed.

The sequence named "Irene" is in Swedish sign language from Swedish Educational TV and shows the head-to-stomach view that is usually used in personal use of videophones for signing. It is recorded in PAL with 25 fps. The presented file q11c17A.mpg is MPEG-1 coded in CIF resolution.

The sequence named "Rosen" is in American sign language at the Gallaudet University in Washington DC and shows a broader view normally useful for two people in picture. It is recorded in NTSC with 30 fps. The presented file q11c17B.mpg is MPEG-1 coded in resolution 352*240.

Both clips present fingerspelling, the feature of sign language that from experience is most challenging for videophones to transfer. Finger movements are so rapid that significant hand positions in many cases are only recorded in one frame.

The clip "Irene", contains a number of eye blinks that are typical grammatical components of sign language used as sentence delimiters. They are short, in many cases only occurring on one or two  frames.

From the clips it can clearly be seen that the quality requirements stated before are valid.

- 25 -30 fps is needed for full perception of sign language

- 12.5 - 15 fps gives some usability

- CIF is needed for smooth perception

- QCIF gives some usability

- Blur in fingers, and face worse than corresponding to QCIF disturbs perception.

Another important factor is the video delay, that should be below 0.4 sec to be good.

Question:

Are these clips of interest for the H.263+ and H.263L testing?

Is the MPEG-1 format appropriate?