| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Third Meeting: Eibsee, Bavaria, Germany, 2-5 December, 1997 | Document Q15-C-19 Filename: q15c19.doc Date generated: 11/26/97 |
| --- | --- |

| Question: | Q.15/16 |  |  |
| --- | --- | --- | --- |
| Source: | Jordi Ribas-Corbera and Shawmin Lei Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA, USA | Tel: Fax: Email: | +1 - 360 - 817 8487 +1 - 360 - 817 8436 jordi@sharplabs.com |
| Title: | Extension of TMN8 rate control to B frames and enhancement layer |  |  |
| Purpose: | Information |  |  |

_____________________________

# 1. Introduction

The frame layer of the current rate control in TMN8 assigns a near constant target number of bits per frame after the first frame is (intra) coded [1]. This strategy is effective for low-delay video communications, in which the coded frames (after the first I) are of type P and the buffer delay must be very small [3].

However, in this paper we show that such a strategy is not as effective when one or several B frames are inserted between the P’s. This is because, for a given level of image quality, the pixels in the B frames can usually be encoded with many fewer bits than those in the P’s. As a result, if we invest a similar number of bits for B and P frames the image quality for the B’s will often be much higher than that of the P’s and, consequently, the image quality will fluctuate throughout the video sequence.

We propose to improve the frame layer of TMN8 rate control in order to reduce the difference in image quality between P and B frames. Our technique consists of two formulas that assign a different target number of bits for those types of frames in a B…BP group. These formulas result from simply extending our macroblock layer (rate control) optimization in [2] to the frame layer and, not surprisingly, indicate that the number of bits for the P and B frames are a function of the prediction error energy in the two types of frames, the number of B frames in a group, and the expected number of bits for encoding the motion vectors and syntax overhead.

We implemented our technique and present results in a wide variety of video sequences and bit rates. Our results show that with the improved frame layer the image quality fluctuates much less and the overall quality improves significantly (up to 1.8 dB in average PSNR).

Finally, in the appendix we briefly explain that TMN8 rate control can be used for the enhancement layer with only a minor modification.

- Improved Frame Layer Rate Control

- We were not able to have this section available on time, but we will bring 60 copies of a revised version of this document (which will include this section) to the meeting.

- Experimental results

We used the UBC’s implementation of an H.263+ codec (version 3.1) with annexes D and F. We implemented our improved frame layer rate control in that codec along with the macroblock layer for the B frames (since rate control for B frames is not currently supported by UBC). We  selected the option of inserting either one or two true B frames between each couple of P frames, i.e., the pattern of picture types was either IBPBPBP… or IBBPBBPBBP…

The experiments that we performed are described in Table 1, and Tables 2 and 3 compare the performance of TMN8 rate control (in average luminance PSNR) with the current and improved frame layers. Figures 1-6 show the PSNR, bits, and buffer fullness for each encoded frame with both frame layers.

4. Conclusions

We have compared the performance of TMN8 rate control with the current and an improved frame layers. With the latter, the bit allocation between P and B frames is more effective. Specifically, the improved frame layer reduces the fluctuation of image quality throughout the video sequence and increases the overall image quality up to 1.8 dB in PSNR.

We propose to replace the current frame layer rate control in TMN8 with the improved version proposed in this paper.

| Test Name | Video Sequence | Pattern of Frames | Bit Rate (Kbps) |
| --- | --- | --- | --- |
| fmn112_a | “foreman” | IBPBP… | 112 |
| fmn112_b | “foreman” | IBBPBBP… | 112 |
| fmn64 | “foreman” | IBPBP… | 64 |
| hall24 | “hall” | IBPBP… | 24 |
| hall48 | “hall” | IBPBP… | 48 |
| hall64_a | “hall” | IBPBP… | 64 |
| hall64_b | “hall” | IBBPBBP… | 64 |
| mad24 | “mother & daughter” | IBPBP… | 24 |
| mad48 | “mother & daughter” | IBPBP… | 48 |
| mad64_a | “mother & daughter” | IBPBP… | 64 |
| mad64_b | “mother & daughter” | IBBPBBP… | 64 |
| sil112_a | “silent” | IBPBP… | 112 |
| sil112_b | “silent” | IBBPBBP… | 112 |
| sil48 | “silent” | IBPBP… | 48 |

Table 1. Description of the experiments: names assigned to each experiment, video data sources, pattern of frames, and target bit rates. The spatial resolution of the video frames was QCIF and the frame rate was of 15 frames per second. As in [2], the bit rate achieved by the rate control was very close to the target in all cases. A total of five seconds were encoded for each video sequence.

|  | TMN8 rate control |  |  |  | TMN8* rate control |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test | PSNR P | PSNR B | ¦ P - B ¦ | PSRN P |  | PSNR B | ¦ P - B ¦ |
| fmn112_a | 33.38 | 34.78 | 1.40 | 34.85 |  | 34.71 | 0.14 |
| fmn112_b | 32.67 | 34.10 | 1.43 | 34.62 |  | 34.67 | 0.05 |
| fmn64 | 30.70 | 31.99 | 1.29 | 32.30 |  | 31.83 | 0.47 |
| hall24 | 31.02 | 31.66 | 0.64 | 31.93 |  | 31.69 | 0.24 |
| hall48 | 33.73 | 34.27 | 0.54 | 34.74 |  | 34.61 | 0.13 |
| hall64_a | 35.15 | 35.70 | 0.55 | 36.32 |  | 36.28 | 0.04 |
| hall64_b | 34.16 | 34.79 | 0.63 | 35.79 |  | 35.74 | 0.05 |
| mad24 | 32.87 | 33.54 | 0.67 | 33.88 |  | 33.44 | 0.44 |
| mad48 | 35.17 | 35.98 | 0.81 | 36.33 |  | 35.84 | 0.49 |
| mad64_a | 36.28 | 37.19 | 0.91 | 37.54 |  | 37.05 | 0.49 |
| mad64_b | 35.66 | 36.58 | 0.92 | 37.17 |  | 36.77 | 0.40 |
| sil112_a | 35.42 | 36.21 | 0.79 | 37.04 |  | 36.78 | 0.26 |
| sil112_b | 34.47 | 35.25 | 0.78 | 36.95 |  | 36.72 | 0.23 |
| sil48 | 31.59 | 32.18 | 0.59 | 32.56 |  | 32.26 | 0.30 |

Table 2.  Results of the average PSNR obtained when using the rate control in TMN8 with the current frame layer (TMN8) and the improved frame layer (TMN8*), for the P (PSNR P) and B (PSNR B) frames. In each case, |P - B| is the absolute value of the difference between the respective PSNR P and PSNR B. With the improved frame layer (TMN8*) the rate control encodes the P and B frames with closer quality (i.e., the value of  | P - B |  is smaller).

| Test | TMN8 PSNR | TMN8* PSNR | Gain in PSNR |
| --- | --- | --- | --- |
| fmn112_a | 34.08 | 34.78 | + 0.70 |
| fmn112_b | 33.63 | 34.64 | + 1.01 |
| fmn64 | 31.35 | 32.07 | + 0.72 |
| hall24 | 31.34 | 31.81 | + 0.47 |
| hall48 | 34.00 | 34.67 | + 0.67 |
| hall64_a | 35.42 | 36.30 | + 0.88 |
| hall64_b | 34.58 | 35.77 | + 1.19 |
| mad24 | 33.20 | 33.66 | + 0.46 |
| mad48 | 35.57 | 36.09 | + 0.52 |
| mad64_a | 36.74 | 37.30 | + 0.56 |
| mad64_b | 36.27 | 37.04 | + 0.77 |
| sil112_a | 35.81 | 36.91 | + 1.10 |
| sil112_b | 34.99 | 36.87 | + 1.88 |
| sil48 | 31.88 | 32.41 | + 0.53 |

Table 3. Results of the average PSNR obtained when using the rate control in TMN8 with the current frame layer (TMN8) and the improved frame layer (TMN8*). The gain in PSNR with the new frame layer is shown in the right-most column.

Figure 1. Comparison of the PSNR per frame of TMN8 rate control with the current frame layer (TMN8, dashed line) and the improved frame layer (TMN8*, solid line), for Test “fmn112_a”. Observe that with the improved frame layer the video sequence is encoded with higher PSNR and smaller PSNR fluctuation.

Figure 2. Comparison of the number of bits per frame used by TMN8 rate control with the current frame layer (TMN8, dashed line) and the improved frame layer (TMN8*, solid line), for Test “fmn112_a”. The peaks and valleys of TMN8* correspond to P and B frames, respectively.

Figure 3. Comparison of the PSNR per frame of TMN8 rate control with the current frame layer (TMN8, dashed line) and the improved frame layer (TMN8*, solid line), for Test “fmn112_b”. Observe that with the new frame layer the video sequence is encoded with higher PSNR and smaller PSNR fluctuation.

Figure 4. Comparison of the number of bits per frame used by TMN8 rate control with its current frame layer (TMN8, dashed line) and TMN8 rate control with the improved frame layer (TMN8*, solid line), for Test “fmn112_b”. The peaks and valleys of TMN8* correspond to P and B frames, respectively.

Figure 5. Comparison of the number of bits in the encoder buffer at the beginning of each frame’s interval with TMN8 rate control with its current frame layer (TMN8, dashed line) and the improved frame layer (TMN8*, solid line), for Test “fmn112_a”. TMN8*’s buffer fullness fluctuates more than TMN8’s, but the buffer fullness is below the maximum (MAX, dotted line) in both cases.

Figure 6. This figure is the same buffer fullness plot as that in Figure 5, but for experiment “fmn112_b”.

6.  References

[1] ITU-T/SG15, Video Codec Test Model, Near-Term, TMN 8, ITU Study Group 16, Video Coding Experts Group, Document Q15-A-59, Portland, June 97.

[2]  J. Ribas-Corbera and S. Lei, “Rate control for low-delay video communications”, ITU Study Group 16, Video Coding Experts Group, Document Q15-A-20, Portland, June 97.

[3]  R. Fryer , “Codec delay model for H.263L novel algorithm evaluation - a proposal,” ITU Study Group 16, Video Coding Experts Group, Document Q15-A-15, Portland, June 97.

Appendix:  Discussion on TMN8 rate control for the enhancement layer

When the spatial/SNR scalability in Annex O is used, an H.263+ codec generates a bit stream for a base layer and another for an enhancement layer , which allows for the decoding of a sequence at more than one quality level. Specifically, some decoders can obtain a lower quality video by decoding only the bit stream for the base layer, while others can obtain a higher quality by decoding both.

Given a total bit rate for the base and enhancement layers, an interesting question is how many bits should be invested into each layer, and an effective rate control method would allocate bits so that the overall quality is as high as possible. However, in most cases, we believe that the number of bits available for the base and enhancement layers will be set by the particular application. For example, one may want to send a base layer bit stream through a channel of rate R1 (e.g., in an ISDN line R1 = 64 Kbps) and an enhancement layer through another channel of rate R2. In that case, the target bit rate for the base layer would be R1 and that for the enhancement layer would be R2.

If we have a target bit rate (and frame rate) for the enhancement layer, observe that TMN8 rate control [1] can be used there without practically any changes. In fact, encoding the enhancement layer is no different from encoding the base layer, except that the prediction for each (enhanced) frame can be improved by the respective frame in the base layer. However, the parameters K and C in [1], which are related to the prediction error and motion/syntax statistics, respectively, will usually be different in both layers and should not be shared; in other words, each layer should have different variables to estimate and keep the values of K and C.

For the purposes of this discussion, we consider the simpler case where there is only one enhancement layer, but recall that there can be more than one.