| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group _________________ Eibsee, 2-5 December 1997 | Document Q15-C-41 Filename: q15c41.doc |
| --- | --- |

| Question: | Q.15/16 |
| --- | --- |
| Source: | Richard Fryer rjf@cs.strath.ac.uk University of Strathclyde, Department of Computer Science 26 Richmond Street Glasgow, Scotland, UK |
| Title: | Requirements Definition for H.263L : A Suggestion |
| Purpose: | Information |

_____________________________

At the Q15/15 meeting in Portland, 24-27 June 1997 it was agreed that there should be an email discussion to attempt to define requirements for H.263L.  In the event there has been little discussion and, so far as I am aware, no conclusions drawn.  Nevertheless the establishment of clear requirements is vital if potential proposers are to understand the necessary properties that their codec(s) must possess.

This document is submitted to act as a starting point for discussions during, and possibly following, the meeting leading to the establishment of clear requirements.

One practical aspect of the requirements is clear --- any proposed codec must be clearly superior to the existing standards H.263, H.263+ and MPEG4.  I take it as given that these codecs are superior to others of older vintage.  It is therefore necessary to establish in what properties superiority must be achieved and how such superiority might be demonstrated.

In my view a useful appreciation of codec properties can be gained by analysis of their success or failure when applied to a challenging task.  Such a task has been posed by Gunnar Hellstrom and adopted as an application area of interest by the ITU.  It is the sending of intelligible sign language over a low bitrate video link.

Since sign language can be part of a two-way conversation [between signers] it is sufficiently challenging to act as a test application.  It stretches the requirements precisely where current codecs experience most difficulty :

- low constant delay

- high frame rate

- high resolution

I therefor propose that as the basis of a requirements specification for H.263L we require that the codec be capable of sustaining an adequately comprehensible two-way conversation between signers in real time and where the signers are seen against complex moving backgrounds.  The reasons for this are

- ‘adequately comprehensible’ --- we have to have a test of success.  What is better than being able to receive the message.  The definition of ‘adequately’ and way of measuring it need it be worked out

- ‘two-way conversation’ --- a useful codec would need to have adequate real-time capability.

- ‘complex moving backgrounds’ --- we all know that if only a small part of the scene is changing then the meaning of frame sizes tends to disappear and that extensive complex motion stretches codec capabilities.  This requirement therefore enforces attention to the full range of codec properties.  How complex etc. is to be decided --- but it should be more than we might feel comfortable with.