| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-16 Filename: q15d16.doc Date generated: 04/07/98 |
| --- | --- |

| Question: | Q.15/16 |  |  |
| --- | --- | --- | --- |
| Source: | Stephan Wenger TU Berlin, Sekr. FR 6-3 Franklinstr. 28-29 D-10587 Berlin, Germany | Tel: Fax: Email: | +1 415 297 2063 +1 604 736 7853 stewe@cs.tu-berlin.de |
| Title: | Some Enhancements of the Picture Header for H.263++ |  |  |
| Purpose: | Proposal |  |  |

_____________________________

## Summary

This document outlines additions and modifications we propose to include in the picture header of subsequent versions of H.263.    A longer Temporal Reference field allows the absolute addressing of every picture in a given picture sequence.    The complete deletion of Picture/GOB/Slice start codes help to improve compression ratio for those networks, which use a Picture/GOB/Slice oriented packetization scheme anyway.    An additional form of picture header abbreviation improves both error resilience and compression ratio for such networks which offer both low-latency, low-quality and high-latency, guarantied quality of service (e. g. Internet and H.223/H.245).    All three proposed mechanisms require a much closer interaction between codec and system environment, and the last one requires to give up the concept of a completely self-contained bit-stream.

## Long Temporal Reference

Currently, the Temporal Reference is an 8-bit (10 bit in some cases) ring counter, counting at the picture clock frequency (usually ~30 Hz).    This picture clock is used for audio synchronization, playback delay calculation and as the Reference Picture Selection mode time-base.    We propose an additional form of TR with a length of 24 bits.    At a penalty of 16 bits per picture, it is possible to address each individual frame in a coded data stream up to about 160 hours length, which we deem to be sufficient for all applications.    This long form of the TR can be used for example in the following way:

Suppose to code an interview-style situation.    Several times, the camera is switched from the reporter to the interviewed person and back.    For such an application, it is possible to use RPS-mode without back channel to use a very old reference picture (the last picture of the previous sub-sequence) instead of sending an I-picture after the camera switch.    The same is obviously possible for video-switching MCUs.

## Deletion of start codes

Currently, all H.263 coded pictures start with a Picture Start Code, which is 22 bits long.    In many system environments including Internet/H.323 systems, there is no need for this information because the picture always has to start at a packet boundary.    As an example, the current RTP payload specification for H.263+ deletes the first 16 zero-bits of each Picture/GOB/Slice header and replaces it by a single bit in the payload specific header, thus achieving some compression gain.    We propose to add language to the next version of the H.263 standard document to reflect this situation.

Some system environments, for example H.324, do currently not allow the deletion of start codes, although in such environment pictures/GOBs/Slices start at packet boundaries, too.    We haven’t yet investigated, whether a similar mechanism can be easily incorporated into H.223 and whether that would improve compression performance.    Unfortunately, no one of our group is a H.223 expert.    We would like to raise this topic here as a discussion point, which might lead to an Q.11 contribution from individual members or from Q.15 itself.

## A new form of picture header abbreviation

Currently, H.263 data streams are completely self-contained, but rely on the loss-less transportation of data.    All data types (e. g. Picture/GOB/Slice headers, DCT Coefficients, Motion vectors etc.) are transmitted with the same priority.

Data partitioning techniques are known to improve error resilience for such transport environments, in which transport channels with different quality of service are available.    In the framework of the newer ITU-T multimedia communication standard families, this is clearly the case: H.245 provide a guarantied error-free communication (with probably long latency), whereas the real-time transport protocols (e. g. H.223 and H.225) do not offer guarantied QoS.

We propose an optional mechanism, which sends several picture header information sets (each containing e. g. used optional modes with parameters, picture size, clock frequency, and similar) to the receiver by external means (e. g. H.245).    This information can be later referenced by a small fixed length code (e. g. four bits for 16 different header information sets).

Our group will continue to work on this issue and come up with a more detailed specification later, if the Q.15 experts feel that this is an interesting direction to look at.