ITU Telecommunication Standardization Sector					Document APC-1334

Study Group 16

Q.12-14/16 Rapporteur Meeting

Yokosuka, 14-17 April 1998

SOURCE: Glen Freundlich, editor H.225.0

email: ggf@lucent.com

voice: +1 303 538 2899

TITLE: H.225.0 Annex I

PURPOSE: Proposal

____________________________________________

Annex I is meant to define packetization for H.263 video bitstreams that contain the new features adopted in version 2 (dated 1998) of H.263 (includes the features using PLUSTYPE or Annexes subsequent to Annex H of H.263). The attached IETF draft describes the RTP payload format for this version of H.263. Assuming that the IETF draft will achieve RFC status, H.225.0 Annex I will reference the RFC rather than reproduce the entire draft. The proposed Annex I would appear as follows:

Annex I - H.263+ Video Packetization

IETF RFC xxxx specifies the RTP payload format for H.263 video bitstreams that contain the new features adopted in version 2 (dated 1998) of H.263 (includes the features using PLUSTYPE or Annexes subsequent to Annex H of H.263).

____________________________________________

Internet Engineering Task Force                 Audio-Video Transport WG

INTERNET-DRAFT                                 C. Bormann / Univ. Bremen

L. Cline / Intel

G. Deisher / Intel

T. Gardos / Intel

C. Maciocco / Intel

D. Newell / Intel

J. Ott / Univ. Bremen

G. Sullivan / PictureTel

S. Wenger / TU Berlin

C. Zhu / Intel

Date Generated: 14 Jan. 1998

RTP Payload Format for the 1998 Version of

ITU-T Rec. H.263 Video (H.263+)

<draft-ietf-avt-rtp-h263-video-01.txt>

Status of This Memo

This document is an Internet-Draft.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas, and

its working groups.  Note that other groups may also distribute working

documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or made obsolete by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference material

or to cite them other than as "work in progress."

To learn the current status of any Internet-Draft, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Distribution of this document is unlimited.

1. Introduction

This document specifies an RTP payload header format applicable to the

transmission of video streams generated based on the 1998 version of

ITU-T Recommendation H.263 [4].  Because the 1998 version of H.263 is a

superset of the 1996 syntax, this format can also be used with the 1996

version of H.263.

The 1998 version of ITU-T Recommendation H.263 added numerous coding

options to improve codec performance over the 1996 version.  The 1998

version is referred to as H.263+ in this document.  Among the new

options, the ones with the biggest impact on the RTP payload

specification and the error resilience of the video content are the

slice structured mode, the independent segment decoding mode (ISD), the

reference picture selection mode, and the scalability mode.  This

section summarizes the impact of these new coding options on

packetization.  Refer to [4] for more information on coding options.

The slice structured mode was added to H.263+ for three purposes: to

provide enhanced error resilience capability, to make the bitstream more

amenable to use with an underlying packet transport such as RTP, and to

minimize video delay.  The slice structured mode supports fragmentation

at macroblock boundaries.

With the independent segment decoding option, a video picture frame is

broken into segments and encoded in such a way that each segment is

independently decodable.  Utilizing ISD in a lossy network environment

helps to prevent the propagation of errors from one segment of the

picture to others.

The reference picture selection mode allows the use of an older

reference picture rather than the one immediately preceding the current

picture.  Usually, the last transmitted frame is implicitly used as the

reference picture for inter-frame prediction.  If the reference picture

selection mode is used, the data stream carries information on what

reference frame should be used, indicated by the temporal reference as

an ID for that reference frame.  The reference picture selection mode

can be used with or without a back channel, which provides information

to the encoder about the internal status of the decoder.  However, no

special provision is made herein for carrying back channel information.

H.263+ also includes bitstream scalability as an optional coding mode.

Three kinds of scalability are defined: temporal, signal-to-noise ratio

(SNR), and spatial scalability.  Temporal scalability is achieved via

the disposable nature of bi-directionally predicted frames, or B-frames.

SNR scalability permits refinement of encoded video frames, thereby

improving the quality (or SNR).  Spatial scalability is similar to SNR

scalability except the refinement layer is twice the size of the base

layer in the horizontal dimension, vertical dimension, or both.

2. Usage of RTP

When transmitting H.263+ video streams over the Internet, the output of

the encoder can be packetized directly.  All the bits resulting from the

bitstream including the fixed length codes and variable length codes

will be included in the packet, with the only exception being that when

the payload of a packet begins with a Picture, GOB, Slice, EOS, or EOSBS

start code, the first two (all-zero) bytes of the start code are removed

and replaced by setting an indicator bit in the payload header.

For H.263+ bitstreams coded with temporal, spatial, or SNR scalability,

each layer may be transported to a different network address.  More

specifically, each layer may use a unique IP address and port number

combination.  The temporal relations between layers shall be expressed

using the RTP timestamp so that they can be synchronized at the

receiving ends in multicast or unicast applications.

The H.263+ video stream will be carried as payload data within RTP

packets.  A new H.263+ payload header is defined in section 4.  This

section defines the usage of the RTP fixed header and H.263+ video

packet structure.

2.1 RTP Header Usage

Each RTP packet starts with a fixed RTP header.  The following fields of

the RTP fixed header are used for H.263+ video streams:

Marker bit (M bit): The Marker bit of the RTP header is set to 1 when

the current packet carries the end of current frame, and is 0 otherwise.

Payload Type (PT): The Payload Type shall specify the H.263+ video

payload format.

Timestamp: The RTP Timestamp encodes the sampling instance of the first

video frame data contained in the RTP data packet.  The RTP timestamp

shall be the same on successive packets if a video frame occupies more

than one packet.  In a multilayer scenario, all pictures corresponding

to the same temporal reference should use the same timestamp.  If

temporal scalability is used (if B-frames are present), the timestamp

may not be monotonically increasing in the RTP stream.  If B-frames are

transmitted on a separate layer and address, they must be synchronized

properly with the reference frames.  Refer to the 1998 ITU-T

Recommendation H.263 [4] for information on required transmission order

to a decoder.  For an H.263+ video stream, the RTP timestamp is based on

a 90 kHz clock, the same as that of the RTP payload for H.261 stream

[5].  Since both the H.263+ data and the RTP header contain time

information, it is required that those timing information run

synchronously.  That is, both the RTP timestamp and the temporal

reference (TR in the picture header of H.263) should carry the same

relative timing information.  If necessary, mathematical rounding should

be applied to the information of the H.263+ data stream to generate the

RTP timestamp (this is especially true for the standard picture clock

frequency of 30000/1001 Hz, and may also be true if custom picture clock

frequencies are to be used; see [4] for details).

2.2 Video Packet Structure

A section of an H.263+ compressed bitstream is carried as a payload

within each RTP packet.  For each RTP packet, the RTP header is followed

by an H.263+ payload header, which is followed by a number of bytes of a

standard H.263+ compressed bitstream.  The size of the H.263+ payload

header is variable depending on the payload involved as detailed in the

section 4.  The layout of the RTP H.263+ video packet is shown as:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    RTP Header                                               ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    H.263+ Payload Header                                    ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    H.263+ Compressed Data Stream                            ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Any H.263+ start codes can be byte aligned by an encoder by using the

stuffing mechanisms of H.263+.  As specified in H.263+, picture, slice,

and EOSBS start codes shall always be byte aligned, and GOB and EOS

start codes may be byte aligned.  For packetization purposes, GOB start

codes should be byte aligned, although this is not absolutely required

herein since it is not required in H.263+.

All H.263+ start codes (Picture, GOB, Slice, EOS, and EOSBS) begin with

16 zero-valued bits.  If a start code is byte aligned and it occurs at

the beginning of a packet, these two bytes shall be removed from the

H.263+ compressed data stream in the packetization process and shall

instead be represented by setting a bit (the P bit) in the payload

header.

3. Design Considerations

The goals of this payload format are to specify an efficient way of

encapsulating an H.263+ standard compliant bitstream and to enhance the

resiliency towards packet losses.  Due to the large number of different

possible coding schemes in H.263+, a copy of the picture header with

configuration information is inserted into the payload header when

appropriate.  The use of that copy of the picture header along with the

payload data can allow decoding of a received packet even in such cases

in which another packet containing the original picture header becomes

lost.

There are a few assumptions and constraints associated with this H.263+

payload header design.  The purpose of this section is to point out

various design issues and also to discuss several coding options

provided by H.263+ that may impact the performance of network-based

H.263+ video.

o The optional slice structured mode described in annex K of H.263+ [4]

enables more flexibility for packetization.  Similar to a picture

segment that begins with a GOB header, the motion vector predictors in

a slice are restricted to reside within its boundaries.  However,

slices provide much greater freedom in the selection of the size and

shape of the area which is represented as a distinct decodable region.

In particular, slices can have a size which is dynamically selected to

allow the data for each slice to fit into a chosen packet size.

Slices can also be chosen to have a rectangular shape which is

conducive for minimizing the impact of errors and packet losses on

motion compensated prediction.  For these reasons, the use of the

slice structured mode is strongly recommended for any applications

used in environments where significant packet loss occurs.

o In non-rectangular slice structured mode, only complete slices should

be included in a packet.  In other words, slices should not be

fragmented across packet boundaries.  The only reasonable need for a

slice to be fragmented across packet boundaries is when the encoder

which generated the H.263+ data stream could not be influenced by an

awareness of the packetization process (such as when sending H.263+

data through a network other than the one to which the encoder is

attached, as in network gateway implementations).  Optimally, each

packet will contain only one slice.

o The independent segment decoding (ISD) described in annex R of [4]

prevents any data dependency across slice or GOB boundaries in the

reference picture.  It can be utilized to further improve resiliency

in high loss conditions.

o If ISD is used in conjunction with the slice structure, the

rectangular slice submode shall be enabled and the dimensions and

quantity of the slices present in a frame shall remain the same

between each two intra-coded frames (I-frames), as required in H.263+.

The individual ISD segments may also be entirely intra coded from time

to time to realize quick error recovery without adding the latency

time associated with sending complete INTRA-pictures.

o When the slice structure is not applied, the insertion of a

(preferably byte-aligned) GOB header can be used to provide resync

boundaries in the bitstream, as the presence of a GOB header

eliminates the dependency of motion vector prediction across GOB

boundaries.  These resync boundaries provide natural locations for

packet payload boundaries.

o H.263+ allows picture headers to be sent in an abbreviated form in

order to prevent repetition of overhead information that does not

change from picture to picture.  For resiliency, sending a complete

picture header for every frame is often advisable.  This means, that

especially in cases with high packet loss probability in which picture

header contents are not expected to be highly predictable, the sender

may always set the subfield UFEP in PLUSPTYPE to '001' in the H.263+

video bitstream.

o In a multi-layer scenario, each layer may be transmitted to a

different network address.  The configuration of each layer such as

the enhancement layer number (ELNUM), reference layer number (RLNUM),

and scalability type should be determined at the start of the session

and should not change during the course of the session.

o All start codes can be byte aligned, and picture, slice, and EOSBS

start codes are always byte aligned.  The boundaries of these

syntactical elements provide ideal locations for placing packet

boundaries.

o We assume that a maximum Picture Header size of 504 bits is

sufficient.  The syntax of H.263+ does not explicitly prohibit larger

picture header sizes, but the use of such extremely large picture

headers is not expected.

4. H.263+ Payload Header

For H.263+ video streams, each RTP packet carries only one H.263+ video

packet.  The H.263+ payload header is always present for each H.263+

video packet.  The payload header is of variable length.  A 16 bit field

of the basic payload header may be followed by an 8 bit field for Video

Redundancy Coding information, and/or by a variable length picture

header as indicated by PLEN. These optional fields appear in the order

given above when present.

If a picture header is included in the payload header, the length of the

picture header in number of bytes is specified by PLEN.  The minimum

length of the payload header is 16 bits, corresponding to PLEN equal to

0 and no VRC information present.

The remainder of this section defines the various components of the RTP

payload header.  Section five defines the various packet types that are

used to carry different types of H.263+ coded data, and section six

summarizes how to distinguish between the various packet types.

4.1 General H.263+ payload header

The H.263+ payload header is structured as follows:

0                   1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  RR     |P|V|  PLEN     |PEBIT|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

RR: 5 bits

Reserved bits.  Shall be zero.

P: 1 bit

Indicates the picture start or a picture segment (GOB/Slice) start or

a video sequence end (EOS or EOSBS).  Two bytes of zero bits then have

to be prefixed to the payload of such a packet to compose a complete

picture/GOB/slice/EOS/EOSBS start code.  This bit allows the omission

of the two first bytes of the start codes, thus improving the

compression ratio.

V: 1 bit

Indicates the presence of an 8 bit field containing information for

Video Redundancy Coding (VRC), which follows immediately after the

initial 16 bits of the payload header if present.  For syntax and

semantics of that 8 bit VRC field see section 4.2.

PLEN: 6 bits

Picture header length in number of bytes.  If no additional picture

header is attached, PLEN is 0.  If PLEN>0, the additional picture

header is attached immediately following the rest of the payload

header.

PEBIT: 3 bits

Indicates the number of bits that shall be ignored in the last byte of

the picture header.  If PLEN is zero, then PEBIT shall also be zero.

4.2 Video Redundancy Coding Header Extension

Video Redundancy Coding (VRC) is an optional mechanism intended to

improve error resilience over packet networks.  Implementing VRC in

H.263+ will require the Reference Picture Selection option described in

Annex N.  By having multiple "threads" of independently inter-frame

predicted pictures, damage of individual frame will cause distortions

only within its own thread but leave the other threads unaffected.  From

time to time, all threads converge to a so-called sync frame (an INTRA

picture or a non-INTRA picture which is redundantly represented within

multiple threads); from this sync frame, the independent threads are

started again.  For a more complete description of VRC see [7].

While a VRC data stream is - like all H.263+ data - totally self-

contained, it may be useful for the transport hierarchy implementation

to have knowledge about the current damage status of each thread.  On

the Internet, this status can easily be determined by observing the

marker bit, the sequence number of the RTP header, and the thread-id and

a circling "packet per thread" number.  The latter two numbers are coded

in the VRC header extension.

The format of the VRC header extension is as follows:

0 1 2 3 4 5 6 7

+-+-+-+-+-+-+-+-+

| TID | Trun  |S|

+-+-+-+-+-+-+-+-+

TID: 3 bits

Thread ID.  Up to 7 threads are allowed. Each frame of H.263+ VRC data

will use as reference information only sync frames or frames within

the same thread.  By convention, thread 0 is expected to be the

"canonical" thread, which is the thread from which the sync frame

should ideally be used.  In the case of corruption or loss of the

thread 0 representation, a representation of the sync frame with a

higher thread number can be used by the decoder.  Lower thread numbers

are expected to contain equal or better representations of the sync

frames than higher thread numbers in the absence of data corruption or

loss.  See [7] for details.

Trun: 4 bits

Monotonically increasing (modulo 16) 4 bit number counting the packet

number within each thread.

S: 1 bit

A bit that indicates that the packet content is for a sync frame.  An

encoder using VRC may send several representations of the same "sync"

picture, in order to ensure that regardless of which thread of

pictures is corrupted by errors or packet losses, the reception of at

least one representation of a particular picture is ensured (within at

least one thread).  The sync picture can then be used for the

prediction of any thread.  If packet losses have not occurred, then

the sync frame contents of thread 0 can be used and those of other

threads can be discarded (and similarly for other threads).  Thread 0

is considered the "canonical" thread, the use of which is preferable

to all others.  The contents of packets having lower thread numbers

shall be considered as generally preferred over those with higher

thread numbers.

5. Packetization schemes

5.1 Picture Segment Packets and Sequence Ending Packets (P=1)

A picture segment packet is defined as a packet that starts at the

location of a Picture, GOB, or slice start code in the H.263+ data

stream.  This corresponds to the definition of the start of a video

picture segment as defined in H.263+.  For such packets, P=1 always.

An extra picture header can sometimes be attached in the payload header

of such packets.  Whenever an extra picture header is attached as

signified by PLEN>0, only the last six bits of its picture start code,

'100000', are included in the payload header.  A complete H.263+ picture

header with byte aligned picture start code can be conveniently

assembled on the receiving end by prepending the sixteen leading '0'

bits.

When PLEN>0, the end bit position corresponding to the last byte of the

picture header data is indicated by PEBIT.  The actual bitstream data

shall begin on an 8-bit byte boundary following the payload header.

A sequence ending packet is defined as a packet that starts at the

location of an EOS or EOSBS code in the H.263+ data stream.  This

delineates the end of a sequence of H.263+ video data (more H.263+ video

data may still follow later, however, as specified in ITU-T

Recommendation H.263).  For such packets, P=1 and PLEN=0 always.

The optional header extension for VRC may or may not be present as

indicated by the V bit flag.

5.1.1 Packets that begin with a Picture Start Code

Any packet that contains the whole or the start of a coded picture shall

start at the location of the picture start code (PSC), and should

normally be encapsulated with no extra copy of the picture header. In

other words, normally PLEN=0 in such a case.   However, if the coded

picture contains an incomplete picture header (UFEP = "000"), then a

representation of the complete (UFEP = "001") picture header may be

attached during packetization in order to provide greater error

resilience.  Thus, for packets that start at the location of a picture

start code, PLEN shall be zero unless both of the following conditions

apply:

1) The picture header in the H.263+ bitstream payload is incomplete

(PLUSPTYPE present and UFEP="000"), and

2) The additional picture header which is attached is not incomplete

(UFEP="001").

A packet which begins at the location of a Picture, GOB, slice, EOS, or

EOSBS start code shall omit the first two (all zero) bytes from the

H.263+ bitstream, and signify their presence by setting P=1 in the

payload header.

Here is an example of encapsulating the first packet in a frame (without

an attached redundant complete picture header):

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  RR     |1|V|0|0|0|0|0|0|0|0|0|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-------------------------------+

| bitstream data without the first two 0 bytes of the PSC       |

+---------------------------------------------------------------+

5.1.2 Packets that begin with GBSC or SSC

For a packet that begins at the location of a GOB or slice start code,

PLEN may be zero or may be nonzero, depending on whether a redundant

picture header is attached to the packet.  In environments with very low

packet loss rates, or when picture header contents are very seldom

likely to change (except as can be detected from the GFID syntax of

H.263+), a redundant copy of the picture header is not required.

However, in less ideal circumstances a redundant picture header should

be attached for enhanced error resilience, and its presence is indicated

by PLEN>0.

Assuming a PLEN of 9, below is an example of a packet that begins with a

GBSC or a SSC:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  RR     |1|V|0 0 1 0 0 1|PEBIT|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1 0 0 0 0 0| picture header starting with TR, PTYPE, ...       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| ...                                                           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| ...           | bitstream data begins with GBSC/SCC ...       .

+-+-+-+-+-+-+-+-+-----------------------------------------------+

Notice that only the last six bits of the picture start code, '100000',

are included in the payload header.  A complete H.263+ picture header

with byte aligned picture start code can be conveniently assembled if

needed on the receiving end by prepending the sixteen leading '0' bits.

5.1.3 Packets that Begin with an EOS or EOSBS Code

For a packet that begins with an EOS or EOSBS code, PLEN shall be zero,

and no Picture, GOB, or Slice start codes shall be included within the

same packet.  As with other packets beginning with start codes, the two

all-zero bytes that begin the EOS or EOSBS code at the beginning of the

packet shall be omitted, and their presence shall be indicated by

setting the P bit to 1 in the payload header.

System designers should be aware that some decoders may interpret the

loss of a packet containing only EOS or EOSBS information as the loss of

essential video data and may thus respond by not displaying some

subsequent video information.  Since EOS and EOSBS codes do not actually

affect the decoding of video pictures, they are somewhat unnecessary to

send at all.  Because of the danger of misinterpretation of the loss of

such a packet, encoders are generally to be discouraged from sending EOS

and EOSBS.

Below is an example of a packet containing an EOS code:

0                   1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  RR     |1|V|0|0|0|0|0|0|0|0|0|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1|1|1|1|1|1|0|0|

+-+-+-+-+-+-+-+-+

5.2 Encapsulating Follow-On Packet (P=0)

A Follow-on packet contains a number of bytes of coded H.263+ data which

does not start at a synchronization point.  That is, a Follow-On packet

does not start with a Picture, GOB, Slice, EOS, or EOSBS header, and it

may or may not start at a macroblock boundary.  Since Follow-on packets

do not start at synchronization points, the data at the beginning of a

follow-on packet is not independently decodable.  For such packets, P=0

always.  If the preceding packet of a Follow-on packet got lost, the

receiver may discard that Follow-on packet as well as all other

following Follow-on packets.  Better behavior, of course, would be for

the receiver to scan the interior of the packet payload content to

determine whether any start codes are found in the interior of the

packet which can be used as resync points.  The use of an attached copy

of a picture header for a follow-on packet is useful only if the

interior of the packet or some subsequent follow-on packet contains a

resync code such as a GOB or slice start code.  PLEN>0 is allowed, since

it may allow resync in the interior of the packet.  The decoder may also

be resynchronized at the next segment or picture packet.

Here is an example of a follow-on packet (with PLEN=0):

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  RR     |0|V|0|0|0|0|0|0|0|0|0|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-------------------------------+

| bitstream data                                                |

+---------------------------------------------------------------+

6. Use of this payload specification

There is no syntactical difference between a picture segment packet and

a Follow-on packet, other than the indication P=1 for picture segment or

sequence ending packets and P=0 for Follow-on packets.  See the

following for a summary of the entire packet types and ways to

distinguish between them.

For a more detailed discussion on how to use the payload specification,

the reader should refer to [8].

It is possible to distinguish between the different packet types by

checking the P bit and the first 6 bits of the payload along with the

header information.  The following table shows the packet type for

permutations of this information (see also the picture/GOB/Slice header

descriptions in H.263+ for details):

--------------+--------------+----------------------+-------------------

First 6 bits | P-Bit | PLEN |  Packet              |  Remarks

of Payload   |(payload hdr.)|                      |

--------------+--------------+----------------------+-------------------

100000       |   1   |  0   |  Picture             |  Typical Picture

100000       |   1   | > 0  |  Picture             |  Note UFEP

1xxxxx       |   1   |  0   |  GOB/Slice/EOS/EOSBS |  See possible GNs

1xxxxx       |   1   | > 0  |  GOB/Slice           |  See possible GNs

Xxxxxx       |   0   |  0   |  Follow-on           |

Xxxxxx       |   0   | > 0  |  Follow-on           |  Interior Resync

--------------+--------------+----------------------+-------------------

See [4] for details regarding the possible values of the six bits (a "1"

bit followed by a five bit GN field explicit or emulated) of GOB, Slice,

EOS, and EOSBS codes.

As defined in this specification, every start of a coded frame (as

indicated by the presence of a PSC) has to be encapsulated as a picture

segment packet.  If the whole coded picture fits into one packet of

reasonable size (which is dependent on the connection characteristics),

this is the only type of packet that needs to be observed.  Due to the

high compression ratio achieved by H.263+ it is often possible to use

this mechanism, especially for small spatial picture formats such as

QCIF and typical Internet packet sizes around 1500 bytes.

If the complete coded frame does not fit into a single packet, two

different ways for the packetization may be chosen.  In case of very low

or zero packet loss probability, one or more Follow-on packets may be

used for coding the rest of the picture.  Doing so leads to minimal

coding and packetization overhead as well as to an optimal use of the

maximal packet size, but does not provide any added error resilience.

The alternative is to break the picture into reasonably small partitions

- called Segments - (by using the Slice or GOB mechanism), that do offer

synchronization points.  By doing so and using the Picture Segment

payload with PLEN>0, decoding of the transmitted packets is possible

even in such cases in which the Picture packet containing the picture

header was lost (provided any necessary reference picture is available).

Picture Segment packets can also be used in conjunction with Follow-on

packets for large segment sizes.

7. Security Considerations

RTP packets using the payload format defined in this specification are

subject to the security considerations discussed in the RTP

specification [1], and any appropriate RTP profile (for example [3]).

This implies that confidentiality of the media streams is achieved by

encryption.  Because the data compression used with this payload format

is applied end-to-end, encryption may be performed after compression so

there is no conflict between the two operations.

A potential denial-of-service threat exists for data encodings using

compression techniques that have non-uniform receiver-end computational

load.  The attacker can inject pathological datagrams into the stream

which are complex to decode and cause the receiver to be overloaded.

However, this encoding does not exhibit any significant non-uniformity.

As with any IP-based protocol, in some circumstances a receiver may be

overloaded simply by the receipt of too many packets, either desired or

undesired.  Network-layer authentication may be used to discard packets

from undesired sources, but the processing cost of the authentication

itself may be too high.  In a multicast environment, pruning of specific

sources may be implemented in future versions of IGMP [5] and in

multicast routing protocols to allow a receiver to select which sources

are allowed to reach it.

A security review of this payload format found no additional

considerations beyond those in the RTP specification.

8. References

[1] H. Schulzrinne, S. Casner, R. Frederick, V. Jacobson, "RTP : A

Transport Protocol for Real-Time Applications", RFC 1889.

[2] "Video Codec for Audiovisual Services at px64 kbits/s", ITU-T

Recommendation H.261, 1993.

[3] "RTP Profile for Audio and Video Conference with Minimal Control",

RFC 1890.

[4] "Video Coding for Low Bitrate Communication", Draft ITU-T

Recommendation H.263, Draft 20, September 1997.

[5] T. Turletti, C. Huitema, "RTP Payload Format for H.261 Video

Streams", RFC 2032.

[6] C. Zhu, "RTP Payload Format for H.263 Video Streams", RFC 2190.

[7] S. Wenger, "Video Redundancy Coding in H.263+", Proc. AVSPN97,

Aberdeen, U.K..

[8] S. Wenger, G. Knorr, J. Ott: "Error resilience support in H.263

V.2", submitted for publication to IEEE T-CSVT, 1997.