| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Video Coding Experts Group (Question 15) _________________ Fourth Meeting: Tampere, Finland, 21-24 April, 1998 | Document Q15-D-44 Filename: q15d44.doc Generated: 15 April ’98 |
| --- | --- |

| Question: | Q.15/SG16 |  |  |
| --- | --- | --- | --- |
| Source: | Torbjörn Einarsson and Per Thorell SCALAR ACTS project Ericsson Telecom Tellusborgsvägen 83 S-126 25 Stockholm Sweden | Tel: Fax: Email: | +46 8 681 3627 +46 8 719 0460 T.Einarsson@clab.ericsson.se |
| Title: | P-pictures in enhancement layers |  |  |
| Purpose: | Proposal for H.263++ |  |  |

_____________________________

## Proposal

We propose that a new picture type, P-pictures in enhancement layers or EF-pictures, is included in H.263++ to provide a simple and efficient means for increasing the frame rate of enhancement layers.

## The problem and the solution

Currently, the only way to increase the frame rate in an enhancement layer is to use B-pictures.

This has a number of disadvantages:

- A delay is introduced because of out-of-order processing of pictures

- Extra memory is needed for one or more frame storages

- The implementation is rather complex

- B-pictures cannot be used as reference for higher layers, which means that the bits spent on B-pictures are wasted for producing additional higher quality layers.

There is a simple solution to all of the above problems, which was unfortunately overseen when creating H.263+. That is to add the possibility of P-pictures in enhancement layers. Since an ordinary P-picture does not have a layer number, we prefer to call the new picture type EF (Enhancement Forward) picture.

## Implementation of EF-pictures

An EF-picture has only forward prediction in contrast to an EP-picture that has both forward and upward prediction. All options that can be used for P-pictures can be used for EF-pictures.
However, since an EF-picture needs an ELNUM field, we suggest that it is signaled as a new picture type using, for example, Picture Type Code “110”.

Normally, an EF-picture would only need an ELNUM field, but it may also be useful to have an RLNUM field. For example, one could construct EF-pictures that are in a separate layer and thus disposable.

## An example

The following example shows the implementation of three layers (QCIF,15Hz), (QCIF,30Hz), (CIF,30Hz) using B-pictures and EF-picture respectively. The advantages of the EF-picture approach is

- Simple implementation

- No extra delay because of out-of-order processing

- No extra frame store is needed

- The EF-picture is used as reference for Enhancement Layer 2.

Figure 1. Three layers with increasing frame rate using B-pictures

Figure 2. Three layers with increasing frame rate using EF-pictures